37	\fBaaa\fR		Adjust after addition

d5 0a	\fBaad\fR		Adjust AX before division

d4 0a	\fBaam\fR		Adjust AX after multiply

3f	\fBaas\fR		Adjust AL after subtraction

.sp \n(pDu

\fBadc\fR		Add with carry

83 /2	\fBadcl\fR	\fIimm8s,rm32\fR

83 /2	\fBadcw\fR	\fIimm8s,rm16\fR

14	\fBadcb\fR	\fIimm8,al\fR

15	\fBadcw\fR	\fIimm16,ax\fR

15	\fBadcl\fR	\fIimm32,eax\fR

15	\fBadcl\fR	\fIimm32\fR

80 /2	\fBadcb\fR	\fIimm8,rm8\fR

81 /2	\fBadcw\fR	\fIimm16,rm16\fR

81 /2	\fBadcl\fR	\fIimm32,rm32\fR

12 /r	\fBadcb\fR	\fIrm8,r8\fR

13 /r	\fBadcw\fR	\fIrm16,r16\fR

13 /r	\fBadcl\fR	\fIrm32,r32\fR

10 /r	\fBadcb\fR	\fIr8,rm8\fR

11 /r	\fBadcw\fR	\fIr16,rm16\fR

11 /r	\fBadcl\fR	\fIr32,rm32\fR

.sp \n(pDu

\fBadd\fR		Add

83 /0	\fBaddl\fR	\fIimm8s,rm32\fR

83 /0	\fBaddw\fR	\fIimm8s,rm16\fR

04	\fBaddb\fR	\fIimm8,al\fR

05	\fBaddw\fR	\fIimm16,ax\fR

05	\fBaddl\fR	\fIimm32,eax\fR

05	\fBaddl\fR	\fIimm32\fR

80 /0	\fBaddb\fR	\fIimm8,rm8\fR

81 /0	\fBaddw\fR	\fIimm16,rm16\fR

81 /0	\fBaddl\fR	\fIimm32,rm32\fR

02 /r	\fBaddb\fR	\fIrm8,r8\fR

03 /r	\fBaddw\fR	\fIrm16,r16\fR

03 /r	\fBaddl\fR	\fIrm32,r32\fR

00 /r	\fBaddb\fR	\fIr8,rm8\fR

01 /r	\fBaddw\fR	\fIr16,rm16\fR

01 /r	\fBaddl\fR	\fIr32,rm32\fR

.sp \n(pDu

\fBand\fR		Logical AND

83 /4	\fBandl\fR	\fIimm8s,rm32\fR

83 /4	\fBandw\fR	\fIimm8s,rm16\fR

24	\fBandb\fR	\fIimm8,al\fR

25	\fBandw\fR	\fIimm16,ax\fR

25	\fBandl\fR	\fIimm32,eax\fR

25	\fBandl\fR	\fIimm32\fR

80 /4	\fBandb\fR	\fIimm8,rm8\fR

81 /4	\fBandw\fR	\fIimm16,rm16\fR

81 /4	\fBandl\fR	\fIimm32,rm32\fR

22 /r	\fBandb\fR	\fIrm8,r8\fR

23 /r	\fBandw\fR	\fIrm16,r16\fR

23 /r	\fBandl\fR	\fIrm32,r32\fR

20 /r	\fBandb\fR	\fIr8,rm8\fR

21 /r	\fBandw\fR	\fIr16,rm16\fR

21 /r	\fBandl\fR	\fIr32,rm32\fR

.sp \n(pDu

63 /r	\fBarpl\fR	\fIr16,rm16\fR	Adjust RPL field of selector

.sp \n(pDu

\fBbound\fR		Check if register is within bounds

62 /r	\fBboundw\fR	\fIm16,r16\fR

62 /r	\fBboundl\fR	\fIm32,r32\fR

.sp \n(pDu

\fBbsf\fR		Bit scan forward

0F bc	\fBbsfw\fR	\fIrm16,r16\fR

0F bc	\fBbsfl\fR	\fIrm32,r32\fR

.sp \n(pDu

\fBbsr\fR		Bit scan reverse

0F bd	\fBbsrw\fR	\fIrm16,r16\fR

0F bd	\fBbsrl\fR	\fIrm32,r32\fR

.sp \n(pDu

\fBbt\fR		Save bit in carry flag

0F a3	\fBbtw\fR	\fIr16,rm16\fR

0F a3	\fBbtl\fR	\fIr32,rm32\fR

0F ba /4	\fBbtw\fR	\fIimm8,rm16\fR

0F ba /4	\fBbtl\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBbtc\fR		Bit test and complement

0F bb	\fBbtcw\fR	\fIr16,rm16\fR

0F bb	\fBbtcl\fR	\fIr32,rm32\fR

0F ba /7	\fBbtcw\fR	\fIimm8,rm16\fR

0F ba /7	\fBbtcl\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBbtr\fR		Bit test and reset

0F b3	\fBbtrw\fR	\fIr16,rm16\fR

0F b3	\fBbtrl\fR	\fIr32,rm32\fR

0F ba /6	\fBbtrw\fR	\fIimm8,rm16\fR

0F ba /6	\fBbtrl\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBbts\fR		Bit test and set

0F ab	\fBbtsw\fR	\fIr16,rm16\fR

0F ab	\fBbtsl\fR	\fIr32,rm32\fR

0F ba /5	\fBbtsw\fR	\fIimm8,rm16\fR

0F ba /5	\fBbtsl\fR	\fIimm8,rm32\fR

.sp \n(pDu

e8	\fBcall\fR	\fIreli\fR	Call Procedure

98	\fBcbtw\fR		Sign extend AL

98	\fBcbw\fR		Sign extend AL

99	\fBcdq\fR		Double word to quad word

f8	\fBclc\fR		Clear carry

fc	\fBcld\fR		Clear direction Flag

fa	\fBcli\fR		Clear interrupt Flag

99	\fBcltd\fR		Double word to quad word

0F 06	\fBclts\fR		Clear task-switched flag in CR0

f5	\fBcmc\fR		Complement carry flag

.sp \n(pDu

\fBcmp\fR		Compare

83 /7	\fBcmpl\fR	\fIimm8s,rm32\fR

83 /7	\fBcmpw\fR	\fIimm8s,rm16\fR

3c	\fBcmpb\fR	\fIimm8,al\fR

3d	\fBcmpw\fR	\fIimm16,ax\fR

3d	\fBcmpl\fR	\fIimm32,eax\fR

3d	\fBcmpl\fR	\fIimm32\fR

80 /7	\fBcmpb\fR	\fIimm8,rm8\fR

81 /7	\fBcmpw\fR	\fIimm16,rm16\fR

81 /7	\fBcmpl\fR	\fIimm32,rm32\fR

3a /r	\fBcmpb\fR	\fIrm8,r8\fR

3b /r	\fBcmpw\fR	\fIrm16,r16\fR

3b /r	\fBcmpl\fR	\fIrm32,r32\fR

38 /r	\fBcmpb\fR	\fIr8,rm8\fR

39 /r	\fBcmpw\fR	\fIr16,rm16\fR

39 /r	\fBcmpl\fR	\fIr32,rm32\fR

.sp \n(pDu

a6	\fBcmpsb\fR		Compare bytes

a7	\fBcmpsl\fR		Compare long

a7	\fBcmpsw\fR		Compare words

99	\fBcwd\fR		Word to double word

98	\fBcwde\fR		Sign extend AX

99	\fBcwtd\fR		Word to double word

98	\fBcwtl\fR		Sign extend AX

27	\fBdaa\fR		Decimal adjust after addition

2f	\fBdas\fR		Decimal adjust after subtraction

.sp \n(pDu

\fBdec\fR		Decrement by 1

48 +r	\fBdecw\fR	\fIr16\fR

48 +r	\fBdecl\fR	\fIr32\fR

fe /1	\fBdecb\fR	\fIrm8\fR

ff /1	\fBdecw\fR	\fIrm16\fR

ff /1	\fBdecl\fR	\fIrm32\fR

.sp \n(pDu

\fBdiv\fR		Unsigned divide

f6 /6	\fBdivb\fR	\fIrm8,al\fR

f6 /6	\fBdivb\fR	\fIrm8\fR

f7 /6	\fBdivw\fR	\fIrm16,ax\fR

f7 /6	\fBdivw\fR	\fIrm16\fR

f7 /6	\fBdivl\fR	\fIrm32,eax\fR

f7 /6	\fBdivl\fR	\fIrm32\fR

.sp \n(pDu

c8	\fBenter\fR	\fIimm8,imm16\fR	Make stack frame for procedure

d9 f0	\fBf2xm1\fR		ST = 2 ** ST - 1

d9 e1	\fBfabs\fR		ST = abs(ST)

.sp \n(pDu

\fBfadd\fR		Floating add

d8 /0	\fBfadds\fR	\fIm32\fR

dc /0	\fBfaddl\fR	\fIm64\fR

d8 c0 +r	\fBfadd\fR	\fIfpreg,st0\fR

d8 c0 +r	\fBfadd\fR	\fIfpreg\fR

de c1	\fBfadd\fR

dc c0 +r	\fBfadd\fR	\fIst0,fpreg\fR

.sp \n(pDu

\fBfaddp\fR		Floating add and pop

de c0 +r	\fBfaddp\fR	\fIst0,fpreg\fR

de c0 +r	\fBfaddp\fR	\fIfpreg\fR

de c1	\fBfaddp\fR

.sp \n(pDu

df /4	\fBfbld\fR	\fIm80\fR	Load Binary Coded Decimal

df /6	\fBfbstp\fR	\fIm80\fR	Store Binary Coded Decimal and Pop

d9 e0	\fBfchs\fR		Change Floating Sign

9B db e2	\fBfclex\fR		Clear floating point exception flags

.sp \n(pDu

\fBfcom\fR		Floating Compare

d8 /2	\fBfcoms\fR	\fIm32\fR

dc /2	\fBfcoml\fR	\fIm64\fR

d8 d0 +r	\fBfcom\fR	\fIfpreg,st0\fR

d8 d0 +r	\fBfcom\fR	\fIfpreg\fR

d8 d1	\fBfcom\fR

.sp \n(pDu

\fBfcomp\fR		Floating Compare and Pop

d8 /3	\fBfcomps\fR	\fIm32\fR

dc /3	\fBfcompl\fR	\fIm64\fR

d8 d8 +r	\fBfcomp\fR	\fIfpreg\fR

d8 d9	\fBfcomp\fR

.sp \n(pDu

de d9	\fBfcompp\fR		Floating Compare and pop twice

d9 ff	\fBfcos\fR		Cosine

d9 f6	\fBfdecstp\fR		Decrement Stack Top Pointer

.sp \n(pDu

\fBfdiv\fR		Floating divide

d8 /6	\fBfdivs\fR	\fIm32\fR

dc /6	\fBfdivl\fR	\fIm64\fR

d8 f0 +r	\fBfdiv\fR	\fIfpreg,st0\fR

d8 f0 +r	\fBfdiv\fR	\fIfpreg\fR

de f1	\fBfdiv\fR

dc f0 +r	\fBfdiv\fR	\fIst0,fpreg\fR

.sp \n(pDu

\fBfdivp\fR		Floating divide and pop

de f0 +r	\fBfdivp\fR	\fIst0,fpreg\fR

de f0 +r	\fBfdivp\fR	\fIfpreg\fR

de f1	\fBfdivp\fR

.sp \n(pDu

\fBfdivr\fR		Reverse floating divide

d8 /7	\fBfdivrs\fR	\fIm32\fR

dc /7	\fBfdivrl\fR	\fIm64\fR

d8 f8 +r	\fBfdivr\fR	\fIfpreg,st0\fR

d8 f8 +r	\fBfdivr\fR	\fIfpreg\fR

de f9	\fBfdivr\fR

dc f8 +r	\fBfdivr\fR	\fIst0,fpreg\fR

.sp \n(pDu

\fBfdivrp\fR		Reverse floating divide and pop

de f8 +r	\fBfdivrp\fR	\fIst0,fpreg\fR

de f8 +r	\fBfdivrp\fR	\fIfpreg\fR

de f9	\fBfdivrp\fR

.sp \n(pDu

dd c0 +r	\fBffree\fR	\fIfpreg\fR	Free Floating Point Register

.sp \n(pDu

\fBfiadd\fR		Add integer to float

da /0	\fBfiaddl\fR	\fIm32\fR

de /0	\fBfiadds\fR	\fIm16\fR

.sp \n(pDu

\fBficom\fR		Compare float to integer

da /2	\fBficoml\fR	\fIm32\fR

de /2	\fBficoms\fR	\fIm16\fR

.sp \n(pDu

\fBficomp\fR		Compare float to integer and pop

da /3	\fBficompl\fR	\fIm32\fR

de /3	\fBficomps\fR	\fIm16\fR

.sp \n(pDu

\fBfidiv\fR		Divide float by integer

da /6	\fBfidivl\fR	\fIm32\fR

de /6	\fBfidivs\fR	\fIm16\fR

.sp \n(pDu

\fBfidivr\fR		Reverse divide integer by float

da /7	\fBfidivrl\fR	\fIm32\fR

de /7	\fBfidivrs\fR	\fIm16\fR

.sp \n(pDu

\fBfild\fR		Load integer

db /0	\fBfildl\fR	\fIm32\fR

df /0	\fBfilds\fR	\fIm16\fR

df /5	\fBfildll\fR	\fIm64\fR

.sp \n(pDu

\fBfimul\fR		Multiply integer to float

da /1	\fBfimull\fR	\fIm32\fR

de /1	\fBfimuls\fR	\fIm16\fR

.sp \n(pDu

d9 f7	\fBfincstp\fR		Increment Stack Top Pointer

9B db e3	\fBfinit\fR		Initialize Floating Point Unit

.sp \n(pDu

\fBfist\fR		Store integer

db /2	\fBfistl\fR	\fIm32\fR

df /2	\fBfists\fR	\fIm16\fR

.sp \n(pDu

\fBfistp\fR		Store integer and pop

db /3	\fBfistpl\fR	\fIm32\fR

df /3	\fBfistps\fR	\fIm16\fR

df /7	\fBfistpll\fR	\fIm32\fR

.sp \n(pDu

\fBfisub\fR		Subtract integer from float

da /4	\fBfisubl\fR	\fIm32\fR

de /4	\fBfisubs\fR	\fIm16\fR

.sp \n(pDu

\fBfisubr\fR		Reverse subtract integer from float

da /5	\fBfisubrl\fR	\fIm32\fR

de /5	\fBfisubrs\fR	\fIm16\fR

.sp \n(pDu

\fBfld\fR		Load Real

d9 c0 +r	\fBfld\fR	\fIfpreg\fR

d9 /0	\fBflds\fR	\fIm32\fR

dd /0	\fBfldl\fR	\fIm32\fR

db /5	\fBfldt\fR	\fIm64\fR

.sp \n(pDu

d9 e8	\fBfld1\fR		Load Constant 1

d9 /5	\fBfldcw\fR	\fIm32\fR	Load Floating Point Control Word

d9 /4	\fBfldenv\fR	\fIm32\fR	Load FPU Environment

d9 ea	\fBfldl2e\fR		Load Constant log(e) base 2

d9 e9	\fBfldl2t\fR		Load Constant log(10) base 2

d9 ec	\fBfldlg2\fR		Load Constant log(2) base 10

d9 ed	\fBfldln2\fR		Load Constant log(2) base e

d9 eb	\fBfldpi\fR		Load Constant pi

d9 ee	\fBfldz\fR		Load Constant 0.0

.sp \n(pDu

\fBfmul\fR		Floating multiply

d8 /1	\fBfmuls\fR	\fIm32\fR

dc /1	\fBfmull\fR	\fIm64\fR

d8 c8 +r	\fBfmul\fR	\fIfpreg,st0\fR

d8 c8 +r	\fBfmul\fR	\fIfpreg\fR

de c9	\fBfmul\fR

dc c8 +r	\fBfmul\fR	\fIst0,fpreg\fR

.sp \n(pDu

\fBfmulp\fR		Floating multiply and pop

de c8 +r	\fBfmulp\fR	\fIst0,fpreg\fR

de c8 +r	\fBfmulp\fR	\fIfpreg\fR

de c9	\fBfmulp\fR

.sp \n(pDu

db e2	\fBfnclex\fR		Clear floating point exception flags no wait

db e3	\fBfninit\fR		Initialize Floating Point Unit no wait

d9 d0	\fBfnop\fR		No Operation

dd /6	\fBfnsave\fR	\fIm32\fR	Store FPU State no wait

d9 /7	\fBfnstcw\fR	\fIm32\fR	Store Control Word no wait

d9 /6	\fBfnstenv\fR	\fIm32\fR	Store FPU Environment no wait

.sp \n(pDu

\fBfnstsw\fR		Store Status Word no wait

dd /7	\fBfnstsw\fR	\fIm16\fR

df e0	\fBfnstsw\fR	\fIax\fR

.sp \n(pDu

d9 f3	\fBfpatan\fR		Partial Arctangent

d9 f8	\fBfprem\fR		Partial Remainder toward 0

d9 f5	\fBfprem1\fR		Partial Remainder < 1/2 modulus

d9 f2	\fBfptan\fR		Partial Tangent

d9 fc	\fBfrndint\fR		Round To Integer

dd /4	\fBfrstor\fR	\fIm32\fR	Resore FPU State

db f4	\fBfrstpm\fR		set 287XL real mode (nop for 387/486)

9B dd /6	\fBfsave\fR	\fIm32\fR	Store FPU State

d9 fd	\fBfscale\fR		Scale

db e4	\fBfsetpm\fR		set 287 protected mode (nop for 387/486)

d9 fe	\fBfsin\fR		Sine

d9 fb	\fBfsincos\fR		Sine and Cosine

d9 fa	\fBfsqrt\fR		Square Root

.sp \n(pDu

\fBfst\fR		Store Real

dd d0 +r	\fBfst\fR	\fIfpreg\fR

d9 /2	\fBfsts\fR	\fIm32\fR

dd /2	\fBfstl\fR	\fIm64\fR

.sp \n(pDu

9B d9 /7	\fBfstcw\fR	\fIm32\fR	Store Control Word

9B d9 /6	\fBfstenv\fR	\fIm32\fR	Store FPU Environment

.sp \n(pDu

\fBfstp\fR		Store Real and pop

dd d8 +r	\fBfstp\fR	\fIfpreg\fR

d9 /3	\fBfstps\fR	\fIm32\fR

dd /3	\fBfstpl\fR	\fIm64\fR

db /7	\fBfstpt\fR	\fIm80\fR

.sp \n(pDu

\fBfstsw\fR		Store Status Word

9B dd /7	\fBfstsw\fR	\fIm16\fR

9B df e0	\fBfstsw\fR	\fIax\fR

.sp \n(pDu

\fBfsub\fR		Floating subtract

d8 /4	\fBfsubs\fR	\fIm32\fR

dc /4	\fBfsubl\fR	\fIm64\fR

d8 e0 +r	\fBfsub\fR	\fIfpreg,st0\fR

d8 e0 +r	\fBfsub\fR	\fIfpreg\fR

de e1	\fBfsub\fR

dc e0 +r	\fBfsub\fR	\fIst0,fpreg\fR

.sp \n(pDu

\fBfsubp\fR		Floating subtract and pop

de e0 +r	\fBfsubp\fR	\fIst0,fpreg\fR

de e0 +r	\fBfsubp\fR	\fIfpreg\fR

de e1	\fBfsubp\fR

.sp \n(pDu

\fBfsubr\fR		Reverse floating subtract

d8 /5	\fBfsubrs\fR	\fIm32\fR

dc /5	\fBfsubrl\fR	\fIm64\fR

d8 e8 +r	\fBfsubr\fR	\fIfpreg,st0\fR

d8 e8 +r	\fBfsubr\fR	\fIfpreg\fR

de e9	\fBfsubr\fR

dc e8 +r	\fBfsubr\fR	\fIst0,fpreg\fR

.sp \n(pDu

\fBfsubrp\fR		Reverse floating subtract and pop

de e8 +r	\fBfsubrp\fR	\fIst0,fpreg\fR

de e8 +r	\fBfsubrp\fR	\fIfpreg\fR

de e9	\fBfsubrp\fR

.sp \n(pDu

d9 e4	\fBftst\fR		Test

.sp \n(pDu

\fBfucom\fR		Unordered compare real

dd e0 +r	\fBfucom\fR	\fIst0,fpreg\fR

dd e0 +r	\fBfucom\fR	\fIfpreg\fR

dd e1	\fBfucom\fR

.sp \n(pDu

\fBfucomp\fR		Unordered compare real and pop

dd e8 +r	\fBfucomp\fR	\fIst0,fpreg\fR

dd e8 +r	\fBfucomp\fR	\fIfpreg\fR

dd e9	\fBfucomp\fR

.sp \n(pDu

da e9	\fBfucompp\fR		Unordered compare %st %st1 and pop twice

9b	\fBfwait\fR		Wait for coprocessor

d9 e5	\fBfxam\fR		Examine

.sp \n(pDu

\fBfxch\fR		Floating exchange

d9 c8 +r	\fBfxch\fR	\fIst0,fpreg\fR

d9 c8 +r	\fBfxch\fR	\fIfpreg,st0\fR

d9 c8 +r	\fBfxch\fR	\fIfpreg\fR

d9 c9	\fBfxch\fR

.sp \n(pDu

d9 f4	\fBfxtract\fR		Extract Exponent and Significand

d9 f1	\fBfyl2x\fR		%st1 * log(%st) base 2

d9 f9	\fBfyl2xp1\fR		%st1 * log(%st + 1.0) base 2

f4	\fBhlt\fR		Halt

ff /2	\fBicall\fR	\fIrm32\fR	Call indirect

.sp \n(pDu

\fBidiv\fR		Signed divide

f6 /7	\fBidivb\fR	\fIrm8,al\fR

f6 /7	\fBidivb\fR	\fIrm8\fR

f7 /7	\fBidivw\fR	\fIrm16,ax\fR

f7 /7	\fBidivw\fR	\fIrm16\fR

f7 /7	\fBidivl\fR	\fIrm32,eax\fR

f7 /7	\fBidivl\fR	\fIrm32\fR

.sp \n(pDu

ff /4	\fBijmp\fR	\fIrm32\fR	Jump indirect

ff /3	\fBilcall\fR	\fIm32\fR	Long call indirect

ff /5	\fBiljmp\fR	\fIm32\fR	Long jump indirect

.sp \n(pDu

\fBimul\fR		Signed multiply

f6 /5	\fBimulb\fR	\fIrm8\fR

f7 /5	\fBimulw\fR	\fIrm16\fR

f7 /5	\fBimull\fR	\fIrm32\fR

0F af /r	\fBimulw\fR	\fIrm16,r16\fR

0F af /r	\fBimull\fR	\fIrm32,r32\fR

6b	\fBimulw\fR	\fIimm8s,rm16,r16\fR

6b	\fBimull\fR	\fIimm8s,rm32,r32\fR

6b /r	\fBimulw\fR	\fIimm8s,r16\fR

6b /r	\fBimull\fR	\fIimm8s,r32\fR

69	\fBimulw\fR	\fIimm16,rm16,r16\fR

69	\fBimull\fR	\fIimm32,rm32,r32\fR

69 /r	\fBimulw\fR	\fIimm16,r16\fR

69 /r	\fBimull\fR	\fIimm32,r32\fR

.sp \n(pDu

\fBin\fR		Input from port

e4	\fBinb\fR	\fIimm8\fR

e5	\fBinw\fR	\fIimm8\fR

e5	\fBinl\fR	\fIimm8\fR

ec	\fBinb\fR	\fI(dx)\fR

ed	\fBinw\fR	\fI(dx)\fR

ed	\fBinl\fR	\fI(dx)\fR

.sp \n(pDu

\fBinc\fR		Increment by one

40 +r	\fBincw\fR	\fIr16\fR

40 +r	\fBincl\fR	\fIr32\fR

fe /0	\fBincb\fR	\fIrm8\fR

ff /0	\fBincw\fR	\fIrm16\fR

ff /0	\fBincl\fR	\fIrm32\fR

.sp \n(pDu

6c	\fBinsb\fR		Input byte from port into ES:(E)DI

6c	\fBinsb\fR	\fI(dx)\fR	Input byte from port into ES:(E)DI

6d	\fBinsl\fR		Input long from port into ES:(E)DI

6d	\fBinsl\fR	\fI(dx)\fR	Input long from port into ES:(E)DI

6d	\fBinsw\fR		Input word from port into ES:(E)DI

6d	\fBinsw\fR	\fI(dx)\fR	Input word from port into ES:(E)DI

cc	\fBint\fR	\fIcon3\fR	Interrupt 3

cd	\fBint\fR	\fIimm8\fR	Interrupt

ce	\fBinto\fR		Int 4 if overflow is 1

cf	\fBiret\fR		Interrupt return

cf	\fBiretd\fR		Different mode different opcode ?

07	\fBja\fR	\fIreli\fR	Jump if above

03	\fBjae\fR	\fIreli\fR	Jump if above or equal

02	\fBjb\fR	\fIreli\fR	Jump if below

06	\fBjbe\fR	\fIreli\fR	Jump if below or equal

02	\fBjc\fR	\fIreli\fR	Jump if carry

e3	\fBjcxz\fR	\fIrel8\fR	Jump if CX is zero

04	\fBje\fR	\fIreli\fR	Jump if equal

e3	\fBjecxz\fR	\fIrel8\fR	Jump if CX is zero

0f	\fBjg\fR	\fIreli\fR	Jump if greater

0d	\fBjge\fR	\fIreli\fR	Jump if greater or equal

0c	\fBjl\fR	\fIreli\fR	Jump if less

0e	\fBjle\fR	\fIreli\fR	Jump if less or equal

e9	\fBjmp\fR	\fIreli\fR	Jump absolute

06	\fBjna\fR	\fIreli\fR	Jump if not above

02	\fBjnae\fR	\fIreli\fR	Jump if not above or equal

03	\fBjnb\fR	\fIreli\fR	Jump if not below

07	\fBjnbe\fR	\fIreli\fR	Jump if not below or equal

03	\fBjnc\fR	\fIreli\fR	Jump if no carry

05	\fBjne\fR	\fIreli\fR	Jump if not equal

0e	\fBjng\fR	\fIreli\fR	Jump if not greater

0c	\fBjnge\fR	\fIreli\fR	Jump if not greater or equal

0d	\fBjnl\fR	\fIreli\fR	Jump if not less

0f	\fBjnle\fR	\fIreli\fR	Jump if not less or equal

01	\fBjno\fR	\fIreli\fR	Jump if not overflow

0b	\fBjnp\fR	\fIreli\fR	Jump if not parity

09	\fBjns\fR	\fIreli\fR	Jump if not sign

05	\fBjnz\fR	\fIreli\fR	Jump if not zero

00	\fBjo\fR	\fIreli\fR	Jump if overflow

0a	\fBjp\fR	\fIreli\fR	Jump if parity

0a	\fBjpe\fR	\fIreli\fR	Jump if parity even

0b	\fBjpo\fR	\fIreli\fR	Jump if parity odd

08	\fBjs\fR	\fIreli\fR	Jump if sign

04	\fBjz\fR	\fIreli\fR	Jump if zero

04	\fBjz\fR	\fIreli\fR	Jump if zero

9f	\fBlahf\fR		Load flags into AH register

.sp \n(pDu

\fBlar\fR		Load access rights byte

0F 02 /r	\fBlarw\fR	\fIrm16,r16\fR

0F 02 /r	\fBlarl\fR	\fIrm32,r32\fR

.sp \n(pDu

9a	\fBlcall\fR	\fIimm16x,imm32\fR	Long call

.sp \n(pDu

\fBlds\fR		load full pointer DS:r16

c5 /r	\fBldsw\fR	\fIm16,r16\fR

c5 /r	\fBldsl\fR	\fIm32,r32\fR

.sp \n(pDu

\fBlea\fR		Load effective address

8d /r	\fBleaw\fR	\fIm16,r16\fR

8d /r	\fBleal\fR	\fIm32,r32\fR

.sp \n(pDu

c9	\fBleave\fR		High level procedure exit

.sp \n(pDu

\fBles\fR		Load full pointer ES:r16

c4 /r	\fBlesw\fR	\fIm16,r16\fR

c4 /r	\fBlesl\fR	\fIm32,r32\fR

.sp \n(pDu

\fBlfs\fR		Load full pointer FS:r16

0F b4 /r	\fBlfsw\fR	\fIm16,r16\fR

0F b4 /r	\fBlfsl\fR	\fIm32,r32\fR

.sp \n(pDu

\fBlgdt\fR		Load m into DGTR

0F 01 /2	\fBlgdtw\fR	\fIm16\fR

0F 01 /2	\fBlgdtl\fR	\fIm32\fR

.sp \n(pDu

\fBlgs\fR		Load full pointer GS:r16

0F b5 /r	\fBlgsw\fR	\fIm16,r16\fR

0F b5 /r	\fBlgsl\fR	\fIm32,r32\fR

.sp \n(pDu

\fBlidt\fR		Load m into IDTR

0F 01 /3	\fBlidtw\fR	\fIm16\fR

0F 01 /3	\fBlidtl\fR	\fIm32\fR

.sp \n(pDu

ea	\fBljmp\fR	\fIimm16x,imm32\fR	Long jump

0F 00 /2	\fBlldt\fR	\fIrm16\fR	Load local descriptor table register

0F 01 /6	\fBlmsw\fR	\fIrm16\fR	Load machine status word

f0	\fBlock\fR		Assert lock signal for next instruction

ac	\fBlodsb\fR		Load string operand byte

ad	\fBlodsl\fR		Load string operand long

ad	\fBlodsw\fR		Load string operand word

e2	\fBloop\fR	\fIrel8\fR	Dec count jmp if count <> 0

e1	\fBloope\fR	\fIrel8\fR	Dec count jmp if count <> 0 and ZF = 1

e0	\fBloopne\fR	\fIrel8\fR	Dec count jmp if count <> 0 and ZF = 0

e0	\fBloopnz\fR	\fIrel8\fR	Dec count jmp if count <> 0 and ZF = 0

e1	\fBloopz\fR	\fIrel8\fR	Dec count jmp if count <> 0 and ZF = 1

cb	\fBlret\fR		Far return

ca	\fBlret\fR	\fIimm16\fR	Far return pop imm16 bytes of parms

.sp \n(pDu

\fBlsl\fR		Load segment limit

0F 03 /r	\fBlslw\fR	\fIrm16,r16\fR

0F 03 /r	\fBlsll\fR	\fIrm32,r32\fR

.sp \n(pDu

\fBlss\fR		Load full pointer SS:r16

0F b2 /r	\fBlssw\fR	\fIm16,r16\fR

0F b2 /r	\fBlssl\fR	\fIm32,r32\fR

.sp \n(pDu

0F 00 /3	\fBltr\fR	\fIrm16\fR	Load task register

.sp \n(pDu

\fBmov\fR		Move data

a0	\fBmovb\fR	\fImoffs,al\fR

a1	\fBmovw\fR	\fImoffs,ax\fR

a1	\fBmovl\fR	\fImoffs,eax\fR

a2	\fBmovb\fR	\fIal,moffs\fR

a3	\fBmovw\fR	\fIax,moffs\fR

a3	\fBmovl\fR	\fIeax,moffs\fR

8a /r	\fBmovb\fR	\fIrm8,r8\fR

8b /r	\fBmovw\fR	\fIrm16,r16\fR

8b /r	\fBmovl\fR	\fIrm32,r32\fR

88 /r	\fBmovb\fR	\fIr8,rm8\fR

89 /r	\fBmovw\fR	\fIr16,rm16\fR

89 /r	\fBmovl\fR	\fIr32,rm32\fR

8c /r	\fBmovw\fR	\fIsreg,rm16\fR

8e /r	\fBmovw\fR	\fIrm16,sreg\fR

b0 +r	\fBmovb\fR	\fIimm8,r8\fR

b8 +r	\fBmovw\fR	\fIimm16,r16\fR

b8 +r	\fBmovl\fR	\fIimm32,r32\fR

c6	\fBmovb\fR	\fIimm8,rm8\fR

c7	\fBmovw\fR	\fIimm16,rm16\fR

c7	\fBmovl\fR	\fIimm32,rm32\fR

0F 20 /r	\fBmovl\fR	\fIctlreg,r32\fR

0F 22 /r	\fBmovl\fR	\fIr32,ctlreg\fR

0F 21 /r	\fBmovl\fR	\fIdbreg,r32\fR

0F 23 /r	\fBmovl\fR	\fIr32,dbreg\fR

0F 24 /r	\fBmovl\fR	\fItreg,r32\fR

0F 26 /r	\fBmovl\fR	\fIr32,treg\fR

.sp \n(pDu

a4	\fBmovsb\fR		Move bytes

a5	\fBmovsl\fR		Move longs

a5	\fBmovsw\fR		Move words

.sp \n(pDu

\fBmovsx\fR		Move with sign extend

0F be /r	\fBmovsxb\fR	\fIrm8,r16\fR

0F be /r	\fBmovsxb\fR	\fIrm8,r32\fR

0F bf /r	\fBmovsxw\fR	\fIrm16,r32\fR

0F be /r	\fBmovsbw\fR	\fIrm8,r16\fR

0F be /r	\fBmovsbl\fR	\fIrm8,r32\fR

0F bf /r	\fBmovswl\fR	\fIrm16,r32\fR

.sp \n(pDu

\fBmovzx\fR		Move with zero extend

0F b6 /r	\fBmovzxb\fR	\fIrm8,r16\fR

0F b6 /r	\fBmovzxb\fR	\fIrm8,r32\fR

0F b7 /r	\fBmovzxw\fR	\fIrm16,r32\fR

0F b6 /r	\fBmovzbw\fR	\fIrm8,r16\fR

0F b6 /r	\fBmovzbl\fR	\fIrm8,r32\fR

0F b7 /r	\fBmovzwl\fR	\fIrm16,r32\fR

.sp \n(pDu

\fBmul\fR		Unsigned multiply

f6 /4	\fBmulb\fR	\fIrm8,al\fR

f6 /4	\fBmulb\fR	\fIrm8\fR

f7 /4	\fBmulw\fR	\fIrm16,ax\fR

f7 /4	\fBmulw\fR	\fIrm16\fR

f7 /4	\fBmull\fR	\fIrm32,eax\fR

f7 /4	\fBmull\fR	\fIrm32\fR

.sp \n(pDu

\fBneg\fR		Negate

f6 /3	\fBnegb\fR	\fIrm8\fR

f7 /3	\fBnegw\fR	\fIrm16\fR

f7 /3	\fBnegl\fR	\fIrm32\fR

.sp \n(pDu

90	\fBnop\fR		No operation

.sp \n(pDu

\fBnot\fR		Invert bits

f6 /2	\fBnotb\fR	\fIrm8\fR

f7 /2	\fBnotw\fR	\fIrm16\fR

f7 /2	\fBnotl\fR	\fIrm32\fR

.sp \n(pDu

\fBor\fR		Logical inclusive OR

83 /1	\fBorl\fR	\fIimm8s,rm32\fR

83 /1	\fBorw\fR	\fIimm8s,rm16\fR

0c	\fBorb\fR	\fIimm8,al\fR

0d	\fBorw\fR	\fIimm16,ax\fR

0d	\fBorl\fR	\fIimm32,eax\fR

0d	\fBorl\fR	\fIimm32\fR

80 /1	\fBorb\fR	\fIimm8,rm8\fR

81 /1	\fBorw\fR	\fIimm16,rm16\fR

81 /1	\fBorl\fR	\fIimm32,rm32\fR

0a /r	\fBorb\fR	\fIrm8,r8\fR

0b /r	\fBorw\fR	\fIrm16,r16\fR

0b /r	\fBorl\fR	\fIrm32,r32\fR

08 /r	\fBorb\fR	\fIr8,rm8\fR

09 /r	\fBorw\fR	\fIr16,rm16\fR

09 /r	\fBorl\fR	\fIr32,rm32\fR

.sp \n(pDu

\fBout\fR		Output from port

e6	\fBoutb\fR	\fIimm8\fR

e7	\fBoutw\fR	\fIimm8\fR

e7	\fBoutl\fR	\fIimm8\fR

ee	\fBoutb\fR	\fI(dx)\fR

ef	\fBoutw\fR	\fI(dx)\fR

ef	\fBoutl\fR	\fI(dx)\fR

.sp \n(pDu

6e	\fBoutsb\fR		Output byte to port into ES:(E)DI

6f	\fBoutsl\fR		Output long to port into ES:(E)DI

6f	\fBoutsw\fR		Output word to port into ES:(E)DI

.sp \n(pDu

\fBpop\fR		Pop a word from the stack

58 +r	\fBpopw\fR	\fIr16\fR

58 +r	\fBpopl\fR	\fIr32\fR

1f	\fBpopw\fR	\fIds\fR

07	\fBpopw\fR	\fIes\fR

17	\fBpopw\fR	\fIss\fR

0F a1	\fBpopw\fR	\fIfs\fR

0F a9	\fBpopw\fR	\fIgs\fR

8f /0	\fBpopw\fR	\fIm16\fR

8f /0	\fBpopl\fR	\fIm32\fR

.sp \n(pDu

\fBpopa\fR		Pop all

61	\fBpopaw\fR

61	\fBpopal\fR

.sp \n(pDu

\fBpopf\fR		Pop stack into flags

9d	\fBpopfw\fR

9d	\fBpopfl\fR

.sp \n(pDu

\fBpush\fR		Push a word on the stack

50 +r	\fBpushw\fR	\fIr16\fR

50 +r	\fBpushl\fR	\fIr32\fR

6a	\fBpushb\fR	\fIimm8s\fR

68	\fBpushw\fR	\fIimm16\fR

68	\fBpushl\fR	\fIimm32\fR

0e	\fBpushw\fR	\fIcs\fR

1e	\fBpushw\fR	\fIds\fR

06	\fBpushw\fR	\fIes\fR

16	\fBpushw\fR	\fIss\fR

0F a0	\fBpushw\fR	\fIfs\fR

0F a8	\fBpushw\fR	\fIgs\fR

ff /6	\fBpushw\fR	\fIm16\fR

ff /6	\fBpushl\fR	\fIm32\fR

.sp \n(pDu

\fBpusha\fR		Push all

60	\fBpushaw\fR

60	\fBpushal\fR

.sp \n(pDu

\fBpushf\fR		Push flags

9c	\fBpushfw\fR

9c	\fBpushfl\fR

.sp \n(pDu

\fBrcl\fR		Rotate carry left

d0 /2	\fBrclb\fR	\fIcon1,rm8\fR

d0 /2	\fBrclb\fR	\fIrm8\fR

d2 /2	\fBrclb\fR	\fIcl,rm8\fR

c0 /2	\fBrclb\fR	\fIimm8,rm8\fR

d1 /2	\fBrclw\fR	\fIcon1,rm16\fR

d1 /2	\fBrclw\fR	\fIrm16\fR

d3 /2	\fBrclw\fR	\fIcl,rm16\fR

c1 /2	\fBrclw\fR	\fIimm8,rm16\fR

d1 /2	\fBrcll\fR	\fIcon1,rm32\fR

d1 /2	\fBrcll\fR	\fIrm32\fR

d3 /2	\fBrcll\fR	\fIcl,rm32\fR

c1 /2	\fBrcll\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBrcr\fR		Rotate carry right

d0 /3	\fBrcrb\fR	\fIcon1,rm8\fR

d0 /3	\fBrcrb\fR	\fIrm8\fR

d2 /3	\fBrcrb\fR	\fIcl,rm8\fR

c0 /3	\fBrcrb\fR	\fIimm8,rm8\fR

d1 /3	\fBrcrw\fR	\fIcon1,rm16\fR

d1 /3	\fBrcrw\fR	\fIrm16\fR

d3 /3	\fBrcrw\fR	\fIcl,rm16\fR

c1 /3	\fBrcrw\fR	\fIimm8,rm16\fR

d1 /3	\fBrcrl\fR	\fIcon1,rm32\fR

d1 /3	\fBrcrl\fR	\fIrm32\fR

d3 /3	\fBrcrl\fR	\fIcl,rm32\fR

c1 /3	\fBrcrl\fR	\fIimm8,rm32\fR

.sp \n(pDu

f3	\fBrep\fR		rep following instruction CX times

f3	\fBrepe\fR		repe following instruction CX times or eq

f2	\fBrepne\fR		repne following instruction CX times or ne

f2	\fBrepnz\fR		alternate name for repnz

f3	\fBrepz\fR		alternate name for repe

c3	\fBret\fR		Return

c2	\fBret\fR	\fIimm16\fR	Return pop imm16 bytes of parms

.sp \n(pDu

\fBrol\fR		Rotate left

d0 /0	\fBrolb\fR	\fIcon1,rm8\fR

d0 /0	\fBrolb\fR	\fIrm8\fR

d2 /0	\fBrolb\fR	\fIcl,rm8\fR

c0 /0	\fBrolb\fR	\fIimm8,rm8\fR

d1 /0	\fBrolw\fR	\fIcon1,rm16\fR

d1 /0	\fBrolw\fR	\fIrm16\fR

d3 /0	\fBrolw\fR	\fIcl,rm16\fR

c1 /0	\fBrolw\fR	\fIimm8,rm16\fR

d1 /0	\fBroll\fR	\fIcon1,rm32\fR

d1 /0	\fBroll\fR	\fIrm32\fR

d3 /0	\fBroll\fR	\fIcl,rm32\fR

c1 /0	\fBroll\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBror\fR		Rotate right

d0 /1	\fBrorb\fR	\fIcon1,rm8\fR

d0 /1	\fBrorb\fR	\fIrm8\fR

d2 /1	\fBrorb\fR	\fIcl,rm8\fR

c0 /1	\fBrorb\fR	\fIimm8,rm8\fR

d1 /1	\fBrorw\fR	\fIcon1,rm16\fR

d1 /1	\fBrorw\fR	\fIrm16\fR

d3 /1	\fBrorw\fR	\fIcl,rm16\fR

c1 /1	\fBrorw\fR	\fIimm8,rm16\fR

d1 /1	\fBrorl\fR	\fIcon1,rm32\fR

d1 /1	\fBrorl\fR	\fIrm32\fR

d3 /1	\fBrorl\fR	\fIcl,rm32\fR

c1 /1	\fBrorl\fR	\fIimm8,rm32\fR

.sp \n(pDu

9e	\fBsahf\fR		Store AH into flags

.sp \n(pDu

\fBsal\fR		Shift arithmetic left

d0 /4	\fBsalb\fR	\fIcon1,rm8\fR

d0 /4	\fBsalb\fR	\fIrm8\fR

d2 /4	\fBsalb\fR	\fIcl,rm8\fR

c0 /4	\fBsalb\fR	\fIimm8,rm8\fR

d1 /4	\fBsalw\fR	\fIcon1,rm16\fR

d1 /4	\fBsalw\fR	\fIrm16\fR

d3 /4	\fBsalw\fR	\fIcl,rm16\fR

c1 /4	\fBsalw\fR	\fIimm8,rm16\fR

d1 /4	\fBsall\fR	\fIcon1,rm32\fR

d1 /4	\fBsall\fR	\fIrm32\fR

d3 /4	\fBsall\fR	\fIcl,rm32\fR

c1 /4	\fBsall\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBsar\fR		Shift arithmetic right

d0 /7	\fBsarb\fR	\fIcon1,rm8\fR

d0 /7	\fBsarb\fR	\fIrm8\fR

d2 /7	\fBsarb\fR	\fIcl,rm8\fR

c0 /7	\fBsarb\fR	\fIimm8,rm8\fR

d1 /7	\fBsarw\fR	\fIcon1,rm16\fR

d1 /7	\fBsarw\fR	\fIrm16\fR

d3 /7	\fBsarw\fR	\fIcl,rm16\fR

c1 /7	\fBsarw\fR	\fIimm8,rm16\fR

d1 /7	\fBsarl\fR	\fIcon1,rm32\fR

d1 /7	\fBsarl\fR	\fIrm32\fR

d3 /7	\fBsarl\fR	\fIcl,rm32\fR

c1 /7	\fBsarl\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBsbb\fR		Subtract with borrow

83 /3	\fBsbbl\fR	\fIimm8s,rm32\fR

83 /3	\fBsbbw\fR	\fIimm8s,rm16\fR

1c	\fBsbbb\fR	\fIimm8,al\fR

1d	\fBsbbw\fR	\fIimm16,ax\fR

1d	\fBsbbl\fR	\fIimm32,eax\fR

1d	\fBsbbl\fR	\fIimm32\fR

80 /3	\fBsbbb\fR	\fIimm8,rm8\fR

81 /3	\fBsbbw\fR	\fIimm16,rm16\fR

81 /3	\fBsbbl\fR	\fIimm32,rm32\fR

1a /r	\fBsbbb\fR	\fIrm8,r8\fR

1b /r	\fBsbbw\fR	\fIrm16,r16\fR

1b /r	\fBsbbl\fR	\fIrm32,r32\fR

18 /r	\fBsbbb\fR	\fIr8,rm8\fR

19 /r	\fBsbbw\fR	\fIr16,rm16\fR

19 /r	\fBsbbl\fR	\fIr32,rm32\fR

.sp \n(pDu

ae	\fBscasb\fR		Compare string bytes

af	\fBscasl\fR		Compare string longs

af	\fBscasw\fR		Compare string words

0F 97	\fBseta\fR	\fIrm8\fR	Set byte if above

0F 93	\fBsetae\fR	\fIrm8\fR	Set byte if above or equal

0F 92	\fBsetb\fR	\fIrm8\fR	Set byte if below

0F 96	\fBsetbe\fR	\fIrm8\fR	Set byte if below or equal

0F 92	\fBsetc\fR	\fIrm8\fR	Set byte if carry

0F 94	\fBsete\fR	\fIrm8\fR	Set byte if equal

0F 9f	\fBsetg\fR	\fIrm8\fR	Set byte if greater

0F 9d	\fBsetge\fR	\fIrm8\fR	Set byte if greater or equal

0F 9c	\fBsetl\fR	\fIrm8\fR	Set byte if less

0F 9e	\fBsetle\fR	\fIrm8\fR	Set byte if less or equal

0F 96	\fBsetna\fR	\fIrm8\fR	Set byte if not above

0F 92	\fBsetnae\fR	\fIrm8\fR	Set byte if not above or equal

0F 93	\fBsetnb\fR	\fIrm8\fR	Set byte if not below

0F 97	\fBsetnbe\fR	\fIrm8\fR	Set byte if not below or equal

0F 93	\fBsetnc\fR	\fIrm8\fR	Set byte if no carry

0F 95	\fBsetne\fR	\fIrm8\fR	Set byte if not equal

0F 9e	\fBsetng\fR	\fIrm8\fR	Set byte if not greater

0F 9c	\fBsetnge\fR	\fIrm8\fR	Set byte if not greater or equal

0F 9d	\fBsetnl\fR	\fIrm8\fR	Set byte if not less

0F 9f	\fBsetnle\fR	\fIrm8\fR	Set byte if not less or equal

0F 91	\fBsetno\fR	\fIrm8\fR	Set byte if not overflow

0F 9b	\fBsetnp\fR	\fIrm8\fR	Set byte if not parity

0F 99	\fBsetns\fR	\fIrm8\fR	Set byte if not sign

0F 95	\fBsetnz\fR	\fIrm8\fR	Set byte if not zero

0F 90	\fBseto\fR	\fIrm8\fR	Set byte if overflow

0F 9a	\fBsetp\fR	\fIrm8\fR	Set byte if parity

0F 9a	\fBsetpe\fR	\fIrm8\fR	Set byte if parity even

0F 9b	\fBsetpo\fR	\fIrm8\fR	Set byte if parity odd

0F 98	\fBsets\fR	\fIrm8\fR	Set byte if sign

0F 94	\fBsetz\fR	\fIrm8\fR	Set byte if zero

0F 94	\fBsetz\fR	\fIrm8\fR	Set byte if zero

0F 01 /0	\fBsgdt\fR	\fImem32\fR	Store gdtr

.sp \n(pDu

\fBshl\fR		Shift arithmetic left

d0 /4	\fBshlb\fR	\fIcon1,rm8\fR

d0 /4	\fBshlb\fR	\fIrm8\fR

d2 /4	\fBshlb\fR	\fIcl,rm8\fR

c0 /4	\fBshlb\fR	\fIimm8,rm8\fR

d1 /4	\fBshlw\fR	\fIcon1,rm16\fR

d1 /4	\fBshlw\fR	\fIrm16\fR

d3 /4	\fBshlw\fR	\fIcl,rm16\fR

c1 /4	\fBshlw\fR	\fIimm8,rm16\fR

d1 /4	\fBshll\fR	\fIcon1,rm32\fR

d1 /4	\fBshll\fR	\fIrm32\fR

d3 /4	\fBshll\fR	\fIcl,rm32\fR

c1 /4	\fBshll\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBshld\fR		Shift double precision left

0F a4	\fBshldw\fR	\fIimm8,r16,rm16\fR

0F a4	\fBshldl\fR	\fIimm8,r32,rm32\fR

0F a5	\fBshldw\fR	\fIcl,r16,rm16\fR

0F a5	\fBshldl\fR	\fIcl,r32,rm32\fR

.sp \n(pDu

\fBshr\fR		Shift right

d0 /5	\fBshrb\fR	\fIcon1,rm8\fR

d0 /5	\fBshrb\fR	\fIrm8\fR

d2 /5	\fBshrb\fR	\fIcl,rm8\fR

c0 /5	\fBshrb\fR	\fIimm8,rm8\fR

d1 /5	\fBshrw\fR	\fIcon1,rm16\fR

d1 /5	\fBshrw\fR	\fIrm16\fR

d3 /5	\fBshrw\fR	\fIcl,rm16\fR

c1 /5	\fBshrw\fR	\fIimm8,rm16\fR

d1 /5	\fBshrl\fR	\fIcon1,rm32\fR

d1 /5	\fBshrl\fR	\fIrm32\fR

d3 /5	\fBshrl\fR	\fIcl,rm32\fR

c1 /5	\fBshrl\fR	\fIimm8,rm32\fR

.sp \n(pDu

\fBshrd\fR		Shift double precision right

0F ac	\fBshrdw\fR	\fIimm8,r16,rm16\fR

0F ac	\fBshrdl\fR	\fIimm8,r32,rm32\fR

0F ad	\fBshrdw\fR	\fIcl,r16,rm16\fR

0F ad	\fBshrdl\fR	\fIcl,r32,rm32\fR

0F ad	\fBshrdw\fR	\fIr16,rm16\fR

0F ad	\fBshrdl\fR	\fIr32,rm32\fR

.sp \n(pDu

0F 01 /1	\fBsidt\fR	\fImem32\fR	Store idtr

0F 00 /0	\fBsldt\fR	\fIrm16\fR	Store ldtr to EA word

0F 01 /4	\fBsmsw\fR	\fIrm16\fR	Store machine status to EA word

f9	\fBstc\fR		Set carry flag

fd	\fBstd\fR		Clear direction flag

fb	\fBsti\fR		Set interrupt flag

aa	\fBstosb\fR		Store string byte

ab	\fBstosl\fR		Store string long

ab	\fBstosw\fR		Store string word

0F 00 /1	\fBstr\fR		Store task register

.sp \n(pDu

\fBsub\fR		Subtract

83 /5	\fBsubl\fR	\fIimm8s,rm32\fR

83 /5	\fBsubw\fR	\fIimm8s,rm16\fR

2c	\fBsubb\fR	\fIimm8,al\fR

2d	\fBsubw\fR	\fIimm16,ax\fR

2d	\fBsubl\fR	\fIimm32,eax\fR

2d	\fBsubl\fR	\fIimm32\fR

80 /5	\fBsubb\fR	\fIimm8,rm8\fR

81 /5	\fBsubw\fR	\fIimm16,rm16\fR

81 /5	\fBsubl\fR	\fIimm32,rm32\fR

2a /r	\fBsubb\fR	\fIrm8,r8\fR

2b /r	\fBsubw\fR	\fIrm16,r16\fR

2b /r	\fBsubl\fR	\fIrm32,r32\fR

28 /r	\fBsubb\fR	\fIr8,rm8\fR

29 /r	\fBsubw\fR	\fIr16,rm16\fR

29 /r	\fBsubl\fR	\fIr32,rm32\fR

.sp \n(pDu

\fBtest\fR		Logical compare

a8	\fBtestb\fR	\fIimm8,al\fR

a9	\fBtestw\fR	\fIimm16,ax\fR

a9	\fBtestl\fR	\fIimm32,eax\fR

a9	\fBtestl\fR	\fIimm32\fR

f6 /0	\fBtestb\fR	\fIimm8,rm8\fR

f7 /0	\fBtestw\fR	\fIimm16,rm16\fR

f7 /0	\fBtestl\fR	\fIimm32,rm32\fR

84 /r	\fBtestb\fR	\fIr8,rm8\fR

85 /r	\fBtestw\fR	\fIr16,rm16\fR

85 /r	\fBtestl\fR	\fIr32,rm32\fR

.sp \n(pDu

0F 00 /4	\fBverr\fR	\fIrm16\fR	Verify segment for read

0F 00 /5	\fBverw\fR	\fIrm16\fR	Verify segment for write

9b	\fBwait\fR		Wait for coprocessor

.sp \n(pDu

\fBxchg\fR		Exchange register

90 +r	\fBxchgw\fR	\fIr16,ax\fR

90 +r	\fBxchgw\fR	\fIax,r16\fR

90 +r	\fBxchgl\fR	\fIr32,eax\fR

90 +r	\fBxchgl\fR	\fIr32\fR

90 +r	\fBxchgl\fR	\fIeax,r32\fR

86 /r	\fBxchgb\fR	\fIrm8,r8\fR

87 /r	\fBxchgw\fR	\fIrm16,r16\fR

87 /r	\fBxchgl\fR	\fIrm32,r32\fR

86 /r	\fBxchgb\fR	\fIr8,rm8\fR

87 /r	\fBxchgw\fR	\fIr16,rm16\fR

87 /r	\fBxchgl\fR	\fIr32,rm32\fR

.sp \n(pDu

d7	\fBxlat\fR		Table lookup translation

d7	\fBxlatb\fR		Table lookup translation

.sp \n(pDu

\fBxor\fR		Logical exclusive OR

83 /6	\fBxorl\fR	\fIimm8s,rm32\fR

83 /6	\fBxorw\fR	\fIimm8s,rm16\fR

34	\fBxorb\fR	\fIimm8,al\fR

35	\fBxorw\fR	\fIimm16,ax\fR

35	\fBxorl\fR	\fIimm32,eax\fR

35	\fBxorl\fR	\fIimm32\fR

80 /6	\fBxorb\fR	\fIimm8,rm8\fR

81 /6	\fBxorw\fR	\fIimm16,rm16\fR

81 /6	\fBxorl\fR	\fIimm32,rm32\fR

32 /r	\fBxorb\fR	\fIrm8,r8\fR

33 /r	\fBxorw\fR	\fIrm16,r16\fR

33 /r	\fBxorl\fR	\fIrm32,r32\fR

30 /r	\fBxorb\fR	\fIr8,rm8\fR

31 /r	\fBxorw\fR	\fIr16,rm16\fR

31 /r	\fBxorl\fR	\fIr32,rm32\fR