The USRobotics Courier HST Modem

(c) Andrzej Taramina, 1988,1989

USR, Courier, HST are trademarks of USRobotics, Inc.

Telix (c) Colin Sampaleanu, Exis Inc.

INTRODUCTION

This document discusses some of the unique features of the HST

modem and gives more specific information on using the HST with

Telix 3.11 for Bulletin Board operations.  It is not deeply

technical, but major differences between the HST and other, more

conventional modems are covered.

HIGH SPEED PROTOCOLS AND MODEM COMPATIBILITY

The HST is fast, which is probably why you bought it in the

first place.  Without going into a philosophical discussion,

suffice it to say that there are many 9600bps modems available

commercially, and most of these modems use totally incompatible

high-speed communications protocols.  Your HST will NOT

communicate at 9600bps with 9600 modems made by Hayes, Microcom,

Fastcomm, etc... Although a 9600bps standard exists, modems

adhering to it are few, far between, and still quite expensive.

You may not like this, but that's the way it is.  The HST will,

however, communicate at lower speed with all modems supporting

the 1200/2400 standards, including other modems incompatible at

the higher speeds.  Don't waste the Sysop's time by asking him

why you can't download at 9600bps if his board is equipped with

a Hayes modem.  Chances are you'll end up in a heated debate as

to which modem is best. Progress is being made, however, on the

creation of an industry-wide 9600bps standard.  USR expect to

deliver a new modem early next year (1989) that will talk to all

common 9600 baud modems. Upgrades will be available to HST owners.

Also, when connected to another HST, the effective

modem-to-modem speed ("link rate") may exceed the nominal

9600bps rate by 10 to 30%, depending on line conditions and the

ability of your computer to send/receive data at the higher

speed.  File transfer rates of 1100-1200 cps between two HST

modems (characters per second) are not uncommon.

ERROR CORRECTION

At 9600bps, when communicating with another HST,  both modems

use sophisticated (and proprietary) error correction techniques

which insure that the data which was received is identical to

the data which was sent.  At lower speeds (1200/2400), the HST

relies on its built-in MNP firmware (Microcom Networking

Protocol) for error-free communications with other MNP-equipped

modems.  This effectively eliminates the need for conventional

"Software" error-correction implemented in software-only file

transfer protocols such as Xmodem, Xmodem/CRC, 1k-Xmodem (a.k.a.

Ymodem), Zmodem and many others.

To take advantage of the much faster and more efficient built-in

error correction, special file transfer protocols must be used.

These protocols do not perform any error-checking and

correction; they rely entirely on the modem to do so.  Such

protocols include Ymodem-G, Imodem, and the registered version

of Zmodem (DSZ, authored and copyrighted by Chuck Forsberg).

Remember that error-free transmissions take place ONLY if you

are connected to an HST (at 9600bps) or to an MNP-compatible

modem of any brand at 1200 or 2400bps.  If setup properly, the

HST will return a special "CONNECT xxxxx/ARQ" message upon

connect (where xxxxx is the connect speed)  which reflects the

fact that an error-free connection has been established with the

other modem.

FACTORS AFFECTING TRANSFER SPEED

There seems to be some widespread misunderstanding of the term

"Speed" as it applies to the HST and other "high-speed" modems.

How fast does it really go?  What is its limit?  More important,

what are the factors that limit the modem's speed?

It is all very simple, really.  One end of the modem talks to

the computer, while the other end talks to another modem.  The

speed you are really interested in is the LINK RATE, which is

the speed at which the two modems transmit data.

If the HST is setup to transmit/receive at the highest possible

link rate, the effective speed will depend on the following

physical factors:

1. The maximum speed that can be achieved by the other modem,

2. The maximum speed at which your computer can communicate with

your HST modem and

3. The maximum speed at which the OTHER computer can communicate

with ITS modem.

It is intuitively obvious that the effective modem-to-modem link

rate cannot possibly exceed the slowest of 1,2, or 3 above.

Unfortunately, you can only control item #2, the speed at which

your computer communicates with your HST modem.  This speed is

NOT determined by the modem, but by the following:

1. The type of computer and communications interface you are

using and

2. The communications software you are using.

PHONE LINE NOISE

In an HST-HST connection, extraneous noise characters will not

appear on the computer's screen.  This led to the false belief

that error-correcting modems eliminate noise.  They do not.

The modem's built-in error-correction firmware (ie, MNP) insures

that the data which is received is the same as the data that was

sent in a manner similar to software-only error-corrction

protocols such as CRC (Cyclical Redundancy Check).  Thus, the

modem themselves initiate the retransmission of blocks of data

which were affected by line noise.  Although the computer does

not see it, noise affects transmissions just as negatively and

results in a slowing down of the data transfer.  On the HST

modem, erratic blinking of the RD/SD lights may be an indication

of poor line quality.

DATA FRAME OPTIMIZATION (MNP4) AND DATA COMPRESSION (MNP5)

With release level 964 of the HST's ROM code, USR implemented

MNP level 4 (Data Frame Optimization) and MNP Level 5 (Data

Compression).  Although 964-level HST modems are fully

downward-compatible with earlier releases of the ROM (ie, 961,

963), these features can only be used with other 964-level (or

more recent) HST modems.

Control of MNP level 4 operation is available via S-register 15,

a new register implemented in release 964 of the HST ROM.  If

bit 4 (decimal value 16) is ON in this register, MNP level 4 is

DISABLED.   MNP level 4 operation is only effective at speeds

of 1200bps and above.

Data Compression (MNP level 5) may be a mixed blessing,

depending mostly on the type of data being transferred by the

HST modem.  If your HST is used primarily for BBS communications

and your file transfers are mostly ARChived files (processed by

SEA's ARC, Vern Buerg's ARCA, or Phil Katz's PKARC), data

compression by the modem is not only redundant, but it may

actually SLOW DOWN your file transfers.  Effectively, the MNP 5

data compression algorithms will attempt to replicate the work

already done by the ARC utility and will fail to reduce the size

of the data any further (it will likely INCREASE the size of

your ARC'ed files), resulting in increased transfer times.

Thus, the most efficient transfer or compressed files on

964-level HST modems occurs when data compression is turned off

(to disdable MNP5, use the AT&K0 command).

ASCII, Text, .EXE and .COM files may benefit greatly from data

compression, yielding transfer rates in the 1400-1900cps range.

To activate automatic data compression, use the AT&K1 command.

HARDWARE LIMITATIONS

How fast your computer can communicate with the modem is

influenced by many factors.  The HST modem itself can

communicate with the computer at a maximum rate of 19,200bps.

Earlier models of the IBM PC and compatibles utilized an

8088 processor running at 4.77Mhz, transmitted data internally

over an 8-bit wide data path, and their serial interfaces (to

which the modem is attached) also used an 8-bit processor chip.

These factors effectively limit the serial rate to

9600-10000bps.  So-called "Turbo" machines which use a

dual-speed 8088 or NEC V20 microprocessor, or some of the 8086

or V30 machines can achieve significantly faster serial speeds.

The new series of AT-class 80286 and 80386 based machines can

provide much faster serial speeds.

The original serial processor chip is usually an 8250, and is

rated at 9600 (some 8250s can run satisfactorily at 19200,

however).  The newer 16450, a plug replacement for the 8250, is

nominally rated at 19200.

Regardless of the speed of your computer and serial interface,

what's at the other end is just as important.  Running a 20Mhz

386 system and with serial interface set at 19200bps is of no

consequence if the system you are communicating with is limited

to a 9600bps serial speed.  The SLOWEST link in the entire

communication chain sets the pace for all the rest, folks.

Also, be aware that computer networks running multi-node

bulletin board systems have many other tasks to perform besides

communicating over telephone lines.  These other processes may

interfere with reliable communications at high speeds, and the

sysops may purposely slow down the serial interface speed to

prevent possible conflicts arising from this situation.  One of

the most common problems in this environment is an aborted upload

when concurrent disk updates on the network's file server take

precedence over communications interupts and cause loss of

incoming data.

Other factors which can negatively affect high speed

transmissions are not readily apparent at lower speeds.  A slow

hard disk, for example, can drop the effective data transfer

rate by a few percentage points.  You may want to try a RAM

disk, for example, which eliminates disk access as a limiting

factor to data transfer performance.

COMMUNICATIONS SOFTWARE

How fast your serial interface operates is a function of the

serial interface itself, since it has an upper speed limit, as

well as the communication software you use, since IT physically

sets the speed of your communications port.  In fact, the

communications software you use may be your worst enemy.

While many commercial communications packages are good and

offer an outstanding array of features, few of them measure up

to Telix in terms of raw data transfer performance, especially

with an HST-class modem.  A number of communications packages

limit the maximum serial rate to 9600bps, regardless of your

computer and serial interface's inherent abilities.  Telix

release 3.11 can set you serial speed all the way up to

19,200bps.

Furthermore, some otherwise good communications packages offer a

very limited choice of transmission protocols which cannot take

advantage of your computer and communications hardware.  Not

providing protocols such as Ymodem-G and Imodem, or an easy

interface to external protocols such as DSZ, severely limits the

effectiveness of your communications setup.  Your choice of

Telix is a good one, offering internal ZModem and many others along

with the most powerful script language around...stick with it!

Andrzej Taramina

58 Aquila Court

Rexdale, Ontario

Canada M9W 5J2

12/13/1988