# Introduction

The VFSM development tools is a set of programs to develop control systems. You can specify and test a control system without writing a single line of code.

## System specification

You specify the system using the VFSM Editor. The Editor contains a Project Manager, a State Machine Editor and a System Manager.

Project Manager
You organize your project with the Project Manager. The project contains files that describe state machines used by your system, io-units, vfsm-templates and system configuration. All files are managed by the Project Manager that keeps track of all changes and assures the system consistency.

State Machine Editor
A control system is built of several state machines and io-units. The State Machine Editor is a text editor to specify the state machine  behavior. In addition the State Machine Editor is used to specify vfsm-templates and io-units. The results of the specification can be exported producing executable files. The specification can be printed for documentation purposes.

System Manager
A system of state machines specified by the State Machine Editor must be adapted to the real control environment. i.e. all objects used by state machines are determined, for instance:

- state machine inputs are linked with appriopriate input signals,

- timer values are determined

- switchpoint for analog inputs are determined, etc.

## System test

You test the system using the VFSM Lab and the VFSM Monitor.

VFSM Lab