Connecting Networker NT  to DAS

# Installation

For a detail discription see DAS-server and ACI NT Installation and Configuration.

- Install DAS and ACI

- Configure DAS and ACI

- Start the AMU and the DAS-Server

- The connection from ACI to DAS must work (Try this by sending ‘dasadmin qversion’ )

- Allocate the drives for the NT clients (‘dasadmin allocd Drive UP clientname’)

- Install Networker as it is discribed in the Networker documentation

# Configuration

## For the detail description have a look in the Networker Administration Guide.

The conditions for the following description is that the SCSI-connections to the drives are correct.

How many devices can be supported from networker, depends on the networker server configuration. See parallelism ability of the networker server.

- Define the media type in the environment variable DAS_MEDIUM at

‘Control Panel/System/Environment’.

Have a look in the DAS Administration Guide to find the correct value.

- Add a Emass Silo library to Networker

- change to the directory ‘networker installation directory\nsr\bin’

- type jbconfig

- follow the instructions

Choose a STL Silo library as jukebox and DAS Silo as the Silo types.

Be sure that you know which is the hostname where the DAS-Server is running and where the STL-library (libstlemass.dll) is placed. Also be sure that you know the pathname of the devices (you can see this by typing ‘inquire’ in the same directory) and that you know the name of the devices that is used from AMU (see graphical configuration of the AMU)

- Add volumes to Networker with the command ‘nsrjb –a –T 000/001-999’

This is an example for a range from 000001 to 000999.

Only the volsers which exists in the Library will be added.

- Define pools regarding to your storage strategy

- Label volumes for the defined pools