User Guide for the mxCS3 Mixer

Copyright (C)1998 Cornel Huth

---------------------------------------------------------------------

Overview

---------------------------------------------------------------------

The mxCS3 mixer is for CS4236B, 4237B, 4238B, 4235/4239 and later

soundcards operating in mode 3.  All software needed is included in

this package.

Technical support for this mixer is available only for this mixer

and only to those that have registered.  If you need support in

using your sound card, or setting up your soundcard, contact the

one that sold you the card or the card maker.

This User Guide steps you through installation and basic use, covering

the following:

I. Basic Setup

A. Stage 1

B. State 2

C. Tech Support

D. What Else Can I Get

E. Controls

F. Switches

II. License Agreement

---------------------------------------------------------------------

Basic Setup

---------------------------------------------------------------------

Installation is simple and easy; just follow the directions.  To

uninstall just remove the reference in A3, below, and delete the

files.

A. Stage 1:  (Pre-boot)

--------------------

1. Unzip the package to any empty directory

2. Copy PARADISE.SYS to your boot drive's root directory

3. Edit config.sys by adding the following line:

DEVICE=PARADISE.SYS

mxCS3 needs paradise version 1.90 or later.  If you're using

an earlier version install the one included here.  It is

backward compatible so any other Paradise apps will work as

always.

4. Move the mxcs3.exe to a directory in PATH

5. Create a program object for mxcs3.exe (optional)

6. If you are using Warp 3 and your clock01.sys driver is smaller

than 4000 bytes -and- you get an error starting this program,

do the following,

a.  Rename your current clock01.sys to clock01.org

b.  Copy the clock01.sys provided to your boot's root

This should not be necessary on Warp 4 GA since it ships with the

same clock01.sys as provided here (though a different date).

Note: your current clock01.sys may be in a directory other than

root.

7. Reboot

B. Stage 2: (Post-boot)

--------------------

1. Start mxcs3.exe

2. The configuration dialog appears.  (You can force this to appear

before anything else in later starts by using the -config switch.)

Enter the needed information, as indicated next:

Enter the base port of the codec. This can be found in the

BASEDEV=CWAUDIO.SYS line in your config.sys file as the

the /P: number.

Enter the control port of the codec. This can be found in the

BASEDEV=CWAUDIO.SYS line in your config.sys file as the

the /C: number.

Use RMVIEW /io if neither of these are on the BASEDEV= line.  In

most cases, /P:534  and /C:538 are the ports used, so if you

can't find this information, use 534 as the base port and 538 as

the control port for the configuration fields.

If your configure settings fail, for example if you enter a wrong

base port and get a message indicating error 9053: Invalid Port,

either delete mxcs3.ini and go again, or use the -config switch when

starting mxcs3.exe.

The SyncLock fields should do fine as they are.  The range of the

period is 31ms to 60000ms (32x/sec to 1x/min),

Press OK (not just ENTER).  The mxcs3.ini file is written, and to the

same directory that the mxcs3.exe is in.  This is the ini file that

is used from now on (see -ini: switch to change this).

3. At this point mxcs3.exe is ready to use.

C. Tech Support

------------

If you have registered and have a support question or comment on

the mixer send e-mail containing the mixer type/version to

mixer@40th.com

If you are evaluating the mixer, no support is available until you

have registered.

Regarding DART, though not related to mxCS3:

If you are using Warp 3 and notice that DART no longer works after

installing the Crystal mode 3 driver it's because the Crystal install

replaces the DART DLLs with obsolete DLLs.  If this is the case,

complain to Crystal and then service the OS/2 Multimedia System with a

recent fixpack (fp26 or later).  This mode 3 Crystal driver is a new

release and so may have problems.  Contact Crystal or IBM if so; these

are not related to mxCS3.

D. What Else Can I Get

-------------------

Take the Express elevator to

http://40th.com/

and have a look around.  Download anything and everything.  Updates

will always be found here.  If you didn't get this package from

here recently, you're already probably using an old version.

E. Controls

--------

Not all controls are used by all soundcards; OEMs can put together a

soundcard in many different way.  mxCS3 includes controls for all the

available hardware mixer registers.  The control names, if not at

first intuitive, should become easy to recognize with use.

To move a knob use the mouse pointer and just turn the knob, or click

around it to where you want it to be set.  You can also use the left/

right arrow keys to more precisely control the position.

The check box "To IM" routes that signal to the input mixer, "To OM"

routes to the output mixer, and "To SL" has that control updated

periodically by SyncLock(tm).

IMSA attenuates the input mixer signal before reaching the ADC.  Use

this if you are sourcing from more than one signal and notice any

over-range condition.  The normal position is full-right.

Each balance control can be quickly centered by using the RMB (right

mouse button).

The upper knobs: Mic, Aux1, Aux2, Line, ADC, and Line Out control

analog mixer levels.  ADC is a gain knob, and 0 is +0dB (15=+22.5dB).

Mic is at +0dB when its level is "15".  Typically, level "23" is

neutral (no gain, no attenuation).  Each point is 1.5dB, except for

Line Out where each point is 2dB.

The lower knobs: Wave, MPU, FM, DSP, Mon, and Master, control digital

mixer levels.  Typically, level "63" is optimal (no gain).  The Master

knob controls both digital and analog mixer levels, and goes to "71".

Levels above "63" adding +1.5dB gain to the analog portion of the master

level.

The controls should be self-explanatory.

F. Switches

--------

The following switches may be used on the command line.  None are

typically needed.

-config   Start up configuration dialog before anything else (use

this or simply delete your mxcs3.ini file)

-nobeep   Disable beeps

-min      Start up minimized

-nosp     Do not save new window position

-nolp     Do not load old window position

-ini:a    a=pathname to ini file, form D:/PATH/ININAME (no extension)

(default is drv:/directory/mxcs3.ini of mxcs3.exe)

-resetbg  restore the default colors

-yellhelp does nothing useful

---------------------------------------------------------------------

II. License Agreement

---------------------------------------------------------------------

Before using this software you must agree to the following:

1. You are required to have one license per computer that has the

software installed.

2. The software is owned by the author, Cornel Huth, and is protected

by United States copyright laws and international treaty provisions.

You are not permitted to make copies of this software except for

archival purposes.

3. You may not rent or lease this software. You may not transfer this

license without the written permission of the author.  If this

software is an update or upgrade, you may not sell or give away

previous versions.

4. You may not reverse engineer, decompile, or disassemble this

software if the intent or result is to alter the software.

5. There are no expressed or implied warranties with this software.

6. All liabilities in the use of this software rest with the user.

7. U.S. Government Restricted Rights.  This software is provided

with restricted rights.  Use, duplication, or disclosure by the

Government is subject to restrictions as set forth in

subparagraph (c)(1)(ii) of the Rights in Technical Data and

Computer Software clause at 52.227-7013. The software is

owned by Cornel Huth/6402 Ingram Rd/San Antonio Texas

78238/USA.  This agreement is governed by the laws of the

Great State of Texas, the United States of America, and all other

countries of Earth.

Any questions concerning this License Agreement should be directed

to Product Support.

Failure to comply with any part of this License Agreement may result

in license revocation.