BETA RELEASE OF OS/2 COMMUNICATION DRIVERS

IBM DISCLAIMS ALL WARRANTIES, WHETHER EXPRESSED OR IMPLIED, INCLUDING

WITHOUT LIMITATION, WARRANTIES OF FITNESS AND MERCHANTABILITY WITH

RESPECT TO THE INFORMATION IN THIS DOCUMENT AND INCLUDED SOFTWARE

PRODUCTS.  . BY FURNISHING THISMATERIAL, IBM GRANTS NO LICENSES TO ANY

RELATED PATENTS OR COPYRIGHTS.

Copyright IBM Corporation, 1992, all rights reserved.

1.0       INSTALLATION PROCEDURE

a.      The following files will be replaced:

x:\OS2\COM.SYS

x:\OS2\MDOS\VCOM.SYS

x:\OS2\MDOS\WINOS2\SYSTEM\COMM.DRV

These files will be  backed up.

b.    Switch to the drive where the BETA files are

located.

c.     Enter "cominst  x:" at the command line where

x: is the drive letter of the OS/2 Partition.

2.0      DESCRIPTION OF DRIVERS

The following is a brief description of the BETA

communication drivers for OS/2 2.0.  There are new

parameters for the COM.SYS which is located in the

CONFIG.SYS file.

2.1      COM.SYS

COM.SYS now has new parameters to indicate the COM port

number, the I/O address, the UNIQUE IRQ level, how to

handle spurious interrupts and to indicate the type of

UART chip.  An example follows:

DEVICE=C:\OS2\COM.SYS (3,xxxx,ii,s,t) [(4,xxxx,ii,s,t)]

where the last parameter is optional.  NOTE:  You must

have COM3 defined inorder for COM4 to work.  The usual

symptom of a bad I/O address or interrupt is when you

open a DOS or OS/2 command prompt and it fails.

n         :  COM port number (usually 3 and 4 but it is

possible to configure COM2 to a

differrent IRQ or I/O Address

xxxx   :  COM port address

ii         :  IRQ level

s         :  Spurious interrupt handling switch

values:

D or d    to deinstall COM driver if

more than 1000 consecutive

spurious interrupts occur.

I or i     to ignore spurious interrupt.

P or p  to post error if more than 1000

consecutive interrupts

occur.  When post error,

some apps can hang.

t         :   Type of UART chip.

values:

F or f     for FIFO 16550 UART chip.

Other characters will be

treated as non FIFO like

16450 or 8250.

If no parameter is passed,

COM.SYS will check the

UART. This is to support

Western Digital's 16550AF

compatible chip which has a

bug in its FIFO support.

W or w For Western Digital's 16550AF

compatible UART  (possible

performance improvement)

COM.SYS also has the following fixes:

1. Change the 0xFF check to see the presence of port in

the given address.

2. Change from DevHlp EOI to writing port 0x20 to

reduce interrupt pass.  (Possible performance

improvement)

3. Better checking of FIFO.

4. COM.SYS does not detect Hayes ESP card so that Hayes

ESP card can be used as a regular dual asynch card.

5. New DOS_SETTING Property :  HW_COM (will be renamed

to COM_DIRECT_ACCESS later on.).  HW_COM removes the

IO hook of VCOM.SYS after check the access control.

If the app only works when VCOM.SYS is not loaded,

HW_COM will help the problem.  When HW_COM is on,

any application which requires the presence of

VCOM.SYS will not work in the DOS session.  This

VCOM.SYS requires the new COM.SYS.

2.2      VCOM.SYS

No parameters or settings required.

2.3      COMM.DRV

COMM.DRV contains the support for COM3 and COM4 in

WinOS2.  This COMM.DRV requires the new VCOM.SYS.