Disk Performance Test

Version 1.0

August 20, 1991

IDK Computer Systems, Inc.

129 Cambridge Street

Syracuse, New York 13210

(315) 475-5598

1.0  Objectives

The objective of this project is to invent a method,

consisting of a computer program and operating instructions, for

testing the performance of disk subsystems operating in an OS/2

protect mode environment.  The computer program, distributed with

this document, is in a file named "DPERF.EXE".  The latter

sections of this document constitute the operating instructions.

2.0  Requirements

The computer program for this test executes on all

80286, 80386 and 80486 based microcomputer systems that are

capable of supporting the Microsoft OS/2 operating system.  The

performance figures yielded by running the program are normalized

so that they are not affected by the type of central processing

unit (CPU) or the fundamental clock rate (MHz) of the CPU.  The

performance figures are be affected by the revision level of the

OS/2 operating system being used, the file system in use, the

disk cache system, sector interleave, and the hardware and device

driver software provided by the disk subsystem vendor.

Concurrent programming is used to test performance of interrupt

handling in the disk subsystem device driver.  Separate

performance figures are shown for read and write tests to measure

the performance of the disk cache system.  The computer program

is able to execute on monochrome or color systems, and with MFM,

RLL, ESDI, IDE or SCSI format drives.  The disk subsystem test is

able to test local area network simulated disk drives.

3.0  Test Method

The computer program for this test consists of two

principal components:  A CPU Performance Test and a Disk

Performance Test.  The CPU Performance Test was developed so that

the results of the Disk Performance Test could be normalized to

eliminate the effects of differing speed CPU hardware.

3.1  CPU Performance Test

During the development phase of this test, compensation

factors were measured that allow the program to display

performace figures which are independent of the CPU type and

fundamental clock frequency.  A CPU Performance Test was written

to consist primarily of sixteen bit data moves from memory to

sixteen bit data moves from register to memory, data compare

instructions, conditional branches, and stack operations.

Thirty-two bit arithmetic, multiplies, divides, thirty-two bit

address calculations and string manipulations were deliberately

omitted from the instruction mix because they are not executed

frequently enough in the operating system code to affect test

results within the 0.005% error margin.  The CPU Performance Test

was designed to mimic the code that is executed in the operating

system.

The CPU Performance Test consists of two nested loops.

The inner loop contains the move, compare, branch and stack

instructions, and executes for a little under 20 microseconds on

an IBM Personnal Computer AT running at 8 MHz.  The inner loop is

executed fifty times to give a nominal execution time of one

millisecond.  The outer loop executes ten thousand times to give

the entire test a duration of ten seconds.  The OS/2 Global

Information Segment, whose address can be obtained by a call to

the operating system, records the elapsed time since the system

was turned on or restarted, in milliseconds, continuously

updated.  The test program records this time at the beginning of

the two nested loops, then again after the loops finish.  By

successive approximation, the number of iterations for the outer

loop was adjusted to the center of the spread which would yield

observed elapsed time of 10,000 milliseconds on the IBM Personal

Computer AT running at 8 MHz.  The value of ten seconds was

chosen for the test so that four significant figures of accuracy

could be maintained with the one millisecond clock resolution of

the operating system.

3.2  Disk Performance Test.

In order to measure the effects of various forms of

caching, directory structures and concurrent I/O processing,

a test was developed which included file writes and file reads of

various sizes in a multi-thread environment.  Five threads were

created in the program, each of which writes a file, then reads

it back.  Elapsed time in milliseconds is reported by each thread

at the completion of writing the file, and at completion of

reading the file back.  The record sizes for the files and the

number of records in each file are shown in Table I.

Table I - Record and FIle Sizes

File Name       Record Size     Number of Records

-------------------------------------------------

DPERF001.TMP    1 byte          3,000

DPERF002.TMP    10 bytes        2,000

DPERF003.TMP    100 bytes       1,000

DPERF004.TMP    1,000 bytes     300

DPERF005.TMP    10,000 bytes    100

The file sizes were chosen to correspond to record sizes

most typically written by application programs.  The number of

records for each file were chosen so that the complete writing of

each file would take approximately the same amount of time.

To compute the normalization that eliminates the effects

of varying CPU performance, the CPU Performace Test (detailed in

the previous section), and the concurrent file write/read test

were executed on several different CPU configurations.  In each

case, the disk drive, the disk controller, and the operating

system were held constant.  The disk drive used for development

was a Seagate model ST412, which is a type 1 drive on the IBM

Personal Computer AT.  The disk controller used for development

was a Western Digital model WD1003-WA2, which is equivalent to

the controller supplied by IBM with the Personal Computer AT.

The operating system used for development was OS/2 version 1.21.

A floppy disk containing the operating system was prepared and

set up to have swapping disabled, caching disabled and

protect-only.  The drive was formatted for the FAT (File

Allocation Table) directory structure, having a sector interleave

of 3.  Table II shows the recorded total execution times for the

various CPU configurations used during test development.

Table II - Execution Time vs. CPU Type (sheet 1 of 2)

System                   CPU Test (ms)   Disk Test (ms)

--------------------------------------------------------

IBM PC/AT - 8 MHz        10000           139593

10000           146656

10000           137531

10000           140969

10000           138657

Generic 386SX -           4313           128781

16 MHz                  4312           135125

4313           138250

4312           128281

4313           125344

Generic 386SX -           8813           142844

8 MHz                   8812           145125

8813           141437

8812           140906

8813           134687

Novacor NEAT 286 -        5875           134218

12 MHz                  5875           136125

5875           128688

5875           134781

5875           135844

Novacor NEAT 286 -        3437           129718

20 MHz                  3438           123969

3437           128656

3438           132469

3437           128813

King of Neat 286 -        4343           132968

16 MHz                  4344           128375

4343           128719

4344           130656

4343           135875

King of Neat 286 -        3469           124875

20 MHz                  3469           132250

3469           123750

3469           127500

3469           124156

Table II - Execution Time vs. CPU Type (sheet 2 of 2)

System                   CPU Test (ms)   Disk Test (ms)

--------------------------------------------------------

King of Neat 286 -        8843           141406

8 MHz                   8844           135531

8843           138687

8844           144219

8843           142781

Generic NEAT 286 -        2875           124062

25 MHz                  2875           122782

2875           117968

2875           126343

2875           126831

Generic NEAT 286 -        9094           143437

8 MHz                   9094           141656

9094           143219

9094           144312

9094           136844

AMD 386, OPTI Chipset -   1188           117343

40 MHz                  1188           116157

1188           121031

1188           114750

1188           118281

AMD 386, OPTI Chipset -   3594           138781

10 MHz                  3594           128250

3594           135843

3594           137125

3594           125656

The total execution times shown in Table II are measured from the

point just prior to creating the first sub-thread in the test

program to the point at which the last of the five threads had

completed reading and deleting its test file.  Total execution of

each thread includes opening its file for writing, writing the

file, closing the file, re-opening the file for reading, reading

the file, closing the file again and deleting the file.

Observation of the test results during development

showed that, though the CPU Performance Test reported a very

constant figure, the execution time of the Disk Performance Test

results were somewhat stochastic.  A BASIC program was written to

use the least squares method to average the relationship between

CPU performance and disk subsystem performance.  Because of the

fact that the total execution times consist of two independent

parts, the CPU contribution and the disk contribution, a linear

relationship exists between the CPU performance and the total

disk test time.  If the execution time of the CPU Performance

Test is plotted along the abscissa of a graph and the total disk

test time is plotted along the ordinate, then the X-intercept

would be the total disk time if there were zero CPU contribution.

The slope of this graph would be the ratio of time required to

execute the operating system execution over the CPU Performance

Test execution time.  The slope of the graph was calculated as

2.464064;  meaning that the operating system overhead for OS/2

version 1.21 for this test is about 25 seconds on an 8 MHz IBM

Personal Computer AT.  The intercept of the graph was calculated

as 118,977.7; meaning that the disk operations would have taken

about 119 seconds had the CPU been infinitely fast.  These, of

course, are average figures.  Some of the observed total disk

test times are actually lower than 119 seconds because of the

random nature of asynchronous events in the operating system.

The BASIC program that was used to compute these figures is

included with this report in a file named "DPERF.BAS".

3.3 Normalization Algorithm

Given that we have a standard for disk subsystem

performance in the configuration of a Seagate ST412 drive with

a Western Digital WD1003-WA2 controller, being driven by OS/2

version 1.21 running from a floppy disk with swapping, caching

and DOS mode disabled, we can begin to compare other disk

subsystems to this.  The standard was selected to be the slowest

configuration that it is reasonably likely that someone might

implement as of the date of this report.  Thus we can compute a

performance figure stating that the performance of a new and

improved system is X number of times greater than the standard.

The performance figure is a ratio of new subsystem speed to the

standard.

During the development of this test, we measured the

ratio of execution time for the operating system overhead vs. a

known loop of common instructions.  This ratio was computed to be

2.464064 as an average over sixty samples.  Multiplying the

execution time, in milliseconds, for the CPU Performance Test for

any test configuration by this ratio will give the number of

milliseconds of operating system overhead for that test

configuration.  Subtracting this number of milliseconds of

operating system overhead from the total disk test time yields

the raw disk time in milliseconds - which is the time that the

test would have taken to execute if the CPU contribution to the

test time were not present.  This raw disk test time can then be

divided by the raw disk test time for the standard ST412

configuration to yield the reciprocal of the disk performance

improvment.  What is actually displayed by the test program is

the standard time, 118,977.7 milliseconds, divided by the raw

disk test time for the new disk subsystem.

4.0  Results

Each time that the computer program for this test runs,

it records test results to a log file named "DPERF.LOG" on the

current drive.  If this file does not exist, then the program

creates it, otherwise it appends new results to the old.  Figure

1 shows an example of a typical log file from ten test runs using

two computers and three disk subsystems:

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/22/91   Time: 11:52:17   Operating System: OS/2 1.21

Test System: Generic NEAT 286 - 25 MHz                Drive: C

Drive Type: Maxtor 7120AT

Controller Type: IDE

Test Conducted by: P. Gleason

Remarks: Run from floppy disk, no swap, no cache,

no DOS.

CPU Performance (relative to 8 MHz 80286): 3.30

Write Test

Rec Size  Records  msec

1         3,000     16562

10        2,000     10187

100       1,000     32250

1,000     300       51312

10,000    100       27843

Read Test

Rec Size  Records  msec

1         3,000      7125

10        2,000      4813

100       1,000      8937

1,000     300        2688

10,000    100       10188

Disk Subsystem Performance (relative to type 1): 2.530

--------------------------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/22/91   Time: 11:55:12   Operating System: OS/2 1.21

Test System: Generic NEAT 286 - 25 MHz                Drive: C

Drive Type: Maxtor 7120AT

Controller Type: IDE

Test Conducted by: P. Gleason

Remarks: Run from floppy disk, no swap, no cache,

no DOS, FAT file system.

CPU Performance (relative to 8 MHz 80286): 3.33

Figure 1 - Disk Performance Test Log File (sheet 1 of 1)

Write Test

Rec Size  Records  msec

1         3,000     15531

10        2,000     14562

100       1,000     34031

1,000     300       51375

10,000    100       28750

Read Test

Rec Size  Records  msec

1         3,000     10937

10        2,000     10438

100       1,000      7594

1,000     300        2718

10,000    100       12062

Disk Subsystem Performance (relative to type 1): 2.522

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/22/91   Time: 11:57:24   Operating System: OS/2 1.21

Test System: Generic NEAT 286 - 25 MHz                Drive: C

Drive Type: Maxtor 7120AT

Controller Type: IDE

Test Conducted by: P. Gleason

Remarks: Run from floppy disk, no swap, no cache,

no DOS, FAT file system.

CPU Performance (relative to 8 MHz 80286): 3.30

Write Test

Rec Size  Records  msec

1         3,000     18593

10        2,000     11562

100       1,000     32281

1,000     300       51218

10,000    100       25656

Figure 1 - Disk Performance Test Log File (sheet 2 of 7)

Read Test

Rec Size  Records  msec

1         3,000      7438

10        2,000      3906

100       1,000      8125

1,000     300        2719

10,000    100       11656

Disk Subsystem Performance (relative to type 1): 2.533

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/22/91   Time: 12:06:03   Operating System: OS/2 1.21

Test System: Generic NEAT 286 - 25 Mhz                Drive: C

Drive Type: Maxtor 7120AT

Controller Type: IDE

Test Conducted by: P. Gleason

Remarks: Run from floppy disk, no swap, no cache,

no DOS, FAT file system.

CPU Performance (relative to 8 MHz 80286): 3.33

Write Test

Rec Size  Records  msec

1         3,000     13750

10        2,000     11625

100       1,000     29562

1,000     300       51312

10,000    100       34906

Read Test

Rec Size  Records  msec

1         3,000     10406

10        2,000      6562

100       1,000      5000

1,000     300        2625

10,000    100       10906

Disk Subsystem Performance (relative to type 1): 2.531

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Figure 1 - Disk Performance Test Log File (sheet 3 of 7)

Date: 8/22/91   Time: 12:10:57   Operating System: OS/2 1.21

Test System: Generic NEAT 286 - 25 MHz                Drive: C

Drive Type: Maxtor 7120AT

Controller Type: IDE

Test Conducted by: P. Gleason

Remarks: Run from floppy disk, no swap, no cache,

no DOS, FAT file system.

CPU Performance (relative to 8 MHz 80286): 3.30

Write Test

Rec Size  Records  msec

1         3,000     16719

10        2,000     16063

100       1,000     31938

1,000     300       52750

10,000    100       29500

Read Test

Rec Size  Records  msec

1         3,000      8656

10        2,000      3718

100       1,000     11281

1,000     300        2750

10,000    100       11281

Disk Subsystem Performance (relative to type 1): 2.456

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/21/91   Time: 11:30:48   Operating System: OS/2 1.21

Test System: AMD 386, OPTI Chipset - 40 MHz           Drive: C

Drive Type: Maxtor LT213A

Controller Type: IDE

Test Conducted by: P.Gleason

Remarks: HPFS 386 present, 2 Mbytes FAT system

cache, Lanman file server, swap on, no DOS, FAT

file system on this drive, lazy on.

CPU Performance (relative to 8 MHz 80286): 8.42

Figure 1 - Disk Performance Test Log File (sheet 4 of 7)

Write Test

Rec Size  Records  msec

1         3,000      5156

10        2,000      5250

100       1,000     24250

1,000     300       35812

10,000    100       17312

Read Test

Rec Size  Records  msec

1         3,000      2750

10        2,000      3531

100       1,000      9406

1,000     300        1250

10,000    100       12031

Disk Subsystem Performance (relative to type 1): 3.454

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/21/91   Time: 11:35:48   Operating System: OS/2 1.21

Test System: AMD 386, OPTI Chipset - 40 MHz           Drive: C

Drive Type: Maxtor LT213A

Controller Type: IDE

Test Conducted by: P. Gleason

Remarks: HPFS 386 present, FAT file system on this

drive, 2048K FAT cache, swap on, no DOS, Lanman

file server, lazy on.

CPU Performance (relative to 8 MHz 80286): 8.42

Write Test

Rec Size  Records  msec

1         3,000      3125

10        2,000      7250

100       1,000     26688

1,000     300       35719

10,000    100       16500

Figure 1 - Disk Performance Test Log File (sheet 5 of 7)

Read Test

Rec Size  Records  msec

1         3,000      2625

10        2,000      3000

100       1,000      7250

1,000     300        1063

10,000    100       12063

Disk Subsystem Performance (relative to type 1): 3.479

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/21/91   Time: 11:39:21   Operating System: OS/2 1.21

Test System: AMD 386, OPTI Chipset - 40 MHz           Drive: D

Drive Type: Fujitsu M2249SA

Controller Type: Always Technology IN-2000

Test Conducted by: P. Gleason

Remarks: HPFS 386, HPFS on this drive, swap, no

DOS, lazy cache.

CPU Performance (relative to 8 MHz 80286): 8.42

Write Test

Rec Size  Records  msec

1         3,000      1907

10        2,000      3407

100       1,000      2844

1,000     300        2625

10,000    100        4250

Read Test

Rec Size  Records  msec

1         3,000      2625

10        2,000      2406

100       1,000      2406

1,000     300          94

10,000    100        1469

Disk Subsystem Performance (relative to type 1): 36.835

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Figure 1 - Disk Performance Test Log File (sheet 6 of 7)

Date: 8/21/91   Time: 11:42:34   Operating System: OS/2 1.21

Test System: AMD 386, OPTI Chipset - 40 MHz           Drive: D

Drive Type: Fujitsu M2249SA

Controller Type: Always Technology IN-2000

Test Conducted by: P. Gleason

Remarks: HPFS 386, HPFS on this drive, swap, no

DOS, Lanman file server, lazy cache.

CPU Performance (relative to 8 MHz 80286): 8.42

Write Test

Rec Size  Records  msec

1         3,000      2438

10        2,000      1282

100       1,000      2657

1,000     300        1938

10,000    100         219

Read Test

Rec Size  Records  msec

1         3,000      1937

10        2,000      1500

100       1,000       250

1,000     300         125

10,000    100        1406

Disk Subsystem Performance (relative to type 1): 66.468

-----------------------------------------------------------------

IDK Disk Subsystem Performance Test

(c) 1991 IDK Computer Systems, Inc.

Syracuse, New York 13210 - (315) 475-5598

Date: 8/21/91   Time: 11:44:33   Operating System: OS/2 1.21

Test System: AMD 386, OPTI Chipset - 40 MHz           Drive: D

Drive Type: Fujitsu M2249SA

Controller Type: Always Technology IN-2000

Test Conducted by: P. Gleason

Remarks: HPFS 386, HPFS on this drive, swap, no

DOS, Lanman file server, lazy cache.

CPU Performance (relative to 8 MHz 80286): 8.42

Figure 1 - Disk Performance Test Log File (sheet 7 of 7)

Write Test

Rec Size  Records  msec

1         3,000      3032

10        2,000      1750

100       1,000       157

1,000     300          94

10,000    100          94

Read Test

Rec Size  Records  msec

1         3,000      2468

10        2,000      3032

100       1,000       218

1,000     300         125

10,000    100        1000

Disk Subsystem Performance (relative to type 1): 40.331