/****************************************************************************

** $Id:  qt/casestudyqws.doc   3.0.3   edited Nov 29 23:48 $

**

** Qt/Embedded Case Study

**

** Copyright (C) 1992-2000 Trolltech AS.  All rights reserved.

**

** This file is part of the Qt GUI Toolkit.

**

** This file may be distributed under the terms of the Q Public License

** as defined by Trolltech AS of Norway and appearing in the file

** LICENSE.QPL included in the packaging of this file.

**

** This file may be distributed and/or modified under the terms of the

** GNU General Public License version 2 as published by the Free Software

** Foundation and appearing in the file LICENSE.GPL included in the

** packaging of this file.

**

** Licensees holding valid Qt Enterprise Edition or Qt Professional Edition

** licenses may use this file in accordance with the Qt Commercial License

** Agreement provided with the Software.

**

** This file is provided AS IS with NO WARRANTY OF ANY KIND, INCLUDING THE

** WARRANTY OF DESIGN, MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

**

** See http://www.trolltech.com/pricing.html or email sales@trolltech.com for

**   information about Qt Commercial License Agreements.

** See http://www.trolltech.com/qpl/ for QPL licensing information.

** See http://www.trolltech.com/gpl/ for GPL licensing information.

**

** Contact info@trolltech.com if any conditions of this licensing are

** not clear to you.

**

**********************************************************************/

/*! \page emb-casestudy.html

\title Qt/Embedded Case Study - Cassiopeia E-100

\section1 Introduction

This document describes the steps involved in installing Linux on an embedded

device and building a Qt/Embedded application.  The target device is the

Cassiopeia E-100/E-105.  The device has a MIPS Vr4121 processor, 16MB RAM

(32MB in the E-105), a Compact Flash slot and a 240x320 16 bit per pixel LCD

display.

The only part of this document that is specific to the Cassiopeia is the

installation of Linux and the development tools.  The example application

can be compiled and run on your desktop machine.

\section1 Installing Linux

All the information and software required to get Linux running on the

VR series of processors is available from the

\link http://www.linux-vr.org/ Linux VR\endlink web site.  In Summary:

\section2 Install the tools

Follow the instructions at

\l http://www.linux-vr.org/tools.html.

\section2 Build the kernel

Get a sample root ramdisk from

\l ftp://ftp.ltc.com/pub/linux/mips/ramdisk/ramdisk.

Follow the instructions at

\l http://www.linux-vr.org/ramdisk.html

to create a ramdisk.o file.

Now build your kernel

\l http://www.linux-vr.org/kernel.html

using this ramdisk object.  Make sure you have at least the following

configuration:

\list

\i Development/incomplete drivers

\i Casio E105 Platform

\i Network and System V IPC

\i RAM disk and Initial RAM disk support

\i Support for console on virtual terminal (so that you can see boot messages)

\i \c /proc and ext2 filesystem support

\i Simple Frame Buffer with HPC device control

\endlist

\section2 Booting Linux

Follow the instructions at

\l http://www.linux-vr.org/booting.html.

You should see the linux boot messages on the LCD display.

\section1 A Qt/Embedded Application

Usually a device such as the Cassiopeia would have a shell, configured as the

Qt/Embedded server, that allows client applications to be launched.

For the purposes of this tutorial, we will write a simple application that

serves as the Qt/Embedded server and client.  A more complete Qt/Embbeded

server can be found in \c $QTDIR/examples/compact.

The application that we will write is a simple note pad.  It will allow

notes to be created, viewed and deleted.  Since the Cassiopeia doesn't have

a keyboard, we will include a simple on-screen keyboard for input.

\section2 Note Pad

Our note pad user interface is very simple.  It consists of a toolbar with

"New" and "Delete" buttons, a combo box to select the note to view and

an editing area.

Take a moment to browse the source code for Note Pad in \c

$QTDIR/examples/notepad/. The code is quite simple, but there are some

things worth noting:

\list 1

\i Two fonts are set - helvetica 10 point as the application's default font,

and helvetica 12 point for the editor.  Since we will use prerendered fonts

these fonts must be prepared as \link emb-fonts.html described

here\endlink.

\i The QApplication is constructed with the QApplication::GuiServer type

specified.  This makes the note pad a Qt/Embedded server.  One server must

be running for Qt/Embedded clients to run.  In this case our application is

both server and client because it is the only application we wish to run on

our device.

\i The Cassiopeia (usually) has no keyboard so we must provide some means

of character input with the pen.  The simplest method is to display a small

keyboard.  The compact example includes a keyboard, so we use this code.

Key and pointer input is Qt/Embedded specific, so it is surrounded

by #ifdef Q_WS_QWS ...  #endif so that we can compile the example with Qt/X11

or Qt/Windows if we wish.

\i The touch panel needs to be calibrated.  There is a calibration module

included in the compact demo, so we use this.

\endlist

\section2 Creating a suitable Qt/Embedded Library

Since our application is quite simple we can remove some unnecessary

features from Qt/Embedded.  Edit \c $QTDIR/src/tools/qconfig.h and

disable the following features:

\code

#define QT_NO_IMAGEIO_BMP

#define QT_NO_IMAGEIO_PPM

#define QT_NO_IMAGEIO_XBM

#define QT_NO_IMAGEIO_PNG

#define QT_NO_ASYNC_IO

#define QT_NO_ASYNC_IMAGE_IO

#define QT_NO_TRUETYPE

#define QT_NO_BDF

#define QT_NO_FONTDATABASE

#define QT_NO_MIME

#define QT_NO_SOUND

#define QT_NO_PROPERTIES

#define QT_NO_CURSOR

#define QT_NO_NETWORKPROTOCOL

#define QT_NO_COLORNAMES

#define QT_NO_TRANSFORMATIONS

#define QT_NO_PSPRINTER

#define QT_NO_PICTURE

#define QT_NO_LISTVIEW

#define QT_NO_CANVAS

#define QT_NO_DIAL

#define QT_NO_WORKSPACE

#define QT_NO_TABLE

#define QT_NO_LCDNUMBER

#define QT_NO_STYLE_MOTIF

#define QT_NO_STYLE_PLATINUM

#define QT_NO_COLORDIALOG

#define QT_NO_PROGRESSDIALOG

#define QT_NO_TABDIALOG

#define QT_NO_WIZARD

#define QT_NO_EFFECTS

\endcode

See \link emb-features.html Qt Features\endlink for a description of the

features that can be disabled.  This leaves us with a small set of

widgets and dialogs necessary for our application.  Cross-compile the

library for the mips target on the x86 platform:

\code

cd $QTDIR

./configure -xplatform linux-mips-g++ -platform linux-x86-g++

make

mipsel-linux-strip $QTDIR/lib/libqt.so.2.2.0

\endcode

The library is stripped to conserve ramdisk space.

\section2 Installation

Compile the application:

\code

cd examples/notepad

make

mipsel-linux-strip notepad

\endcode

We have chosen to link the application dynamically.  While this is important

if we plan to run multiple applications, it is a waste of space in an

application such as notepad that is supposed to be the only application

running.  You can link statically by configuring with:

\code

./configure -static -xplatform linux-mips-g++ -platform linux-x86-g++

\endcode

We must install our application and its support files in the ramdisk.  Mount

the ramdisk using loopback device (you will need loopback device support

in your kernel):

\code

mkdir /mnt/ramdisk

mount -o loop ~/ramdisk /mnt/ramdisk

\endcode

Copy the Qt/Embedded library to the ramdisk \c /lib directory and make the

necessary links:

\code

cd /mnt/ramdisk/lib

cp $QTDIR/lib/libqt.so.2.2.0 .

ln -s libqt.so.2.2.0 libqt.so.2.2

ln -s libqt.so.2.2.0 libqt.so.2

\endcode

The fonts must be installed in \c /usr/local/qt-embedded/etc/fonts:

\code

cd /mnt/ramdisk

mkdir usr/local

mkdir usr/local/qt-embedded

mkdir usr/local/qt-embedded/etc

mkdir usr/local/qt-embedded/etc/fonts

cp helvetica_100_50.qlf helvetica_120_50.qlf usr/local/qt-embedded/etc/fonts

\endcode

When the kernel boots it looks for several files to run.  In order to

have our application run when the kernel boots, we change its name to

\c /bin/sh.  A \c /tmp directory is also used by Qt/Embedded:

\code

cp $QTDIR/examples/notepad/notepad /mnt/ramdisk/bin/sh

mkdir /mnt/ramdisk/tmp

umount /mnt/ramdisk

\endcode

Create a ramdisk object, link it with the kernel, copy it to the compact flash

and boot Linux.  You should see the calibration screen appear on the LCD

display.

*/