��  � ���Ŀ ���Ŀ ���Ŀ ����Ŀ ���Ŀ ����Ŀ ���Ŀ

��  � ��    ��  � ��      ��   ��  �   ��   ��          Version 4.13

��  � ���¿ ����� ���¿   ��   ���Ĵ   ��   ���¿

��  �    �� �� �     ��   ��   ��  �   ��      ��         12/30/95

����� ����� �� �� �����   ��   ��  �   ��   �����

General Information

�����������������������������������������������������������������������

This document provides general information about the entire "family" of

USRSTATS software.

�����������������������������������������������������������������������

What is it?

�����������������������������������������������������������������������

USRSTATS is a collective name for a small group of software

applications which complement USRobotics modems.  USRSTATS is actually

comprised of several different programs, each filling a specific

application need.

�����������������������������������������������������������������������

What does it do?

�����������������������������������������������������������������������

The programs generate a multi-page report of all of the diagnostic

information available in the modem.  This report can be saved to a log

file, posted as a message, displayed on the screen, etc. etc.

�����������������������������������������������������������������������

Why would I want to do that?

�����������������������������������������������������������������������

The information contained in the Link Diagnostics of the USRobotics

modems can reveal important information about connection problems and

modem performance.  However, the modem presents this data in a form

that is difficult to interpret.  USRSTATS organizes and presents this

data in a manner that is more understandable, useful, and even

educational.  BBS Sysops will note that this information now becomes

available to all callers while on-line, regardless of the brand or

model of modem the *caller* is using.  Note further that callers can

now view the diagnostics *from the BBS end*, which has not previously

been possible.

�����������������������������������������������������������������������

What does it work with?

�����������������������������������������������������������������������

USRSTATS can be used with a wide variety of general-purpose

communications programs and BBS systems.  Its modular design allows

installation as a standard BBS "door", or it may be called through the

"external" file transfer protocol mechanism of a communications program

like Telix or Qmodem.  It can be used in connection with your

software's scripting or macro language.   You can even use it right

from the DOS prompt, to create reports from data captured to a file on

disk.

�����������������������������������������������������������������������

How does it work?

�����������������������������������������������������������������������

The heart of the software is the USRSTAT2.EXE program.  This DOS

executable program reads an input file containing captures of the

diagnostic data from the modem, and generates the formatted output

report file.  This is the *only* function of the USRSTAT2.EXE program.

Retrieving the data from the modem and storing it in a file on disk is

the responsibility of the STPCB, STGEN, and FTT programs, or of the

comm program script file or macro that you create.  This "host" program

then performs a "shell" to USRSTAT2.EXE with the proper command-line

options.  USRSTAT2.EXE reads the input file that is passed to it, and

creates the formatted report in the specified filename.  Control is

then returned to the host program, which is responsible for displaying

the completed report on the screen, saving it in a message, saving it

to disk, etc.  Refer to USRSTAT2.DOC for more details.

�����������������������������������������������������������������������

�����������������������������������������Ŀ

�Please read the above "How does it work?"�

�   section again, it is VERY important   �

�     that you understand this point!     �

�������������������������������������������

USRSTATS Applications for BBS software

�����������������������������������������������������������������������

STPCB - This is a PCBoard PPE program which acts as the "host" for

USRSTAT2.EXE on PCBoard systems.  It is highly configurable,

very Sysop-friendly, and allows capturing the reports into the

PCBoard message base.  See the STPCB file for further details.

STGEN - This is a DOS executable program which functions as a "door"

program for using USRSTAT2.EXE with virtually any BBS software.

It is similar to the STPCB program in appearance and operation,

but does not have the ability to *directly* import the reports

into a message base.  See the STGEN file for further details.

FTT -   This is a PCBoard PPE that combines STPCB with a special file

transfer door to allow controlled, repeatable modem throughput

testing.  See the FTT file for further details.

USRSTATS Applications for communications software

�����������������������������������������������������������������������

STGEN - This program can be used with most DOS-based communications

programs, such as Qmodem, Telix, {Commo}, etc.   STGEN can be be

installed as an "external protocol" and called through a batch file,

or it can be used through a DOS shell.

USRSTAT2.EXE - The USRSTATS report generator module can be called

directly by use of your communication program's keyboard macro facility

or scripting language.   Several sample scripts are included in the

USRSTATS distribution archive.

�����������������������������������������������������������������������

Now we need to know how to create this input capture file in the first

place.

If using USRSTATS with PCBoard, the STPCB program handles this

automatically.   The STGEN "door" program does the same, but is

designed to work with virtually any BBS software.  STGEN will also work

with most communication software, like Procomm, Qmodem, or others that

allow shelling to DOS or use of external protocols.

If none of these apply to you, you'll need to do it manually:

Use the "Capture Buffer" function in your comm software to create a

file containing the output of the I6, I7, I11, Y11, and (optionally) I4

commands while in online command mode.  As an example, Procomm Plus for

DOS uses ALT-F1 to open a capture file.  You provide a file name, and

PCPlus then copies everything that comes in on the screen out to that

disk file.  Now you need to get the modem into Online Command Mode by

issuing the +++ escape sequence.  Allow a slight pause, then type the

three plus signs.  The modem should respond with "OK".  You may now

issue commands to the modem even though the connection is still up.

(If the connection drops when you hit the 3 pluses, you have DIP switch

#9 in the wrong position.)   The next step is to issue the modem

diagnostic commands so they can be captured to the disk file.  The

USRSTAT2.EXE program is capable of processing the information from the

following commands:

ATI4

ATI6

ATI7

ATI11   <-- These 2 commands are only available on

ATY11   <-- the V.34 and V.FC modems!

After issuing the commands, we need to put the modem back into Data

mode so you can resume your connection.  This is done with the ATO

command.  NOTE That's the letter "O", NOT the number 0!

Just type ATO and hit the <enter> key, and the modem should respond

with the same connect message you got when you first connected.

(NOTE:  If the modem at the BBS end does not have the escape code

disabled (as any on-line service should), the BBS will seem to "lock

up" after this point.   What has happened is that the modem at the BBS

end saw the 3 plus signs you typed at your end, and it ALSO went into

command mode.   The trouble is, once in command mode, the modem no

longer passes anything you type, so there's no way to get the BBS side

back into Data mode.   Send a message to the Sysop of that BBS, and

tell him to set *his* S2 register to 255.

Just to make it easier, you can run all these commands together like

this:

ATI4I6I7I11Y11O

and the modem will process them all in order, and go right back to Data

mode when it's finished.

The last step of this process is to close the capture buffer, (ALT-F1

in PCplus) and then, optionally, shell out to DOS to process the file.

Has the light bulb over your head turned on yet?   YES, that's it!

USE A KEYBOARD MACRO!  OR EVEN BETTER, USE A SCRIPT TO DO ALL THIS!

Imagine, with a little effort, you can write a script for your favorite

comm program to do ALL of this for you at the touch of a key!

One of our users has written a macro for the {COMMO} program to do just

this.  This macro and documentation are included in the USRSTATS

package, along with a similar QMODEM macro.  These examples should give

you some ideas and hints about implementing this in your favorite comm

program.

What you end up with is a USRSTATS report file.  What you do with it at

this point is up to you.   If you use the /A mode, you can just keep

collecting them in one giant file.   If you are on-line at the time you

do this, you could upload that report file back to the BBS, or post it

as a message.  You could merge it into a text file or database.  The

possibilities are endless.

�����������������������������������������������������������������������

What about support?

The home of STPCB and USRSTATS is:

The Majestik Moose BBS,  708-843-2871

Only calls at 9600 BPS and faster are accepted.  MNP/V42 is required.

The latest version of this program is always available as a free

download in the Main Board area.  First time callers can get immediate

free access to these files by bypassing the registration prompt.

You will be allowed 30 minutes per call and complete access to the

"Free Downloads" area of the BBS.

If you need help with STPCB or USRSTATS, you may leave a message for

the SYSOP.  I'll help you if I can, but I make no promises.  After all,

the software is free.  Do NOT call and ask questions that are already

answered in the documentation.  You will receive only a very brief

answer under those circumstances.

STPCB and USRSTATS are available through ftp.usr.com on the Internet.

Use anonymous login, and your Internet ID as the password.

STPCB and USRSTATS are available (and running) on the USRobotics

Customer Support BBS at 708-982-5092.

At the present time, there is no registration required for this

software.  If you really want to pay me a few bucks for the many hours

of effort that went into this, I'll be glad to take it from you.  Call

The Majestik Moose and leave a (C)omment.

�����������������������������������������������������������������������

Notice to software authors:

Permission to use whole or edited portions of the USRSTATS

documentation is DENIED.  Read some books, learn how this stuff works,

and write your own material instead of butchering mine!

�����������������������������������������������������������������������

Disclaimer

Use this software at your own risk.  It works fine for me.  All product

and feature names are trademarks of their respective owners.  This

software is NOT a product of USRobotics, Inc.

Joseph C. Frankiewicz

The Majestik Moose BBS

708-843-2871

�����������������������������������������������������������������������

Modem Escape Sequences

An "Escape Sequence" is a mechanism or method for forcing the modem

into "Command Mode" while it is already connected and on-line.

"Command Mode" is normally associated with the OFF-LINE condition,

where the modem will accept and act upon "AT" commands entered from the

terminal.  However, it is also possible to put the modem into this mode

DURING a call, WITHOUT disconnecting the call first.  This ability is

required for making use of certain diagnostic commands which only

return valid data when used in this on-line command mode.

USRobotics modems offer 2 methods of getting into on-line command mode.

Each has its own advantages and disadvantages, depending on the point

of view.

The first method is the &D1 setting.   When the modem is set to &D1, a

momentary "toggling" of the DTR signal (pin 20), from ON to OFF and

back ON again, will suspend the data connection and place the modem

into command mode.  USRSTATS documents refer to this as "DTR

switching".

The second method is the infamous "Escape Character with Guard Time."

A designated ASCII character is defined as the "Escape Character", and

when the modem receives three of these characters in a row, with a

defined period of inactivity both before and after, it suspends the

data connection and places the modem in command mode.  The character

used for the escape is defined in the S2 register, and the default

value is 43, the "+" character.  (This is where the often-seen "+++"

comes from.)  The inactivity or "Guard" time is defined in the S12

register, with a default of 1 second.  USRSTATS documents refer to this

as "Escape Character switching".

When using DTR switching, the remote user does not see the "+++"

characters on the screen when the BBS modem is placed into command

mode.  This provides a professional seamless interface.  Also, the BBS

modem is not susceptible to entering command mode if the _caller_ is

also using Escape Character switching with an identical Escape

Character definition.

The disadvantage of DTR switching is that software cannot drop

connections by simply toggling the DTR pin, it must _also_ send an ATH

or ATZ command.  It is possible to work around this in PCBoard, however

you may be using other software in your system that cannot be altered

this way.

Using Escape Character switching, your software retains the ability to

drop connections by toggling the DTR pin, as normal.  The disadvantages

are that the remote user will see the "+++" sequence on screen when

STPCB runs, and there is a possibility that the remote user may ALSO

be trying to use Escape Character switching with the same Escape

Character that the BBS is using.   This results in the BBS modem going

into command mode when the intended target was the remote modem.  Since

the BBS modem can no longer pass any data to the user, and the BBS

software is unaware of what happened, the user has no alternative but

to drop the connection and call back.  For this reason, avoid using the

standard S2 register value of 43.  (The "+" character.)   A value of 1

(S2=1) usually works well.

�����������������������������������������������������������������������

[END]