unit buzGlobal;          {buzStpl  buzGlobal}

{Combines GCTPC, paksz,

cproj (code), proj (list), report (error report)

data requirements

All Called Projections data requirements}

{.$define Tracer}

{$define Locatable}

{.$define Grip}

{.$define Sosmap}

{.$define DTM}

interface

uses SysUtils,Dialogs,Math;

(*  from Proj*)

const

GEO: Integer = 0; //0 = Geographic

UTM: Integer = 1; //1 = Universal Transverse Mercator (UTM)

SPCS: Integer = 2; //2 = State Plane Coordinates

ALBERS: Integer = 3; //3 = Albers Conical Equal Area

LAMCC: Integer = 4; //4 = Lambert Conformal Conic

MERCAT: Integer = 5; //5 = Mercator

PS: Integer = 6; //6 = Polar Stereographic

POLYC: Integer = 7; //7 = Polyconic

EQUIDC: Integer = 8; //8 = Equidistant Conic

TM: Integer = 9; //9 = Transverse Mercator

STEREO: Integer = 10; //10 = Stereographic

LAMAZ: Integer = 11; //11 = Lambert Azimuthal Equal Area

AZMEQD: Integer = 12; //12 = Azimuthal Equidistant

GNOMON: Integer = 13; //13 = Gnomonic

ORTHO: Integer = 14; //14 = Orthographic

GVNSP: Integer = 15; //15 = General Vertical Near-Side Perspective

SNSOID: Integer = 16; //16 = Sinusiodal

EQRECT: Integer = 17; //17 = Equirectangular

MILLER: Integer = 18; //18 = Miller Cylindrical

VGRINT: Integer = 19; //19 = Van der Grinten

HOM: Integer = 20; //20 = (Hotine) Oblique Mercator

ROBIN: Integer = 21; //21 = Robinson

SOM: Integer = 22; //22 = Space Oblique Mercator (SOM)

ALASKA: Integer = 23; // 23 = Alaska Conformal

GOOD: Integer = 24; //24 = Interrupted Goode Homolosine

MOLL: Integer = 25; //25 = Mollweide

IMOLL: Integer = 26; //26 = Interrupted Mollweide

HAMMER: Integer = 27; //27 = Hammer

WAGIV: Integer = 28; //28 = Wagner IV

WAGVII: Integer = 29; //29 = Wagner VII

OBEQA: Integer = 30; //30 = Oblated Equal Area

USDEF: Integer = 99; //99 = User defined

IN_BREAK: Integer = -2;

{  COEFCT: Integer = 15;} //  projection coefficient count

{  PROJCT: Integer = 30;} //  projection count  ?

{  DATMCT: Integer = 20; }// ???? datum count   ?

{  SPHDCT: Integer = 31;} //  spheroid count

{  MAXPROJ: Integer = 31;} //  Maximum projection number

MAXUNIT: Integer = 5; //  Maximum unit code number

{  GEO_TERM: Integer = 0;} //  Array index for print-to-term flag

{  GEO_FILE: Integer = 1;} //  Array index for print-to-file flag

GEO_TRUE: Integer = 1;

//*  True value for geometric true/false flags */

GEO_FALSE: Integer = -1;

//*  False val for geometric true/false flags */

{iter: integer = 0;} //* First time flag    */

const

{#define}{already in System PI:Double=  3.141592653589793238;}

HALF_PI: Double = (PI * 0.5);

TWO_PI: Double = (PI * 2.0);

EPSLN: Double = 1.0E-10;

R2D: Double = 57.2957795131;

D2R: Double = 0.01745329251994328;

S2R: Double = 4.848136811095359E-6;

OK: Integer = 0;

ERROR: Integer = -1;

{from CPROJ}

MAX_VAL: Integer = 4;

MAXLONG: Integer = 2147483647;

DBLLONG: Double = 4.61168601E18;

type      {0..14 + 7 for base info

15:Projection

16:Units

17:UTM Zone

18:Datum

19:Spheroid

20:State Plane Zone

21: Z units}

parameters = array[0..21] of double;

{  parray31 = array[0..31] of Integer;

ppa = array[0..31, 0..16] of double;}

var

{inpj,outpj: parray31;} //* input projection array

//array [0..31{MAXPROJ + 1}] of Integer;

{indat,outdat: parray31;}//* input datum array

//array [0..31] of Integer;

{inzn,outzn: parray31;} // zone array

//array [0..31] of Integer;

{pdin,pdout: ppa;  }

//array [0..31,0..16]{[MAXPROJ + 1][15]} of double;  //* input projection parm array  */

inparm,  outparm: parameters; //array [0..31]of double;

{parm:parameters;}{array [0..16]of double;}

iflg, inv_trans,for_trans:integer;{Passage rather than Function calls}

bIgnoreErrorsCB,bIgnoreDatumCB:Boolean;

// Table of unit codes as specified by state

// laws as of 2/1/92 for NAD 1983 State Plane projection,

//    1 = U.S. Survey Feet,

//    2 = Meters,

//    5 = International Feet

{static long}

NADUT: array[1..134 {134}] of Integer =

(1, 5, 1, 1, 5, 1, 1, 1, 1, 2,

1, 1, 1, 2, 1, 1, 2, 2, 1, 1,

5, 2, 1, 2, 5, 1, 2, 2, 2, 1,

1, 1, 5, 2, 1, 5, 2, 2, 5, 2,

1, 1, 5, 2, 2, 1, 2, 1, 2, 2, 1, 2, 2, 2,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0,

0, 0, 0, 0, 0, 0, 0, 0);

NAD27: array[1..134 {134}] of Integer =(101, 102,

5010, 5300,  201,  202,  203,  301,  302,  401,  402,  403,

404,  405,  406,  407,  501,  502,  503,  600,  700,  901,

902,  903, 1001, 1002, 5101, 5102, 5103, 5104, 5105, 1101,

1102, 1103, 1201, 1202, 1301, 1302, 1401, 1402, 1501, 1502,

1601, 1602, 1701, 1702, 1703, 1801, 1802, 1900, 2001, 2002,

2101, 2102, 2103, 2111, 2112, 2113, 2201, 2202, 2203, 2301,

2302, 2401, 2402, 2403, 2501, 2502, 2503, 2601, 2602, 2701,

2702, 2703, 2800, 2900, 3001, 3002, 3003, 3101, 3102, 3103,

3104, 3200, 3301, 3302, 3401, 3402, 3501, 3502, 3601, 3602,

3701, 3702, 3800, 3901, 3902, 4001, 4002, 4100, 4201, 4202,

4203, 4204, 4205, 4301, 4302, 4303, 4400, 4501, 4502, 4601,

4602, 4701, 4702, 4801, 4802, 4803, 4901, 4902, 4903, 4904,

5001, 5002, 5003, 5004, 5005, 5006, 5007, 5008, 5009, 5201,

5202, 5400);

NAD83: array[1..134 {134}] of Integer = (101, 102,

5010, 5300,  201,  202,  203,  301,  302,  401,  402,  403,

404, 405,   406, 0000,  501,  502,  503,  600,  700,  901,

902,  903, 1001, 1002, 5101, 5102, 5103, 5104, 5105, 1101,

1102, 1103, 1201, 1202, 1301, 1302, 1401, 1402, 1501, 1502,

1601, 1602, 1701, 1702, 1703, 1801, 1802, 1900, 2001, 2002,

2101, 2102, 2103, 2111, 2112, 2113, 2201, 2202, 2203, 2301,

2302, 2401, 2402, 2403, 2500, 0000, 0000, 2600, 0000, 2701,

2702, 2703, 2800, 2900, 3001, 3002, 3003, 3101, 3102, 3103,

3104, 3200, 3301, 3302, 3401, 3402, 3501, 3502, 3601, 3602,

3701, 3702, 3800, 3900, 0000, 4001, 4002, 4100, 4201, 4202,

4203, 4204, 4205, 4301, 4302, 4303, 4400, 4501, 4502, 4601,

4602, 4701, 4702, 4801, 4802, 4803, 4901, 4902, 4903, 4904,

5001, 5002, 5003, 5004, 5005, 5006, 5007, 5008, 5009, 5200,

0000, 5400);

var

{FROM SPHDZ static double}

{  major: array[0..19 ] of Double =

( 6378206.4,  6378249.145, 6377397.155,  6378157.5,

6378388.0,  6378135.0,   6377276.3452, 6378145.0,

6378137.0,  6377563.396, 6377304.063,  6377340.189,

6378137.0,  6378155.0,   6378160.0,    6378245.0,

6378270.0,  6378166.0,   6378150.0,    6370997.0);}

major: array[0..30{SPHDCT-1}] of Double =(

6378206.4,		// 0: Clarke 1866 (default) */

6378249.145,		// 1: Clarke 1880 */

6377397.155,		// 2: Bessel */

6378157.5,		// 3: International 1967 */

6378388.0,		// 4: International 1909 */

6378135.0,		// 5: WGS 72 */

6377276.3452,		// 6: Everest */

6378145.0,		// 7: WGS 66 */

6378137.0,		// 8: GRS 1980 */

6377563.396,		// 9: Airy */

6377304.063,		// 10: Modified Everest */

6377340.189,		// 11: Modified Airy */

6378137.0,		// 12: WGS 84 */

6378155.0,		// 13: Southeast Asia */

6378160.0,		// 14: Australian National */

6378245.0,		// 15: Krassovsky */

6378270.0,		// 16: Hough */

6378166.0,		// 17: Mercury 1960 */

6378150.0,		// 18: Modified Mercury 1968 */

6370997.0,		// 19: Sphere of Radius 6370997 meters*/

6377483.865,		// 20: Bessel 1841(Namibia) */

6377298.556,		// 21: Everest (Sabah & Sarawak) */

6377301.243,		// 22: Everest (India 1956) */

6377295.664,		// 23: Everest (Malaysia 1969) */

6377304.063,		// 24: Everest (Malay & Singapr 1948)*/

6377309.613,		// 25: Everest (Pakistan) */

6378388.0,		// 26: Hayford */

6378200.0,		// 27: Helmert 1906 */

6378160.000,		// 28: Indonesian 1974 */

6378160.0,		// 29: South American 1969 */

6378165.0);		// 30: WGS 60 */

{static double}

minor: array[0..30]of Double = (

{   6356583.8,      6356514.86955,  6356078.96284, 6356772.2,

6356911.94613,  6356750.519915, 6356075.4133,  6356759.769356,

6356752.31414,  6356256.91,     6356103.039,   6356034.448,

6356752.314245, 6356773.3205,   6356774.719,   6356863.0188,

6356794.343479, 6356784.283666, 6356768.337303,6370997.0);}

6356583.8,		// 0: Clarke 1866 (default) */

6356514.86955,		// 1: Clarke 1880 */

6356078.96284,		// 2: Bessel */

6356772.2,		// 3: International 1967 */

6356911.94613,		// 4: International 1909 */

6356750.519915,		// 5: WGS 72 */

6356075.4133,		// 6: Everest */

6356759.769356,		// 7: WGS 66 */

6356752.31414,		// 8: GRS 1980 */

6356256.91,		// 9: Airy */

6356103.039,		// 10: Modified Everest */

6356034.448,		// 11: Modified Airy */

6356752.314245,		// 12: WGS 84 */

6356773.3205,		// 13: Southeast Asia */

6356774.719,		// 14: Australian National */

6356863.0188,		// 15: Krassovsky */

6356794.343479,		// 16: Hough */

6356784.283666,		// 17: Mercury 1960 */

6356768.337303,		// 18: Modified Mercury 1968 */

6370997.0,		// 19: Sphere of Radius 6370997 meters*/

6356165.382966,		// 20: Bessel 1841(Namibia) */

6356097.571445,		// 21: Everest (Sabah & Sarawak) */

6356100.228368,		// 22: Everest (India 1956) */

6356094.667915,		// 23: Everest (Malaysia 1969) */

6356103.038993,		// 24: Everest (Malay & Singapr 1948)*/

6356108.570542,		// 25: Everest (Pakistan) */

6356911.946128,		// 26: Hayford */

6356818.169,		// 27: Helmert 1906 */

6356774.504086,		// 28: Indonesian 1974 */

6356774.719,		// 29: South American 1969 */

6356783.287);		// 30: WGS 60 */

{PURPOSE:  The following functions are included in REPORT.C

INIT:Initializes the output device for error messages and report headings.

P_ERROR:Reports errors to the terminal, a specified file, or both.

PTITLE, RADIUS, RADIUS2, CENLON, CENLONMER, CENLAT, ORIGIN,

STANPARL, STPARL1, OFFSET, GENRPT, GENRPT_LONG,

PBLANK:Reports projection parameters to the terminal,specified file, or both.

}

{procedure PError(what, where: string);}

procedure DoProjError(What,i,i2,i3: Integer;iD:Double);

{function Init(ipr, jpr: Integer; efile, pfile: string): Integer;

procedure close_file; }

procedure Reported(A: string);

procedure ptitle(A: string);

procedure radius(A: double);

procedure radius2(A, B: double);

procedure cenlon(A: double);

procedure cenlonmer(A: double);

procedure cenlat(A: double);

procedure origin(A: double);

procedure stanparl(A, B: double);

procedure stparl1(A: double);

procedure offsetp(A, B: double);

procedure genrpt(A: double; S: string);

procedure GenrptLong(A: Integer; S: string);

procedure pblank;

{PURPOSE:  The following functions are included in CPROJ.C.}

function asinz(con: double): double;

{double asinz (double con);

ASINZ:	  Eliminates roundoff errors.}

function msfnz(eccent, sinphi, cosphi: double): double;

{double msfnz (double eccent, double sinphi, double cosphi);

MSFNZ:	  Computes the constant small m for Oblique Equal Area.}

function qsfnz(eccent, sinphi, cosphi: double): double;

{double qsfnz (double eccent, double sinphi, double cosphi);

QSFNZ:	  Computes the constant small q for Oblique Equal Area.}

function phi1z(eccent, qs: double; var flag: Integer): double;

{double phi1z (double eccent, double qs, long int *flag);

PHI1Z:	  Computes phi1 for Albers Conical Equal-Area.}

function phi2z(eccent, ts: double; var flag: Integer): double;

{double phi2z(double eccent, double ts, long int *flag);

PHI2Z:	  Computes the latitude angle, phi2, for Lambert

Conformal Conic and Polar Stereographic.... Mercator too}

function phi3z(ml, e0, e1, e2, e3: double; var flag: Integer):  double;

{double phi3z(double ml, double e0, double e1, double e2, double e3, long int *flag);

PHI3Z:	  Computes the latitude, phi3, for Equidistant Conic.}

function phi4z(eccent, e0, e1, e2, e3, a, b: double; var c, phi: double): double;

{double phi4z (double eccent, double e0, double e1, double e2, double e3,

double a, double b, double *c, double *phi);

PHI4Z:	  Computes the latitude, phi4, for Polyconic.              }

function pakcz(pak: double): double;

{double pakcz(double pak);

PAKCZ:	  Converts a 2 digit alternate packed DMS format to

standard packed DMS format.}

function pakr2dm(pak: double): double;

{double pakr2dm(double pak);

PAKR2DM:  Converts radians to 3 digit packed DMS format.}

(*double paksz(double ang, long int *iflg)

// angle which in DMS    error flag number *)

function Paksz(ang: double; var iflg: Integer): Double;

{function paksz(ang,iflg: double): double;}

function tsfnz(eccent, phi, sinphi: double): double;

{double tsfnz(double eccent, double phi, double sinphi);

TSFNZ:	  Computes the small t for Lambert Conformal Conic and

Polar Stereographic.}

function sign(x: double): Integer;

{int sign(double x);

SIGN:	  Returns the sign of an argument.}

function AdjustLon(x: double): double;

{double adjust_lon(double x);

ADJUST_LON:  Adjusts a longitude angle to range -180 to 180.}

function AdjustLonRadians(x: double): double;

// Angle in radians

function e0fn(x: double): double;

{double e0fn(double x);}

function e1fn(x: double): double;

{double e1fn(double x);}

function e2fn(x: double): double;

{double e2fn(double x);}

function e3fn(x: double): double;

{double e3fn(double x);}

{  E0FN, E1FN, E2FN, E3FN:

Computes the constants e0,e1,e2,and e3 for

calculating the distance along a meridian.}

function e4fn(x: double): double;

{double e4fn(double x);

E4FN:	  Computes e4 used for Polar Stereographic.}

function mlfn(e0, e1, e2, e3, phi: double): double;

{double mlfn(double e0, double e1, double e2, double e3, double phi);

MLFN:	  Computes M, the distance along a meridian.}

function CalcUtmZone(lon: double): Integer;

{long calc_utm_zone(double lon);

CALC_UTM_ZONE:	Calculates the UTM zone number.}

// Function to calculate UTM zone number--

//  NOTE Longitude entered in DEGREES!!!

Procedure sincos(val: double; var sin_val, cos_val: double);

{  SINCOS:	  Calculates the sine and cosine.}

(* Misc macros   ----------- Cproj.h

Function SQUARE(x)       x * x   /* x**2 */

#define CUBE(x)     x * x * x   /* x**3 */

#define QUAD(x) x * x * x * x   /* x**4 */

#define GMAX(A, B)      ((A) > (B) ? (A) : (B)) /* assign maximum of a and b */

#define GMIN(A, B)      ((A) < (B) ? (A) : (B)) /* assign minimum of a and b */

#define IMOD(A, B)      (A) - (((A) / (B)) * (B)) /* Integer mod function  Result:=A mod B */

*)

// Helpful functions for getting

//the string representations of units and datums

// from Projection.cpp

function ProjectiontoString(todoProjection :Integer): string;

function datumToString( todoDATUM :Integer ):string;

function unitToString( todoUNIT  :Integer ):string;

function packedDMSToDouble(  packedDMS:double ):double;

function packedDMSToString(  packedDMS:double;  bLat:boolean ):string;

implementation

uses

{$ifdef Tracer}

DRegForm, DGlobalsUnit;

{$endif} // Tracer

{$ifdef Locatable}

LMap,LoGlobals;

{$endif} // Locatable

{$ifdef Grip}

{uses LMap,LoGlobals;}

{$endif} // Grip

{$ifdef Sosmap}

{uses LMap,LoGlobals;  }

{$endif} // Sosmap

{$ifdef DTM}

{uses LMap,LoGlobals;}

{$endif} // DTM

procedure DoProjError(What,i,i2,i3: Integer;iD:Double);

var  { mtError a red stop sign.}

s:array[0..79] of Char;

MessageHdr,Messagestr: string;

HelpContextl: Longint;

begin

HelpContextl := 0;

case What of

-1: Messagestr :='User defined';

0:MessageHdr :='INFORMATIONAL';

1: MessageHdr :='phi1z-conv';

2: MessageHdr :='phi2z-conv';

3: MessageHdr :='PHI3Z-CONV';

4: MessageHdr :='phi4z-conv';

5: MessageHdr :='untfz-unit';

11: MessageHdr :='utm-forint';

12: MessageHdr :='utm-invint';

21: MessageHdr :='state-init';

31: MessageHdr :='alber-forinit';

32: MessageHdr :='alber-invinit';

41: MessageHdr :='lamcc-for';

42: MessageHdr :='lamcc-inv';

53: MessageHdr :='mer-forward';

81: MessageHdr :='eqcon_for';

93: MessageHdr :='tm-for';

95: MessageHdr :='TM-INVERSE';

103: MessageHdr :='ster-for';

113: MessageHdr :='lamaz-forward';

115: MessageHdr :='lamaz-inverse';

123: MessageHdr :='azim-for';

125: MessageHdr :='azim-inv';

133: MessageHdr :='gnomfor-conv';

143: MessageHdr :='orth-for';

145: MessageHdr :='orth-inv';

153: MessageHdr :='gvnsp-for';

155: MessageHdr :='gvnsp-for';

164: MessageHdr :='sinusoidal-inverse';

174: MessageHdr :='equi-inv';

201: MessageHdr :='omer-init';

202: MessageHdr :='omer-init..omerforint..omerinvint';

205: MessageHdr :='omer-for';

214: MessageHdr :='som-forward';

215: MessageHdr :='som-inverse';

234: MessageHdr :='robinv-conv';

235: MessageHdr :='alcon-inv';

241: MessageHdr :='IntMoll-forward';

242: MessageHdr :='Mollweide-forward';

243: MessageHdr :='Wagneriv-Forward';

251: MessageHdr :='goode-forward';

252: MessageHdr :='goode-inverse';

1101: MessageHdr :='untfz-code';

1116 : MessageHdr :='paksz-deg';

1117 : MessageHdr :='paksz-min';

1118 : MessageHdr :='paksz-sec';

2002 : MessageHdr :='Datum Conversion';

End;

case i of

-1: Messagestr :='Unsupported format';

0: Messagestr :=	'Invalid spheroid selection'

+#13#10+'Reset to 0';

2: Messagestr :=	'Convergence error';

3: Messagestr :=	'Latitude failed to converge after 15 iterations';

4: Messagestr :=	'Latitude failed to converge';

5: Messagestr :=	'Illegal source or target unit code';

6: Messagestr :=	'Output file name not specified';

11: Messagestr :=	'Illegal zone number';

21: Messagestr :=	'Illegal zone #'

+Inttostr(i2)+{<zone_code>}

' for spheroid # '

+Inttostr(i3){<sph_code>};

22: Messagestr :=	'Error opening State Plane parameter file';

23: Messagestr :=	'Illegal spheroid # '+Inttostr(i2){<sph_code>};

31: Messagestr :=	'Equal latitudes for St. Parallels'

+#13#10+'on opposite sides of equator';

41: Messagestr :=	'Equal Latitudes for St. Parallels'

+#13#10+'on opposite sides of equator';

44: Messagestr :=	'Point can not be projected';

53: Messagestr :=	'Transformation cannot be computed at the poles';

81: Messagestr :=	'Standard Parallels on opposite sides of equator';

93: Messagestr :=	'Point projects into infinity';

95: Messagestr :=	'Latitude failed to converge';

103: Messagestr := 'Point projects into infinity';

113: Messagestr :=	'Point projects to a circle of radius = '+Inttostr(i2){ <radius>};

115: Messagestr :=	'Input data error';

123: Messagestr :=	'Point projects into a circle of radius = '+Floattostr(iD){ <radius>};

125: Messagestr :=	'Input data error';

133: Messagestr :=	'Point projects into infinity';

143: Messagestr :=	'Point can not be projected';

145: Messagestr :=	'Input data error';

153: Messagestr :=	'Point cannot be projected';

155: Messagestr :=	'Input data error';

164: Messagestr :=	'Input data error';

174: Messagestr :=	'Input data error';

201: Messagestr :=	'Input data error';

202: Messagestr :=	'Input data error';

205: Messagestr :=	'Point projects into infinity';

214: Messagestr :=	'50 iterations without convergence';

234: Messagestr :=	'Too many iterations in inverse';

235: Messagestr :=	'Too many iterations in inverse';

236: Messagestr :=	'Too many iterations in inverse';

241: Messagestr :=	'Iteration failed to converge';

251: Messagestr :=	'Iteration failed to converge';

252: Messagestr :=	'Input data error';

1101: Messagestr :=	'Incompatable unit codes'+#13#10+

unitToString(i2) + '  '+unitToString(i3);

1116: Messagestr :=	'Illegal DMS field';

2002: Messagestr :=	'FATAL ERROR: '+Inttostr(i2);

end;

{$ifdef Tracer}

RegistrationForm.ProLogMemo.Lines.Append(MessageHdr);

RegistrationForm.ProLogMemo.Lines.Append(Messagestr);

{$endif} // Tracer

{$ifdef Locatable}

MapForm.ProLogMemo.Lines.Append(MessageHdr);

MapForm.ProLogMemo.Lines.Append(Messagestr);

{$endif} // Locatable

{$ifdef Grip}

{uses LMap,LoGlobals;}

{$endif} // Grip

{$ifdef Sosmap}

{uses LMap,LoGlobals;  }

{$endif} // Sosmap

{$ifdef DTM}

{uses LMap,LoGlobals;}

{$endif} // DTM

if ((ErrorBeepOn) and (not VoicesON)) then Beep;

if ((ErrorBeepOn) and (VoicesON)    { Beep(10000, 1000);}

and (FileExists(ExtractFilePath(ParamStr(0))

+ 'ERRORV.WAV')) )then

begin

StrPCopy(s, ExtractFilePath(ParamStr(0)) + 'ERRORV.WAV');

if (MMSysHandle <> 0) then  PlaySound(s,  0);

end;

MessageDlgPos(MessageHdr+#13#10+Messagestr, mtError,[mbOK], {mbYesNoCancel,}

HelpContextl, MessageX, MessageY);

end;

//  from report Function to report errors

//  void p_error(char *what, char *where)

//  initialize output device

//  int init(long int ipr, long int jpr, char *efile, char *pfile)

//  flag for printing errors (0,1,or 2)

//  flag for printing parameters (0,1,or 2)

//  name of error file

//  name of parameter file

//  Omitted due to rewrite... use Memo and Defines

//  Functions to report projection parameters

procedure Reported(A: string);

begin

{$ifdef Tracer}

RegistrationForm.ProLogMemo.Lines.Append(A);

{$endif} // Tracer

{$ifdef Locatable}

MapForm.ProLogMemo.Lines.Append(A);

{$endif} // Locatable

{$ifdef Grip}

{uses LMap,LoGlobals;}

{$endif} // Grip

{$ifdef Sosmap}

{uses LMap,LoGlobals;  }

{$endif} // Sosmap

{$ifdef DTM}

{uses LMap,LoGlobals;}

{$endif} // DTM

End;

procedure ptitle(A: string);

begin

Reported('PROJECTION PARAMETERS:');

Reported(A);

{  MapForm.ProLogMemo.Lines.Append('PROJECTION PARAMETERS:');

MapForm.ProLogMemo.Lines.Append(A);}

end;

procedure radius(A: double);

begin

Reported('Radius of Sphere: '+Floattostr(A));

{  MapForm.ProLogMemo.Lines.Append('Radius of Sphere: '+Floattostr(A));}

end;

procedure radius2(A, B: double);

begin

{fprintf(fptr_p,"   Semi-Minor Axis of Ellipsoid:     %lf meters\n",B);}

Reported('Semi-Major Axis of Ellipsoid: meters\'+Floattostr(A));

{  MapForm.ProLogMemo.Lines.Append('Semi-Major Axis of Ellipsoid: meters\'+Floattostr(A));}

Reported('Semi-Minor Axis of Ellipsoid: meters\'+Floattostr(B));

{  MapForm.ProLogMemo.Lines.Append('Semi-Minor Axis of Ellipsoid: meters\'+Floattostr(B));}

end;

procedure cenlon(A: double);

begin

{fprintf(fptr_p,"   Longitude of Center:     %lf degrees\n",A*R2D);}

Reported('Longitude of Center: degrees\'+Floattostr(A*R2D));

{  MapForm.ProLogMemo.Lines.Append('Longitude of Center: degrees\'+Floattostr(A*R2D));}

end;

procedure cenlonmer(A: double);

begin

{printf("   Longitude of Central Meridian:     %lf degrees\n",A*R2D);}

Reported('Longitude of Central Meridian: degrees\'+Floattostr(A*R2D));

{  MapForm.ProLogMemo.Lines.Append('Longitude of Central Meridian: degrees\'+Floattostr(A*R2D));}

end;

procedure cenlat(A: double);

begin

{printf("   Latitude  of Center:     %lf degrees\n",A*R2D);}

Reported('Latitude of Center: degrees\'+Floattostr(A*R2D));

{  MapForm.ProLogMemo.Lines.Append('Latitude of Center: degrees\'+Floattostr(A*R2D));}

end;

procedure origin(A: double);

begin

{printf("   Latitude of Origin:     %lf degrees\n",A*R2D);}

Reported('Latitude  of Origin: degrees\'+Floattostr(A*R2D));

{  MapForm.ProLogMemo.Lines.Append('Latitude  of Origin: degrees\'+Floattostr(A*R2D));}

end;

procedure stanparl(A, B: double);

begin

{printf("   1st Standard Parallel:     %lf degrees\n",A*R2D);}

Reported('1st Standard Parallel: degrees\'+Floattostr(A*R2D));

{  MapForm.ProLogMemo.Lines.Append('1st Standard Parallel: degrees\'+Floattostr(A*R2D));}

Reported('2nd Standard Parallel: degrees\'+Floattostr(B*R2D));

{  MapForm.ProLogMemo.Lines.Append('2nd Standard Parallel: degrees\'+Floattostr(B*R2D));}

end;

procedure stparl1(A: double);

begin

{printf("   Standard Parallel:     %lf degrees\n",A*R2D);}

Reported('Standard Parallel: degrees\'+Floattostr(A*R2D));

{  MapForm.ProLogMemo.Lines.Append('Standard Parallel: degrees\'+Floattostr(A*R2D));}

end;

procedure offsetp(A, B: double);

begin

{fprintf(fptr_p,"   False Easting:      %lf meters \n",A);}

Reported('False Easting: meters\'+Floattostr(A));

{  MapForm.ProLogMemo.Lines.Append('False Easting: meters\'+Floattostr(A));}

Reported('False Northing: meters\'+Floattostr(B));

{  MapForm.ProLogMemo.Lines.Append('False Northing: meters\'+Floattostr(B));}

end;

procedure genrpt(A: double; S: string);

begin {

{printf("   %s %lf\n", S, A);}

Reported(S+Floattostr(A));

{  MapForm.ProLogMemo.Lines.Append(S+Floattostr(A));}

end;

procedure GenrptLong(A: Integer; S: string);

begin

{printf("   %s %d\n", S, A);}

Reported(S+Floattostr(A));

{  MapForm.ProLogMemo.Lines.Append(S+Floattostr(A));}

end;

procedure pblank;

begin

{fprintf(fptr_p,"\n");}

Reported(#13#10);

{  MapForm.ProLogMemo.Lines.Append(#13#10);}

end;

{from CPROJ}

//* Function to eliminate roundoff errors in asin

//----------------------------------------------*/

function asinz(con: double): double;

begin

if (abs(con) > 1.0) then

begin

if (con > 1.0) then con := 1.0

else con := -1.0;

end;

result := (arcsin(con));

{ ArcSin returns the inverse sine of X.

X must be between -1 and 1.

The return value will be in the range [-Pi/2..Pi/2], in radians.}

end;

//* Function to compute the constant small m which is the radius of

//   a parallel of latitude, phi, divided by the semimajor axis.

//---------------------------------------------------------------*/

function msfnz(eccent, sinphi, cosphi: double): double;

var con: double;

begin

con := eccent * sinphi;

result := ((cosphi / (sqrt(1.0 - con * con))));

end;

//* Function to compute constant small q which is the radius of a

//   parallel of latitude, phi, divided by the semimajor axis.

//------------------------------------------------------------*/

function qsfnz(eccent, sinphi, cosphi: double): double;

var con: double;

begin

if (eccent > 1.0E-7) then

begin

con := eccent * sinphi;

result := ((1.0 - eccent * eccent)

* (sinphi / (1.0 - con * con) - (0.5 / eccent) *

Ln((1.0 - con) / (1.0 + con))));

{e := Exp(1.0);

Ln returns the natural logarithm (Ln(e) = 1)

of the real-type expression X.}

end

else result := (2.0 * sinphi);

end;

//* Function to compute phi1, the latitude for the inverse of the

//   Albers Conical Equal-Area projection.

//-------------------------------------------*/

function phi1z(eccent, qs: double; var flag: Integer): double;

//* Eccentricity angle in radians		*/

//* Angle in radians				*/

//* Error flag number				*/

var

eccnts, dphi, con, com, sinpi, cospi, phi: Double;

{double asinz(double con);}

i: Integer;

begin

phi := asinz(0.5 * qs);

if (eccent < EPSLN) then

begin

result := (phi);

exit;

end;

eccnts := eccent * eccent;

for i := 1 to 25 do

begin

sincos(phi, sinpi, cospi);

con := eccent * sinpi;

com := 1.0 - con * con;

dphi := 0.5 * com * com / cospi * (qs / (1.0 - eccnts) - sinpi /

com +

0.5 / eccent * ln((1.0 - con) / (1.0 + con)));

phi := phi + dphi;

if (abs(dphi) <= 1E-7) then

begin

result := (phi);

exit;

end;

end;

{  PError('Convergence error', 'phi1z-conv');}

DoProjError(1,2,0,0,0);

flag := 001;

result := (ERROR);

end;

//* Function to compute the latitude angle, phi2, for the inverse of the

//   Lambert Conformal Conic and Polar Stereographic projections.

//----------------------------------------------------------------*/

function phi2z(eccent, ts: double; var flag: Integer): double;

//* Spheroid eccentricity		*/

//* Constant value t			*/

//* Error flag number			*/

var

eccnth, phi, con, dphi, sinpi: double;

i: Integer;

begin

flag := 0;

eccnth := 0.5 * eccent;

phi := HALF_PI - 2 * arctan(ts);

for i := 0 to 15 do

begin

sinpi := sin(phi);

con := eccent * sinpi;

dphi := HALF_PI -

2 * arctan(ts * (power( ((1.0 - con)

/ (1.0 + con)), eccnth) ) )

- phi;

phi := phi + dphi;

if (abs(dphi) <= 0.0000000001) then

begin

result := (phi);

exit;

end;

end;

{  PError('Convergence error', 'phi2z-conv');}

DoProjError(2,2,0,0,0);

flag := 002;

result := (002);

end;

// Function to compute latitude, phi3,

//for the inverse of the Equidistant Conic projection.

//-----------------------------------------------------------------

function phi3z(ml, e0, e1, e2, e3: double; var flag: Integer): double;

// Constant, Constant, Constant, Constant,Constant	, Error flag number

var

phi, dphi: double;

i: Integer;

begin

phi := ml;

for i := 0 to 15 do

begin

dphi := (ml + e1 * sin(2.0 * phi) - e2 * sin(4.0 * phi) + e3 *

sin(6.0 * phi))

/ e0 - phi;

phi := phi + dphi;

if (abs(dphi) <= 0.0000000001) then

begin

flag := 0;

result := (phi);

exit;

end;

end;

{  PError('Latitude failed to converge after 15 iterations',

'PHI3Z-CONV');}

DoProjError(3,3,0,0,0);

flag := 3;

result := (3);

end;

// Function to compute, phi4,

//the latitude for the inverse of the Polyconic projection.

//------------------------------------------------------------

function phi4z(eccent, e0, e1, e2, e3, a, b: double;

var c, phi: double): double;

// Spheroid eccentricity squared

var

sinphi, sin2ph, tanphi, ml, mlp, con1, con2, con3, dphi: double;

i: Integer;

begin

phi := a;

for i := 1 to 15 do

begin

sinphi := sin(phi);

tanphi := tan(phi);

c := tanphi * sqrt(1.0 - eccent * sinphi * sinphi);

sin2ph := sin(2.0 * phi);

{        ml = e0 * *phi - e1 * sin2ph + e2 * sin (4.0 *  *phi);

mlp = e0 - 2.0 * e1 * cos (2.0 *  *phi) + 4.0 * e2 *

cos (4.0 *  *phi);}

ml := e0 * phi - e1 * sin2ph + e2 * sin(4.0 * phi) - e3 *

sin(6.0 * phi);

mlp := e0 - 2.0 * e1 * cos(2.0 * phi) + 4.0 * e2 *

cos(4.0 * phi) - 6.0 * e3 * cos(6.0 * phi);

con1 := 2.0 * ml + c * (ml * ml + b) - 2.0 * a * (c * ml + 1.0);

con2 := eccent * sin2ph * (ml * ml + b - 2.0 * a * ml) / (2.0 *

c);

con3 := 2.0 * (a - ml) * (c * mlp - 2.0 / sin2ph) - 2.0 * mlp;

dphi := con1 / (con2 + con3);

phi := phi + dphi;

if (abs(dphi) <= 0.0000000001) then begin

result := (OK);

exit;

end;

end;

{  PError('Lattitude failed to converge', 'phi4z-conv'); }

DoProjError(4,4,0,0,0);

result := (004);

end;

// Function to convert 2 digit alternate packed DMS format (+/-)DDDMMSS.SSS

//   to 3 digit standard packed DMS format (+/-)DDDMMMSSS.SSS.

//-----------------------------------------------------------------

function pakcz(pak: double): double;

// Angle in alternate packed DMS format

var

con, secs: double;

degs, mins: Integer;

sgna: char;

begin

sgna := ' ';

if (pak < 0.0) then sgna := '-';

con := abs(pak);

degs := Round((con / 10000.0) + 0.001);

con := con - degs * 10000;

mins := Round((con / 100.0) + 0.001);

secs := con - mins * 100;

con := (degs) * 1000000.0 + (mins) * 1000.0 + secs;

if (sgna = '-') then con := -con;

result := (con);

end;

// Function to convert radians to

// 3 digit packed DMS format (+/-)DDDMMMSSS.SSS

//---------------------------------------------------------

function pakr2dm(pak: double): double;

// Angle in radians

var

con, secs: double;

degs, mins: Integer;

sgna: char;

begin

sgna := ' ';

pak := pak * R2D;

if (pak < 0.0) then sgna := '-';

con := abs(pak);

degs := round(con);

con := (con - degs) * 60;

mins := round(con);

secs := (con - mins) * 60;

con := ((degs) * 1000000.0 + (mins) * 1000.0 + secs);

if (sgna = '-') then con := -con;

result := (con);

end;

{function paksz(ang,iflg: double): double;

PURPOSE:	This function converts a packed DMS angle to seconds.  The

standard packed DMS format is:

degrees * 1000000 + minutes * 1000 + seconds

Example:	ang = 120025045.25 yields

deg = 120

min = 25

sec = 45.25

The algorithm used for the conversion is as follows:

1.  The absolute value of the angle is used.

2.  The degrees are separated out:

deg = ang/1000000 	(fractional portion truncated)

3.  The minutes are separated out:

min = (ang - deg * 1000000) / 1000 	   (fractional

portion	truncated)

4.  The seconds are then computed:

sec = ang - deg * 1000000 - min * 1000

5.  The total angle in seconds is computed:

sec = deg * 3600.0 + min * 60.0 + sec

6.  The sign of sec is set to that of the input angle.}

(*

function paksz(ang,iflg: double): double;

{double ang;		 angle which in DMS

long *iflg;	error flag number}

var

fac,		// sign flag

deg,		// degree variable

min,		// minute variable

sec,		// seconds variable

tmp:Double;		// temporary variable

i:Integer;		 // variable

Begin

{iflg := 0;}

if (ang < 0.0)then fac := -1 else   fac := 1;

// find degrees

sec := abs(ang);

tmp := 1000000.0;

i := Round( sec/tmp);

if (i > 360)then

begin

{  p_error("Illegal DMS field","paksz-deg"); }

iflg := 1116;

DoProjError(1116,1116,0,0,iflg);

Result:=(ERROR);

exit;

end

else

deg := i;

// find minutes

sec := sec - deg * tmp;

tmp := 1000;

i := Round( sec / tmp);

if (i > 60)then

begin

{  p_error("Illegal DMS field","paksz-min");}

iflg := 1116;

DoProjError(1116,1116,0,0,iflg);

Result:=(ERROR);

exit;

end else  min := i;

// find seconds

sec := sec - min * tmp;

if (sec > 60)then

begin

{  p_error("Illegal DMS field","paksz-sec");}

iflg := 1116;

DoProjError(1116,1116,0,0,iflg);

Result:=(ERROR);

exit;

end else  sec := fac * (deg * 3600.0 + min * 60.0 + sec);

deg := sec / 3600.0;

Result:=(deg);

End;

*)

(* double paksz(double ang, long int *iflg)

/* angle which in DMS		*/

/* error flag number		*/

NAME                            PAKSZ

PURPOSE:	This function converts a packed DMS angle to seconds.

The standard packed DMS format is:

degrees * 1000000 + minutes * 1000 + seconds

Example:	ang = 120025045.25 yields

deg = 120

min = 25

sec = 45.25

The algorithm used for the conversion is as follows:

1.  The absolute value of the angle is used.

2.  The degrees are separated out:

deg = ang/1000000 	(fractional portion truncated)

3.  The minutes are separated out:

min = (ang - deg * 1000000) / 1000 	   (fractional

portion	truncated)

4.  The seconds are then computed:

sec = ang - deg * 1000000 - min * 1000

5.  The total angle in seconds is computed:

sec = deg * 3600.0 + min * 60.0 + sec

6.  The sign of sec is set to that of the input angle.

*)

function Paksz(ang: double; var iflg: Integer): Double;

var

fac, //* sign flag			*/

deg, //* degree variable		*/

min, //* minute variable		*/

sec, //* seconds variable		*/

tmp: double; //* temporary variable		*/

i: Integer; //* temporary variable		*/

begin

iflg := 0;

if (ang < 0.0) then fac := -1 else fac := 1;

//* find degrees-------------*/

sec := abs(ang);

tmp := 1000000.0;

i := round(sec / tmp);

if (i > 360) then

begin

{PError('Illegal DMS field', 'paksz-deg');}

DoProjError(1116,1116,0,0,0);

iflg := 1116;

result := (ERROR);

Exit;

end

else deg := i;

//* find minutes-------------*/

sec := sec - (deg * tmp);

tmp := 1000;

i := round(sec / tmp);

if (i > 60) then

begin

{PError('Illegal DMS field', 'paksz-min');  }

DoProjError(1117,1116,0,0,0);

iflg := 1116;

result := (ERROR);

Exit;

end else min := i;

//* find seconds-------------*/

sec := sec - (min * tmp);

if (sec > 60) then

begin

{ PError('Illegal DMS field', 'paksz-sec');}

DoProjError(1118,1116,0,0,0);

iflg := 1116;

result := (ERROR);

Exit;

end

else sec := fac * (deg * 3600.0 + min * 60.0 + sec);

deg := sec / 3600.0;

result := (deg);

end;

//////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////

// Function to compute the constant small t for use in the forward

//   computations in the Lambert Conformal Conic and the Polar

//   Stereographic projections.

//--------------------------------------------------------------

function tsfnz(eccent, phi, sinphi: double): double;

// Eccentricity of the spheroid

// Latitude phi

// Sine of the latitude

var

con, com: double;

begin

con := eccent * sinphi;

com := 0.5 * eccent;

con := power(((1.0 - con) / (1.0 + con)), com);

result := (tan(0.5 * (HALF_PI - phi)) / con);

end;

//* Function to return the sign of an argument

//  ------------------------------------------*/

function sign(x: double): Integer;

begin

if (x < 0.0) then result := (-1) else result := (1);

end;

// Function to adjust a longitude angle to range from

// -180 to 180 radians   added if statments

//  --------------------------------------------------------

// Modified by Mike Childs - 4/21/98

function AdjustLon(x: double): double;

begin

{    // Check to see if x is already OK}

if (abs(x) <= PI) then result := x

else

begin

x := x - sign(x) * Round(x / (2 * PI)) * 2 * PI;

if (x > PI) then x := x - (2 * PI);

result := x;

end;

end;

function AdjustLonRadians(x: double): double;

// Angle in radians

var count: Integer;

begin

for count := 0 to MAX_VAL do

begin

if (abs(x) <= PI) then break

else

if ((abs(x / PI)) < 2) then

x := x - (sign(x) * TWO_PI)

else

if ((abs(x / TWO_PI)) < MAXLONG) then

begin

x := x - (((x / TWO_PI)) * TWO_PI);

end

else

if ((abs(x / (MAXLONG * TWO_PI))) < MAXLONG) then

begin

x := x - (((x / (MAXLONG * TWO_PI)))

* (TWO_PI * MAXLONG));

end

else

if ((abs(x / (DBLLONG * TWO_PI))) < MAXLONG) then

begin

x := x - (((x / (DBLLONG * TWO_PI)))

* (TWO_PI * DBLLONG));

end

else

x := x - (sign(x) * TWO_PI);

end;

result := x;

end;

//* Functions to compute the constants e0, e1, e2, and e3 which are used

//   in a series for calculating the distance along a meridian.  The

//   input x represents the eccentricity squared.

//----------------------------------------------------------------*/

function e0fn(x: double): double;

begin result := (1.0 - 0.25 * x * (1.0 + x / 16.0 * (3.0 + 1.25 *

x))); end;

function e1fn(x: double): double;

begin result := (0.375 * x * (1.0 + 0.25 * x * (1.0 + 0.46875 * x)));

end;

function e2fn(x: double): double;

begin result := (0.05859375 * x * x * (1.0 + 0.75 * x)); end;

function e3fn(x: double): double;

begin result := (x * x * x * (35.0 / 3072.0)); end;

//* Function to compute the constant e4 from the input of the eccentricity

//   of the spheroid, x.  This constant is used in the Polar Stereographic

//   projection.

//--------------------------------------------------------------------*/

function e4fn(x: double): double;

var

con, com: double;

begin

con := 1.0 + x;

com := 1.0 - x;

Result := (sqrt((power(con, con)) * (power(com, com))));

end;

// Function computes the value of M which is the distance along a meridian

//   from the Equator to latitude phi.

//------------------------------------------------

function mlfn(e0, e1, e2, e3, phi: double): double;

begin

Result := (e0 * phi - e1 * sin(2.0 * phi) + e2 * sin(4.0 * phi) - e3

* sin(6.0 * phi));

end;

// Function to calculate UTM zone number--

//  NOTE Longitude entered in DEGREES!!!

//  ---------------------------------------------------------------------------*/

function CalcUtmZone(lon: double): Integer;

begin

Result := Round((((lon + 180.0) / 6.0) + 1.0));

end;

(*/* Not part of the original gctpc distribution. */

#ifdef NEED_SINCOS

void

sincos(double val, double *sin_val, double *cos_val);

procedure SinCos(Theta: Extended; var Sin, Cos: Extended); register;

Description

Call SinCos to obtain the sine and cosine

of an angle expressed in radians.

Theta specifies the angle.

The sine and cosine are returned

by the Sin and Cos parameters, respectively.

#endif*)

procedure sincos(val: double; var sin_val, cos_val: double);

begin

SinCos(val, sin_val, cos_val);

end;

function ProjectiontoString(todoProjection :Integer): string;

var  str :string;

Begin

// TODO: Make this better

str := 'DATUM: ';

str :=str+ datumToString(1{ d_datum} );

str :=str+ '\r\nUNIT: ';

str :=str+ unitToString( 2{d_unit} );

str :=str+ '\r\n';

Result:=str;

End;

function datumToString( todoDATUM :Integer ):string;

Begin

Case  todoDATUM  of

0: {ADINDAN:}

Result:= 'ADINDAN';

1: {ARC1950:}

Result:= 'ARC1950';

2: {ARC1960:}

Result:= 'ARC1960';

3: {AUSTRALIAN_GEODETIC_1966:}

Result:= 'AUSTRALIAN GEODETIC 1966';

4: {AUSTRALIAN_GEODETIC_1984:}

Result:= 'AUSTRALIAN GEODETIC 1984';

5: {CAMP_AREA_ASTRO:}

Result:= 'CAMP AREA ASTRO';

6: {CAPE:}

Result:= 'CAPE';

7: {EUROPEAN_DATUM_1950:}

Result:= 'EUROPEAN DATUM 1950';

8: {EUROPEAN_DATUM_1979:}

Result:= 'EUROPEAN DATUM 1979';

9: {GEODETIC_DATUM_1949:}

Result:= 'GEODETIC DATUM 1949';

10: {HONG_KONG_1963:}

Result:= 'HONG KONG 1963';

11: {HU_TZU_SHAN:}

Result:= 'HU TZU SHAN';

12: {INDIAN:}

Result:= 'INDIAN';

13: {NAD27:}

Result:= 'NAD27';

14: {NAD83:}

Result:= 'NAD83';

15: {OLD_HAWAIIAN_MEAN:}

Result:= 'OLD HAWAIIAN MEAN';

16: {OMAN:}

Result:= 'OMAN';

17: {ORDNANCE_SURVEY_1936:}

Result:= 'ORDNANCE SURVEY 1936';

18: {PULKOVO_1942:}

Result:= 'PULKOVO 1942';

19: {PROVISIONAL_S_AMERICAN_1956:}

Result:= 'PROVISIONAL SOUTH AMERICAN 1956';

20: {TOKYO:}

Result:= 'TOKYO';

21: {WGS_72:}

Result:= 'WGS72';

22: {WGS_84:}

Result:= 'WGS84';

23: {NO_DATUM:}

Result:= 'NO DATUM';

else

Result:= 'Unknown Datum';

end;

End;

function unitToString( todoUNIT  :Integer ):string;

Begin

case todoUNIT of

0{RADIANS}:

Result:= 'radians';

1 {US_FEET}:

Result:= 'feet';

2 {METERS}:

Result:= 'meters';

3 {ARC_SECONDS}:

Result:= 'arc seconds';

4 {ARC_DEGREES}:

Result:= 'arc degrees';

5 {INTERNATIONAL_FEET}:

Result:= 'international feet';

6 {STATE_ZONE_TABLE}:

Result:= 'state zone table units';

else{default}

Result:= 'unknown units';

end;{Case}

End;

function packedDMSToDouble(  packedDMS:double ):double;

var  min,deg,sign:Double;

Begin

sign := 1.0;

if ( packedDMS < 0.0 )then

begin

sign := -1.0;

packedDMS := -packedDMS;

end;

deg := packedDMS / 1000000;

packedDMS := packedDMS-(deg * 1000000);

min := packedDMS / 1000;

packedDMS := packedDMS-(min * 1000);

result := deg;

result := result+(min / 60.0);

result := result+(packedDMS / 3600.0);

result := result*sign;

End;

function packedDMSToString(  packedDMS:double;  bLat:boolean ):string;

Begin

{string

Projection::packedDMSToString( double packedDMS, bool bLat )

begin

char nsFlag = 'N';

char ewFlag = 'E';

if ( packedDMS < 0 )

begin

nsFlag = 'S';

ewFlag = 'W';

packedDMS *= -1;

end

short deg = packedDMS / 1000000;

packedDMS -= deg * 1000000;

short min = packedDMS / 1000;

packedDMS -= min * 1000;

double sec = packedDMS;

char flag = bLat ? nsFlag : ewFlag;

char strTemp[20];

sprintf( strTemp, "%d %02d\' %05.2f\" %c", deg, min, sec, flag );

return strTemp;

}

Result:='';

End;

end.