EED3 SCHEMATICS ASCII DATABASE FILE FORMAT

Schematics ASCII Database records are explained in sequence in which they

MUST appear in the file.

The first parameter in each record is record type. Record type parameter

must ALWAYS be of fixed (3 characters) lenght.

I.  Schematics Database Common Data

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Header      |

|

| 000,databasename,s,sunit,cunit,cref,Xmin,Ymin,Xmax,Ymax

|

| where:

|

|  000              record type

|

|  database name    1 to 8 characters of database name

|

|  s                schematics database flag

|                   if s=S then EED III database

|                   if s=S2 then EED II conversion database

|

|  sunit            if I then all widths/sizes in inches

|                   if M then all widths/sizes in mm

|

|  cunit            if I then all coordinates in inches

|                   if M then all coordinates in mm

|

|  Cref             if B then all coordinates as in schematcs

|                   database i.e x0,y0 in the center of format

|                   outline

|                   if R then all coordinates as displacement from

|                   Xmin,Ymin of format outline

|  Xmin,Ymin        Coordinates of Xmin,Ymin of format outline

|                    (MAINPAGE)

|

|  Xmax,Ymax        Coordinates of Xmax,Ymax of format outline

|                    (MAINPAGE)

|

| Example:

|

|  000,L,GPOST   ,I,I,B,-2.500,-2.500,2.500,2.500

|

-------------------------------------------------------------------------------

Integration |

status      | 002,intgstatus

Only with   |

integrated  | where:

databases   |

|  001              record type

|

|  intgstatus       database integration status

|

|        b3         1 - back annotation required

|        b2         1 - front annotation required

|        b1         1 - layout database exists

|        b0         1 - schematics databse exists

|

| Example:

|

|  002,11

-------------------------------------------------------------------------------

Text        |

size table  | 030,sizecode,xsize,ysize,dcode

|

| where:

|

|  030              record type

|

|  sizecode         text character size code in range from 1 to 7

|

|  xsize            text character vertical dimension

|

|  ysize            text character horizontal dimension

|

|

| Text size table consists of 7 records.  Text  character  x

| and  y  sizes must multiplication of .012" which is lowest

| allowed text sizes.  If  xsize=0  and  ysize=0  then  EED3

| defaults will be used.

| Maximal text size is 1.200".

| Ascending sequence for size codes is required.

|

| Example:

|

|  030, 1,.060,.040

|  030, 2,.120,.080

|  030, 3,.180,.160

|  030, 4,.240,.160

|         .

|         .

|

-------------------------------------------------------------------------------

II. Symbol Library

All coordinates are expressed as displacement from coordinates

of entry number 1 which are always X=0 and Y=0

Symbol definition consist of several records. Structure is

following:

Record type:   Number:      Contents:

100             1            symbol header

101             1 or more    entry definition

102             0 or more    single line primitive

103             0 or more    single rectangle primitive

104             0 or more    single circle primitive

105             0 or more    single arc primitive

106             0 or more    single text string

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Library     |

symbol      | 100,smbref,smbname,xname,yname,xdesc,ydesc,entries

header      |

| where:

|

|  100              record type

|

|  smbref           symbol reference number within database

|

|  smbname          symbol name 1 to 8 characters

|

|  xname,yname      coordinates for component name

|

|  xdesc,ydesc      coordinates for component description text

|

|  entries          number of entries in symbol

|

| Example:

|

|   100, 1,R47     ,.200,.200,.400,.400, 14

|

------------------------------------------------------------------------------

Single      |

entry/pad   | 101,xentry,yentry,xpatxt,ypatxt,parot

definition  |

| where

|

|  101              record type

|

|  xentry,yentry    entry coordinates

|

|  xpatxt,ypatxt    pin assignment (pinout) text location

|

|  parot            pin assignment (pinout) text rotation parot*90

|

| Example:

|

| 101,.000,.000,.-050,.010,0

|                  .

-------------------------------------------------------------------------------

Line        |

primitive   | 102,xstrt,ystrt,xend,yend,stylecode,side,mask,link

|

| where:

|

|  102              record type

|

|  xstrt,ystrt      start of line coordinates

|

|  xend,yend        end of line coordinates

|

|  stylecode        style code 0 to 6

|

|  side             0

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         1 - filling line 0 - common line

|        b0         1

|

|  link             0 for outlines

|

| Example:

|

|  102,-.100,.100,.050,.100, 0, 0, 1, 0

-------------------------------------------------------------------------------

Rectangle   |

primitive   | 103,xstrt,ystrt,xdim,ydim,stylecode,side,mask,link

|

| where:

|

|  103              record type

|

|  xstrt,ystrt      start corner coordinates

|

|  xdim,ydim        horizontal and vertical dimensions

|

|  stylecode        style code 0 to 6

|

|  side             0

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         0

|        b0         1

|

|  link             0

|

| Example:

|

|

|  103,-.100,.100,.050,.100, 0, 0, 1, 0

-------------------------------------------------------------------------------

Circle      |

primitive   | 104,xcent,ycent,radius,stylecode,side,mask,link

|

| where:

|

|  104              record type

|

|  xcent,ycent      circle center coordinates

|

|  radius           circle radius

|

|  stylecode        style code 0 to 6

|

|  side             0

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         0

|        b0         1

|

|  link             0

|

| Example:

|

|  104,-.100,.100,.250, 0, 0, 1, 0

-------------------------------------------------------------------------------

Arc         |

primitive   | 105,xcent,ycent,radius,sangle,aangle,stylecode,side,mask,link

|

| where:

|

|  105              record type

|

|  xcent,ycent      circle center coordinates

|

|  radius           circle radius

|

|  sangle           arc start angle 0-360 deg.

|

|  aangle           arc angle 0=360 deg.

|

|  stylecode        style code 0 to 6

|

|  side             0

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         0

|        b0         1

|

|  link             0

|

| Example:

|

|  105,-.100,.100,.250, 45, 180, 0, 0, 1, 0

-------------------------------------------------------------------------------

Text        |

string      | 106,xtxt,ytxt,rot,sizecode,color,mask,link,textstring

|

| where:

|

|  106              record type

|

|  xtxt,ytxt        first character in string coordinates

|

|  rot              text rotation rot*90 deg

|

|  sizecode         text size code

|

|  color            text color

|           0       color of component outlines

|           1       color of component name

|           2       color of component description

|           3       color of pinout

|           4       color of component text

|           5       color of bus

|           6       color of bus name

|           7       color of bus memeber

|           8       color of wire

|           9       color of net name

|          10       color of net label

|          11       color of free text

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0 - common text  1 - COMPNAME,COMPDESC

|        b2         0

|        b1         0

|        b0         1

|

|  link             0 - common text

|                   1 - COMPNAME

|                   2 - COMPDESC

|

|  textstring       text string

|

| Example:

|

|  106,.200,.200, 0, 1, 0, 8, 1,COMPNAME

|  106,.400,.400, 0, 1, 0, 8, 2,COMPDESC

|  106,-.100,.100, 1, 1, 0, 1, 0,Data In

------------------------------------------------------------------------------

III. Design/Page Notes global symbol

All coordinates are expressed as defined previously in

database header record.

Symbol definition consist of several records. Structure is

following:

Record type:   Number:      Contents:

160             1            symbol header

162             0 or more    single line primitive

163             0 or more    single rectangle primitive

164             0 or more    single circle primitive

165             0 or more    single arc primitive

166             0 or more    single text string

Symbols with reference number from -32001 to -32100 define

page notes. Symbol name=page name to which this notes are

attached.

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Design/Page |

Notes symbol| 160,-10001,@DES

header      |

| where:

|

|  160              record type

|

|  -10001           fixed reference number for design notes

|                   global symbol

|   or

|

|  -3000p           reference number for page p notes

|

|  @DES             fixed name for design notes global symbol

|                   or page name

|

------------------------------------------------------------------------------

Design Notes|

primitives  | 162               same format as 102 (line)

|

| 163               same format as 103 (rectangle)

|

| 164               same format as 104 (circle)

|

| 165               same format as 105 (arc)

|

| 166               same format as 106 (text)

|

|

| Example:

|

|  162,-.100,.100,.050,.100, 0, 0, 1, 0

|  163,-.100,.100,.050,.100, 0, 0, 1, 0

|  164,-.100,.100,.250, 0, 0, 1, 0

|  165,-.100,.100,.250, 45, 180, 0, 0, 1, 0

|  166,-.100,.100,0, 2, 0, 1,0,DESIGN NOTES

------------------------------------------------------------------------------

IV. Device Description Library.

Every device description consist of several records.

Structure is following:

Record type:   Number:      Contents:

200             1            device header

201             1 or more    Voltage&Gnd pinout

202             1 or more    Group header

203             1 or more    group pinout

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Device      |

header      | 200,devref,devname,prefix,laysmb,no.groups,status,laydesc

|

| where:

|  200              record type

|

|  devref           device reference number within database

|

|  devname          device name 1 to 8 characters

|

|  prefix           layout component name prefix 1 to 4 characters

|

|  laysymb          name of layout symbol to represent device  in

|                   layout database

|

|  no.goups         number of groups within device

|

|  status           XXXXXXXX - internal 8 bit mask (0 default)

|

|  laydesc          text string to appear with layout componet

|                   referencing this device

|                   "none" stands for "no text condition"

|

|  Example:

|

|   200, 1,7400    ,IC,R47     , 4, 0,none

------------------------------------------------------------------------------

Voltage     |

Ground      | 201,volt,pinnumber

pinout      |

| where:

|  201              record type

|

|  volt             voltage designator in form:

|          VCn      n from 1 to 9 for Vcc1 to Vcc9

|          GN0      for GND

|

|  pinnumber        pin number (layout symbol) where voltage/GND

|                   is connected

|

| Example:

|

|  201,VC1, 14

|  201,GN0, 7

------------------------------------------------------------------------------

Group       |

header      | 202,schsymb,simcode,rpower,rdelay,fpower,fdelay,nparams,schdesc

|

| where:

|  202              record type

|

|  schsymb          schematic symbol to represent this group in

|                   schematic data base

|

|  simcode          simulation model ID number

|

|  rpower           power level for logic high

|           S       Supply -         priority level 1

|           D       Drive  -         priority level 2

|           R       Resisitive -     priority level 3

|           Z       High impedence - priority level 4

|

|  rdelay           propagation Low->High in simulator time steps

|

|  fpower           power level for logic low

|           S       Supply -         priority level 1

|           D       Drive  -         priority level 2

|           R       Resisitive -     priority level 3

|           Z       High impedence - priority level 4

|

|  fdelay           propagation High->Low in simulator time steps

|

|  nparams          no. of parameters required to define simulation

|                   model

|

|  schdesc          text string to appear with schematics componet

|                   referencing this device

|                   "none" stands for "no text condition"

|

|  Example

|

|   202,G00     , 11,D, 1,D, 1, 0,none

------------------------------------------------------------------------------

Group       |

pinout      | 203,pinnumber,dir,simentry,vcode,swaplevel

|

| where:

|  203              record type

|

|  pinnumber        layout symbol pin number

|

|  dir              direction

|            I      input

|            O      output

|            T      tristate

|

|  simentry         logic entry number for simulation model

|

|  vcode            pin packaging no visibility on schematics

|            v      pin number visible

|            i      pin number invisible

|

|  swaplevel        all pins with the same number are ligically

|                   swapable

|                   -1 designates unswapable pin

|

| Example:

|

|  203, 3,O, 0,v,-1

|  203, 1,I, 0,v,1

|  203, 2,I, 1,v,1

------------------------------------------------------------------------------

V. Netlist

Every net description consist of several records.

Structure is following:

Record type:   Number:      Contents:

300             1            net header

301             1 or more    node

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Net         |

header      | 300,netref,netname,status,busref,busmemb

|

| where

|

|  300              record type

|

|  netref           net reference number within database

|

|  status           net 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      1 if SPL  otherwise 0

|           b3      1 if named  otherwise 0

|           b2      1 if bus othervise 0

|           b1      1 if in bus otherwise 0

|           b0      1 if active otherwise 0

|

|  busref           if net in bus then bus net refernce number

|

|  busmemb          if net in bus then bus member number

|

|  Example:

|

|   300, 1,SPL1    , 25, 0, 0

|   300, 2,UN1     , 73, 0, 0

------------------------------------------------------------------------------

Node record |

| 301,status,compref,entry

|

| where:

|

|  301              record type

|

|  status           node 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      X

|           b0      1 if physically connected otherwise 0

|

|  compref          component reference number

|

|  entry            entry number for node

|

|  Example:

|

|   301, 1, 2, 2

|   301, 1, 1, 13

------------------------------------------------------------------------------

VI. List of references schematic vs layout components

Stucture of layout component definition is following:

Record type:   Number:      Contents:

650             1            layout component definition

651             1 or more    packaging information

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Schematic   |

component   | 650,lcompref,lcompname,status,devref,group

|

| where:

|

|  650              record type

|

|  lcompref         layout component reference number within

|                   database

|

|  lcompname        layout component name 1 to 8 characters

|

|  status           8 bit status: XXXXXXXX

|

|  devref           reference no. for device used by component

|

|  group            no. of group within device used by component

|

| Example:

|

|  650, 1,IC1     , 65, 1, 4

|

------------------------------------------------------------------------------

Packaging   |

information | 651,group,scompref,0

|

| where:

|

|  651              record type

|

|  group            group sequence number (starting from 1)

|

|  scompref         schematics component reference no.

|

|  0                value 0

|

| Example:

|

|  651, 1, 2, 0

------------------------------------------------------------------------------

VII. Page information

Every page description consist of several records.

Structure is following:

Record type:   Number:      Contents:

400             1            page header

401             5 or more    format outline corners

Coordinates for format outline corners are expressed as defined

previously in database header record.

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Page header |

| 400,pageref,pagename,status,Xzoom,Yzoom,scal,Xmin,Ymin,Xmax,Ymax

|

| where:

|

|  400              record type

|

|  pageref          page reference number within data base

|

|  pagename         1 to 8 characters of page name

|                   MAINPAGE for page no. 1

|

|  status           page status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      1 if page active otherwise 0

|           b0      1

|

|  Xzoom,Yzoom      Coordinates of last ZOOM center point

|

|  scal             Scale factor in last ZOOM

|            1      scale  5:1

|            2      scale  2:1

|            5      scale  1:1

|           10      scale  1:2

|           20      scale  1:5

|           50      scale  1:10

|          100      scale  1:20

|

|  Xmin,Ymin        Coordinates of Xmin,Ymin of board outline

|

|  Xmax,Ymax        Coordinates of Xmax,Ymax of board outline

|

| Example:

|

|  400, 1,MAINPAGE, 3,.000,.000, 10,-2.500,-2.500,2.500,2.500

------------------------------------------------------------------------------

Format      |

outline     | 401,xcorn,ycorn

corner      |

| where:

|

|  401              record type

|

|  xcorn,ycorn      Outline corner X/Y coordinates

|

| Example:

|

|  401,-2.500,-2.500

|  401,-2.500,2.500

|  401,2.500,2.500

|  401,2.500,-2.500

|  401,-2.500,-2.500

------------------------------------------------------------------------------

VIII. List of junction points

Coordinates for junction points are expressed as defined

previously in database header record.

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Junction    |

point       | 500,jpref,pageref,status,laymask,xjp,yjp

|

| where:

|

|  500              record type

|

|  jpref            junction point reference number within

|                   data base

|

|  pageref          page reference number to which junction point

|                   is attached

|

|  status           junction point 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      X

|           b0      1

|

|  laymask          16 bit mask = 7FFEhex

|

|  xjp,yjp          junction point coordinates

|

| Example:

|

|  500, 1, 1, 1, 32766,-2.200,-1.500

|

------------------------------------------------------------------------------

IX. List of components

Coordinates for component placement are expressed as defined

previously in database header record.

Stucture of layout component definition is following:

Record type:   Number:      Contents:

600             1            component definition

606             0 or more    pin assigment text strings

or component text

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Component   |

| 600,compref,cmpname,pageref,status,symbref,devref,xcomp,ycomp,

|        cscale,crot,cmirror,cmask,lcompref,lgroup,igroup,0,0

|

| where:

|

|  600              record type

|

|  compref          component reference number within database

|

|  compname         component name 1 to 8 characters

|

|  pageref          reference number for page to which copmponent is

|                   attached

|

|  status          component 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      0 - unlocked   1 - locked

|           b2      0 - name invisible   1 - name visible

|           b1      1 - display combined symbol

|                   0 - no combined symbol displayed

|                   (combined symbol is secondary symbol with

|                    reference no. = -compref)

|                   combined symbol is used in schematics data base

|                   to store component text strings and pin assignemnt

|                   text

|           b0      1

|

|  symbref          reference no. of symbol used to display

|                   component

|

|  devref           reference no. of component device

|

|  xcomp,ycomp      coordinates for component entry 1

|                   (component placement coordinates)

|

|  cscale           component display scale = 4 ( 1:1)

|

|  crot             component rotation crot*90 deg

|

|  cmirror          0 - unmirrored display

|                   1 - mirrored display

|

|  cmask            reference symbol primitives display mask

|

|  lcompref         layout component reference number

|

|  lgroup           group number within layout component

|

|  igroup           initial group number within device

|

|  0                value 0

|

|  0                value 0

|

| Example:

|

|  600,1,C1,1,3,1,1,-2.300,-2.300,4,0,0,7,7,4,1,0,0

------------------------------------------------------------------------------

Component   |

text/       | 606,xtxt,ytxt,rot,sizecode,color,mask,link,textstring

component   |

name text/  | where:

pinout      |

text        |  606              record type

|

|  xtxt,ytxt        first character in string coordinates

|

|  rot              text rotation rot*90 deg

|

|  sizecode         text size code

|

|  color            text color

|           0       color of component outlines

|           1       color of component name

|           2       color of component description

|           3       color of pinout

|           4       color of component text

|           5       color of bus

|           6       color of bus name

|           7       color of bus memeber

|           8       color of wire

|           9       color of net name

|          10       color of net label

|          11       color of free text

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         1 for pinout text otherwise 0

|        b2         1 for component description otherwise 0

|        b1         1 for layout component name otherwise 0

|        b0         1 for component text/component value

|                     otherwise 0

|

|  link             number of schematics componet entry in

|                   case of pinout text

|                   1 for componet value

|

|  textstring       text string

|

| Example:

|

|  606,-.150,-.100, 0, 1, 0, 2, 0,T1

|  606,-.150,.100, 0, 1, 3, 8, 1,2

|  606,.250,.100, 0, 1, 3, 8, 2,1

|  606,.250,.-.100, 0, 1, 3, 8, 3, 3

|  606,.850,.050, 0, 1, 4, 1, 0,COMPONENT TEXT

------------------------------------------------------------------------------

X. List of wires and buses

Coordinates for wire/bus bending points are expressed as

defined previously in database header record.

Structure of trace description is following:

Record type:   Number:      Contents:

700             1            wire/bus  header

701             2 or more    wire/bus segments

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Wire/bus    |

header      | 700,wreref,netref,pageref,status

|

| where:

|

|  700              record type

|

|  wreref           wire/bus reference number within database

|

|  netref           net reference number to which wire/bus belongs

|

|  pageref          page reference number to which wire/bus is

|                   attached

|

|  status           trace 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      1 - bus oterwise 0

|           b0      1

|

| Example:

|

|  700, 1, 1, 1, 193

------------------------------------------------------------------------------

Trace       |

segment     | a. 701,status,side,size,compref,entry

|   or

| b. 701,status,side,size,jpref

|   or

| c. 701,status,side,size,xbpoint,xbpoint

|

| where:

|  700              record type

|

|  status           seqment end 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      1 component otherwise   0

|           b1      1 junction point   otherwise 0

|           b0      1 bending point absolute otherwise 0

|

|  side             5 for bus

|                   8 for wire

|

|  size             1 for wire

|                   15 for bus

|

| a. variant (b2b1b0 in status = 100)

|

|  compref          component reference no. to which segment

|                   is connected

|

|  entry            entry number within component to which segement

|                   is connected

|

| b. variant (b2b1b0 in status = 010)

|

|  jpref            junction point reference no. to which segment

|                   is connected

|

| c. variant (b2b1b0 in status = 001)

|

|  xbpoint,ybpoint  bending point coordinates

------------------------------------------------------------------------------

XI. Text strings

Coordinates for text strings are expressed as defined

previously in database header record.

Structure of trace description is following:

Record type:   Number:      Contents:

800             0 or more    text string

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Text string |

| 800,link,pageref,status,xtxt,ytxt,color,size,rot,textstring

|

| where:

|

|  800              record type

|

|  link             in case of net associated text this fiels

|                   contains net reference number

|

|  pageref          page reference number to which text is

|                   attached

|

|  status           text 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      1 volotage  otherwise 0 (analog sim.)

|           b2      1 net label otherwise 0 (analog sim.)

|           b1      1 net name otherwise 0

|           b0      1

|

|  xtxt,ytxt        coordinates of first character in text string

|

|  color            text color

|           0       color of component outlines

|           1       color of component name

|           2       color of component description

|           3       color of pinout

|           4       color of component text

|           5       color of bus

|           6       color of bus name

|           7       color of bus memeber

|           8       color of wire

|           9       color of net name

|          10       color of net label

|          11       color of free text

|

|  size             text size code

|

|  rot              text rotation rot*90

|

|  textstring       text string

|

| Example:

|

|  800, 0, 1, 1,-1.400,.300, 0, 1, 0,Counter

------------------------------------------------------------------------------

XII. End of File record

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

End of File |

| 999

|

| where:

|

|  999              record type

------------------------------------------------------------------------------