Programming the Sound Blaster ADC/DAC:

2x6h    DSP Reset Port                       Write Only

2xAh    DSP Read Data Port                   Read Only

2xCh    DSP Write Data or Command            Write

2xCh    DSP Write Buffer Status (bit 7)      Read

2xEh    DSP Data Available Status Bit 7)     Read Only

x = 1,2,3,4,5,6 for the Sound Blaster <= 1.5

x = 1,2,3,4,5,6 for the Sound Blaster Micro Channel Version

x = 2,4 for the Sound Blaster 2.0

x = 2,4 for the Sound Blaster Pro

The DSP: -----------------------------------------------------------------------

Due to the different mode and the hardware performance of different audio

cards, the maximum sampling rate is different.

Input (ADC):

Sound Blaster (<=1.5)   Mono/Normal mode:       8-bit only  4KHz to 13KHz

Sound Blaster (2.0)     Mono/Normal mode:       8-bit only  4KHz to 13KHz

Mono/High Speed mode:   8-bit only  13KHz to 15KHz

Sound Blaster Pro       Mono/Normal mode:       8-bit only  4KHz to 23KHz

Mono/High Speed mode:   8-bit only  23KHz to 44.1KHz

Output (DAC):

Sound Blaster (<=1.5)   Mono/Normal mode:       8-bit only  4KHz to 23KHz

Sound Blaster (2.0)     Mono/Normal mode:       8-bit only  4KHz to 23KHz

Mono/High Speed mode:   8-bit only  23KHz to 44.1KHz

Sound Blaster Pro       Mono/Normal mode:       8-bit only  4KHz to 23KHz

Mono/High Speed mode:   8-bit only  23KHz to 44.1KHz

Some DSP Commands:

10h  Direct mode 8-bit DAC (single byte data transfer)

14h  DMA mode 8-bit DAC

20h  Direct mode 8-bit ADC (single byte data transfer)

24h  DMA mode 8-bit ADC

40h  Set Time Constant

48h  Set Block Size

91h  High Speed DMA mode 8-bit DAC

99h  High Speed DMA mode 8-bit ADC

D1h  Turn on Speaker

D3h  Turn off Speaker

D0h  Halt DMA in progress

D4h  Continue DMA

E1h  Get DSP version (read major ver then minor)

To reset the DSP:

1.  Write a 01h to port 2x6h

2.  Wait for 3 microseconds

3.  Write a 00h to port 2x6h

4.  Read port 2xAh until a 0AAh is read (see below for how to read from 2xAh)

If there is no 0AAh after about 100 reads, abort and declare that there is no

Sound Blaster present (or error)

To write to the DSP (all writes to 2xCh MUST follow this procedure)

1.  Read 2xCh until bit 7 is clear

2.  Write to 2xCh

To read from the DSP (all reads from 2xAh MUST follow this procedure)

1.  Read 2xEh until bit 7 is set

2.  Read from 2xAh

Interrupts:

In DMA DAC and DMA ADC modes, a single interrupt will occur after the

block of data has been read/written.  To clear the interrupt, read 2xEh

once (as well as clearing the PIC).

Ignoring Interrupts:

The interrupt can be ignored if you poll the DMAC (DMA Controller).

Once the DMAC reports a count of 0FFFFh the transfer is finished, read

2xEh once and you are finished.  You can also poll the DMA status register

and wait for the Terminal Count reached bit to be set (I found that the first

read after starting the transfer had the bit set, but not the second).

Note to VESA/Local Bus Video users:

These video cards use DMA channel 1 which is the DMA channel used by

the Sound Blaster.  Any video accesses will screw up the output of the

Sound Blaster.

Calculating the Time Constant:

Normal Speed:

Time Constant = 256 - (1,000,000 / sampling rate)

= 256 - (1,000,000 / 8,000 )

= 131

High Speed:

Time Constant = (MSByte of) 65536 - (256,000,000 / sampling rate)

= (MSByte of) 65536 - (256,000,000 / 44,100)

= (MSByte of) 59731

= (MSByte of) 0E953h

= 0E9h

Direct mode DAC:

1.  Write a D1h to 2xCh

2.  Write a 10h to 2xCh

3.  Write the 8-bit data sample to 2xCh

4.  Wait for the correct timing (must do your own timing)

Repeat steps 2-4 until end of data

5.  Write a D3h to 2xCh

Normal speed DMA mode DAC:

1.  Write a D1h to 2xCh

2.  Setup Interrupt service routine

3.  Write a 40h to 2xCh

4.  Write Time Constant to 2xCh

5.  Program the DMAC (DMA Controller)

6.  Write 14h to 2xCh

7.  Write the LSByte of Data Length - 1

8.  Write the MSByte of Data Length - 1

9.  Service Interrupt (may need to repeat steps 5-7 in the ISR)

10. Restore original Interrupt Service Routine

11. Write a D3h to 2xCh

Commands can be written to the DSP while waiting for the interrupt

High speed DMA mode DAC:

1.  Write a D1h to 2xCh

2.  Setup Interrupt service routine

3.  Write a 40h to 2xCh

4.  Write Time Constant to 2xCh

5.  Program the DMAC (DMA Controller)

6.  Write 48h to 2xCh

7.  Write the LSByte of Data Length - 1

8.  Write the MSByte of Data Length - 1

9.  Write 91h to 2xCh

10. Service Interrupt (may need to repeat steps 5-7 in the ISR)

11. Restore original Interrupt Service Routine

12. Write a D3h to 2xCh

Commands CANNOT be written to the DSP while waiting for the interrupt

Resetting the DSP is the procedure used to halt DMA in progress

Direct mode ADC:

1.  Write a 20h to 2xCh

2.  Read the 8-bit data sample from 2xAh

3.  Wait for the correct timing (must do your own timing)

Repeat steps 1-3 until finished

Normal speed DMA mode ADC:

1.  Setup Interrupt service routine

2.  Write a 40h to 2xCh

3.  Write Time Constant to 2xCh

4.  Program the DMAC (DMA Controller)

5.  Write 24h to 2xCh

6.  Write the LSByte of Data Length - 1

7.  Write the MSByte of Data Length - 1

8.  Service Interrupt (may need to repeat steps 5-7 in the ISR)

9.  Restore original Interrupt Service Routine

Commands can be written to the DSP while waiting for the interrupt

High speed DMA mode ADC:

1.  Setup Interrupt service routine

2.  Write a 40h to 2xCh

3.  Write Time Constant to 2xCh

4.  Program the DMAC (DMA Controller)

5.  Write 48h to 2xCh

6.  Write the LSByte of Data Length - 1

7.  Write the MSByte of Data Length - 1

8.  Write 99h to 2xCh

9.  Service Interrupt (may need to repeat steps 5-7 in the ISR)

10. Restore original Interrupt Service Routine

Commands CANNOT be written to the DSP while waiting for the interrupt

Resetting the DSP is the procedure used to halt DMA in progress

The Mixer:  --------------------------------------------------------------------

The mixer can only be found on the Sound Blaster Pro.

It mixes the following audio sources:

-digitized voice

-FM synthesized music

-CD-audio

-line-in

-microphone input

-PC speaker output

It allows software to control volume on:

-digitized voice

-FM synthesized music

-CD-audio

-line-in

-microphone mixing

-overall master volume

The mixer setting can be done with 2 I/O ports, 2x4h and 2x5h.

x=2,4 for the Sound Blaster Pro

2x4h is the address port (write only), 2x5h is the data port (read/write)

The programming sequence is as follows:

1.  Write the address of the mixers register to 2x4h

2.  Read/Write the mixers register value from/to 2x5h

Mixer registers:

Register| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |

--------+----+----+----+----+----+----+----+----+

00h     |               Data Reset              |

02h     |                Reserved               |

04h     |   Voice Volume L  |   Voice Volume R  |

06h     |                Reserved               |

08h     |                Reserved               |

0Ah     | x  | x  | x  | x  | x  |  MIC Mixing  |

0Ch     | x  | x  |  In Filter   |   ADC   | x  |

0Eh     | x  | x  |DNFI| x  | x  | x  |VSTC| x  |

20h     |                Reserved               |

22h     |  Master Volume L  |  Master Volume R  |

24h     |                Reserved               |

26h     | FM Volume L       | FM Volume R       |

28h     | CD Volume L       | CD Volume R       |

2Ah     |                Reserved               |

2Ch     |                Reserved               |

2Eh     |   Line Volume L   |   Line Volume R   |

x=don't care

Reserved=preserve original value

Register Descriptions:

Reset Register (00h):

You can write any 8-bit value to this register to reset the mixer

Voice Volume Register (04h):

d[7..4] voice volume left

d[3..0] voice volume right

The default level is 9.

Microphone Mixing Register (0Ah):

The default level is 0.

Input Setting Register (0Ch):

ADC - input source:

d[2] d[1]

0    0    Microphone (default)

0    1    CD audio

1    0    Microphone

1    1    Line-in

In Filter - filter select:

d[5] d[4] d[3]

0    x    0   Low Filter (default)

0    x    1   High Filter

1    x    x   No Filter

Output Setting Register (0Eh):

This register specifies the voice output features.

DNFI  0 output filter is ON (default)

1 output filter is bypassed

VSTC  0 Mono voice mode (default)

1 stereo voice mode

Master Volume Register (22h):

d[7..4]  master volume left

d[3..0]  master volume right

The default level is 9.

FM Volume Register (26h):

d[7..4] fm volume left

d[3..0] fm volume right

The default level is 9.

CD Volume Register (28h):

d[7..4] cd volume left

d[3..0] cd volume right

The default level is 0

Line-In Volume Register (2Eh):

d[7..4] line-in volume left

d[3..0] line-in volume right

The default level is 0