TERMSERV 1.1 - an example terminal server

Copyright (c) 1990, 1999 Erick Engelke

TERMSERV.EXE (in binary form) is distributed freely in the hope that it

may prove interesting or useful, but with no warrantee of any kind.

Purpose

=======

Termserv allows incoming TELNET sessions direct access to serial

ports in the local machine.

So one might hook up four modems and use this to TELNET to the modem

pool.  Or one may connect to the serial line of a network switch

and use this program to access the switch's console.

This program also includes an FTP server and a Web server, including

real-time generation of a usage graph.

Finally, this program will optionally Email to a specified Email address

each time it is restarted, and SYSLOG entries on restart, and as each

connection arrives and later departs.

Configuration

=============

Edit the TCP.CFG file and set the following parameters:

[settings]

ip=x.x.x.x              dotted IP address of this station, or BOOTP

netmask=255.255.255.0   or whatever is the appropriate network mask

gateway=x.x.x.x         dotted IP address of the router

nameserver=x.x.x.x      dotted IP address of the name server

smtp.gateway=x.x.x.x    dotted IP address of an Email relay host

smtp.hostname=blah.blah DNS name for this machine, required by

many SMTP servers, must be set correctly

ftpd.password = "frog"  any case sensitive password for your FTP service

email.notify = "erick@ertos.com"  the Email address of recipient

syslog.host = x.x.x.x   option host to receive our syslog information

userid=joe              userid/password for web access

password=blow

[com1]                    repeat this section for com1 through com4

speed=9600

irq=4

Programming

===========

This is a multithreaded program using the commercial eRTOS development

system.  eRTOS is great for developing embedded network and/or serial

applications - this program demonostrates both network and serial i/o

and is actually one of the example projects.

This entire program took less than three hours to write.  The non-web

part is two printed pages of C, and the web part (including graphics)

is another two pages.  eRTOS includes code to do graphics, and charts.

For more information about eRTOS, see http://www.ertos.com