The 132 column VGA display device driver is similar to the standard

VGA driver for presentation manager except that it drives the display screen

to a resolution of 664x480 instead of the standard 640x480. The extra

horizontal pixels allow for displaying 132 columns of text in 3270 terminal

emulation sessions or any other Presentation Manager AVIO applications. Each

character is 5 pels wide so most suitable for occasional use. Users who

require 132 columns all the time would be better of using XGA or 8514/A.

Installation instructions:

You should rename the VGA132.DLL file to DISPLAY.DLL and place it in

a temporary directory, the CONFIG.SYS file is then edited to place this

temporary directory AT THE FRONT of the LIPBATH. The machine is then rebooted.

After reboot the user can copy the new display.dll into the OS2/DLL directory

and remove the temporary directory from the LIPBATH statement.