CURVE FITTER

version 2.1

by

David C. Young

For use with MSDOS computers.

Copyright 1991, 1992 David C. Young

PROGRAM FUNCTION:

This program finds both coefficients and exponents for a curve of best

fit by the least squares definition of best fit.

The program contains functions for creating and revising it's data

input files.  There is also a function titled "Display any ASCII file", which

can be used to display the output files.

Each data field is read from a data file.  Each data point in the file

is accompanied by a key piece of information, which uniquely identifies that

data point.

The program will read in any number of data files and only use those

data points containing information for every field.

The program works with or without a math coprocessor and takes

advantage of the coprocessor if it is present.

STARTING THE PROGRAM:

Start the program by changing to the directory or drive containing the

program and typing CURVE.

The program can be installed on a hard drive by using the DOS copy

command to copy all of the files to the desired location on the hard drive.

USING THE PROGRAM:

The basic process of using this program is:

1.  Create data files.  The data in the data files is keyed.

An example of using keys would be if you wanted to find an equation to

predict the price of your favorite stock, you might want to enter various

items that could be used to predict, such as Gross National Product, or

the price of tea in China.  The price of your stock would also be in a file

keyed by month.  You would key these values by the date that they are for.

Thus you would have a file with the price of tea in China during various

months.  The month would be the key value.  Once you had several files, all

keyed by month, you might guess that the price of your stock varies in the

same way as the GNP times the price of tea in China.

If you don't have all of the necessary data (due to irregular

delivery of the Bejing Times), the program will still work.  It will just

only take into account those months, for which you have a complete set of

data, without bothering you with the details.

2.  Type in an equation.  For the example above, the equation

might be S=A*G*T+B or S=A*S^C+B*T^D+E where S is the stock price, G is

the GNP, T is the price of tea in China and A through E are numbers that

you want the computer to calculate.

3.  Specify which letters represent known data points to be

read from data files.  S, G & T in our example are read from files.

C & D are computed exponents.  A, B & E are computed coeficients.

4.  Input file names for summary and analysis files.

A summary file contains the stuff put on screen at the end of the calculation,

the values for computed numbers and the average & maximum deviations (measures

of how well the equation fits the data).  An analysis file contains the

key values, actual values (for your stock) and computed values.  Analysis

files can be read into many popular spread sheets and graphing programs,

so that you can graph the data to better see how well the calculation

works.

5.  For computed exponents, input starting values and the

number of decimal places to calculate.

The equation to be input consists of character strings to

represent both known pieces of data and numbers to be calculated,

along with an equals sign "=" and any valid operators or function:

Valid operators are:

+      : addition             *         : multiplication

/      : division             ^         : exponentiation

General functions are:

ln()   : natural log          log()     : common log

sqr()  : square               sqrt()    : square root

int()  : integer part         frac()    : fractional part

abs()  : absolute value       exp()     : exponent of e

Radian trigonometric functions are:

sin()  : sine                 arcsin()  : inverse sine

cos()  : cosine               arccos()  : inverse cosine

tan()  : tangent              arctan()  : inverse tangent

Degree trigonometric functions are:

dsin() : sine                 darcsin() : inverse sine

dcos() : cosine               darccos() : inverse cosine

dtan() : tangent              darctan() : inverse tangent

Although subtraction is not a valid operation, constants can be

negative.

The equation may also contain sets of "()", "[]" or "{}".  These

are interpreted as controlling the order of operations, but do not

imply multiplication.

Examples of valid equations:

a = b * c + d                   a * b ^ c + d = e

a = b * ( c + -1*d )            y = slope * x + intercept

y = s * ( 1.234 + x )^e + i     y = c * sin(x)^e + n

A=B*C^D+E*F^G+H*I^J+K*L^M+N

SAMPLE SESSION:

This section gives an example of the steps to actually do the curve

fit, once the input files have been created.

The scenario is that two input files named "X.DAT" and "Y.DAT" have

been created and you want to find the values of c, d and n for the equation

y = c*x^n + d

Here y is to be taken from the file Y.DAT and x is to be taken from the

file X.DAT.  The variables c and d are coeficients for the program to

compute and the variable n is an exponent to be computed.  For the exponent

n, you expect the best value to be slightly over 1 so you would like to

give it the initial value of 1.2 .

The text in double quotes "" is what is actually typed.

From the directory containing the input files, type "CURVE" to

start the program.

From the Main Menu, type "2", cooresponding to the menu pick

'Fit curves.'.

The program will give a prompt asking you to enter a valid equation.

Type

"y = c*x^n + d"

Now a prompt asks

'Is "y" a known value to be input from a data file (y/n)?'

Type "y" then it asks for a file name and you type "Y.DAT".

This same prompt is repeated for the variable c and you repsond

by typing "n".  For the variable x, type "y" and specify the file name

"X.DAT".  For the variable n and d, also respond by typing "n".

The program now asks if you want a summary file and an analysis

file.  If you respond "y", it will then prompt you for a file name.

Next, you should get a message saying

'Enter an initial value for exponent "n".

The value can be any valid non-zero double number.'

You respond by typing your guess of "1.2" .  There is no prompt for

the variables c and d because the optimal coeficients are computed

by the least squares method, where as the exponents are varied to

minimize the average deviation between the left and right sides

of the equation.

The program now prompts

'To how many decimal places do you want calculated values (1-38)?'

You respond by typing "10".  The program will vary exponents to find the

optimal exponents until the variations are less than ten to the power -10

or the limit of the machine precision, (about 15 significant figures)

which ever is satisfied first.

The program will display the values of the coeficients and

exponents as they are computed.

Once the computation is completed, there should be a screen

starting with the line

'Curvefit complete for the equation:'

This screen is nearly identical to the summary file.

Finally, you should get a prompt asking if you want to fit

another equation.

DATA FILE FORMAT:

The data file is an ASCII file consisting of a header and up to 65000

key and data values.

The numbers identifying what type of key and data values are present

are as follows:

1 - string

2 - real

3 - character

4 - integer

The key field can be of any of these types.  The data field can be

used for curve fitting only if it is of type real or integer.  Integer data

fields are treated by curve fitter as real values.

The data file format is:

<number of data points (integer)>

<file description (string)>

<type of key field (string)>

<type of key field (integer)>

<type of data field (string)>

<type of data field (integer)>

<key value (first)> <data value (first)>

<key value (second)> <data value (second)>

.

.

.

<key value (last)> <data value (last)>

<end of file>

SUMMARY FILE FORMAT:

The summary file is an ASCII file, which contains exactly the same

information, which is displayed on the screen at the end of a calculation.

The summary file format is:

<equation (string)>

Average Deviation = <average deviation (real)>

Maximum Deviation = <maximum deviation (real)>

Known data points

Known <letter in equation> taken from <file name>

(same as line above for each data file used)

Computed Exponents (if any)

Exponent <letter in equation> = <value (real)>

(same as line above for each exponent computed)

Computed Coefficients

Coefficients <letter in equation> = <value (real)>

(same as line above for each coefficient computed)

<end of file>

ANALYSIS FILE FORMAT:

The analysis file is an ASCII file containing information for

comparing the known values to the calculated values for the property being

modeled.

The analysis file format is:

<number of data points (integer).

<equation (string)>

<type of key field (string)>

<type of key field (integer - same as data file)>

<type of data field (string)>

<type of data field (integer - same as data file)>

<key value (first)> <known data value> <calculated data value>

<key value (second)> <known data value> <calculated data value>

.

.

.

<key value (last)> <known data value> <calculated data value>

<end of file>

LIMITATIONS OF THE PROGRAM:

Some known deficiencies with the program that I hope to get around to

fixing in the future are.

1.  There is no global search routine incorporated in this

version.

2.  There is no perturbational routine to aid in finding

minimums in complex spaces.

3.  Many functions are not supported.

4.  The exponents that are found represent local minimums

only, so pick your initial values wisely, or try a few that you think might

be in the right range.

5.  Only exponents can be optimized.  Thus a variable that

is not an exponent or a coeficient can not be computed.

6.  There are no descriptive statistics available to

analyze your results.

7.  It is up to the user to determine the form of the

equation to use.  The computer can not suggest a reasonable form.

8.  There must be at least one linear coefficient being

fitted.

However, to the best of my knowledge, for what this program does do,

it does it correctly.

ABOUT CURVE FITTING:

The program generally finds the coefficients of best fit for each term

in an equation and finds the exponent of best fit for any variable desired.

The exponents are gotten through a multivarible simplex minimization routine.

The coefficients are gotten at every step of the way through the matrix

algebra least squares method (mathematically equivalent to linear regression).

If your theory shows that an equation should have a particular form,

it is best to work with that form.  However, if you don't know what form to

use and want to fit your data by a brute force method, here are some

suggestions:

1.  Have the program find a coefficient for every term in the

equation.

2.  Have the program find the constant term.

3.  The generic most powerful fitting is one in which every

term consists of a fitted coefficient and a single variable with a fitted

exponent .. and then the constant term is added on.  This is often the best

fit because the most parameters are being fitted.

4.  Remember that you can fit anything if you are fitting more

parameters than there are items in your data set, however this fit may be

useless when applied to new data points falling between or past the original

data points.  For best results always make sure that you have considerably

more data points than the number of parameters that are being fitted,

otherwise you may be fooling yourself.

LEGAL STUFF:

Version 2.1 of this program is being offered FREE to the world with no

guarantees expressed or implied ... etc, etc, etc.  Version 2.1 may be freely

distributed to anyone and everyone, as long as these instructions are

kept with it.

SALES PITCH:

I have completed version 3.1 of this program.  The new features

present in version 3.1 are:

1.  Nonlinear parameters other than exponents can be

fitted.

2.  It does global searches for nonlinear parameters

using grid, Monte Carlo or anealing searchs.

If you would like to buy version 3.1, send $15.00 to:

David Young

4485 Fairlane

Okemos, MI  48864

Please, specify what size and density of disk for me to send.  I will send

you one disk containing version 3.1 (or whatever the most recent version is)

containing the program and documentation files.  This price does not include

any updates past the version that I send you, but I will try to keep you

informed of future versions.  Please, send cash, check or money orders.  I

cannot accept credit card orders.  Version 3.1 is sold as is with no

guarantees expressed or implied.

If you have any questions, you can also reach me by e-mail at:

internet:  young@slater.cem.msu.edu