USRobotics Courier HST Qmodem Service Script Production Release 2.0

-------------------------------------------------------------------

(c) Copyright Dan Moore, 1987-1989

The USRobotics logo, Courier, and HST are trademarks of

USRobotics, Inc.

QModem and QModem SST are trademark of John Friel III and the

Forbin Project, Inc.

USE RESTRICTION AND DISCLAIMER

This script is copyrighted solely to prevent others from reaping

financial gain through its sale and/or use.  Permission has been

given to John Friel III and the Forbin Project, Inc., to

distribute this script as part of the QModem SST program

package, release 4.0 for private use only.

The HST20 script and its accompanying files and documentation

may not be distributed as part of any other computer

communication hardware or software package without the written

permission of the author.  This script may be copied and

distributed freely by private individuals who wish to share it

with others as long as no specific charge is assessed for this

service other than minimal fees (not to exceed $5) to cover the

distribution medium.

Permission is hereby granted to System Operators of active

Bulletin Boards to post the HST20 archive file for public

distribution.  Due to the easily modifiable nature of QModem

scripts,  modified versions of the script should bear a

different name and have their origin clearly identified.  The

NAME of the script is copyrighted as well as its contents. The

HST20.ARC file or other compressed versions of the HST20.ARC

original may not be modified for any purpose whatsoever.

COMMERCIAL USES OF THE HST SCRIPT

The series of HSTxx Qmodem Scripts may not be used for

marketing, demonstrations, advertising, problem diagnosis or

correction OR FOR ANY COMMERCIAL OR BUSINESS PURPOSES by modem

manufacturers, their agents, dealers, resellers, or any company

or individual involved in the sale, marketing, maintenance, or

business use of modems or other computing devices WITHOUT

A COMMERCIAL LICENSE TO USE THE SCRIPT.  Superficial editing of

the script's code to mask its origin and avoid licensing

costs is a violation of the author's copyright, and will be

pursued to the full extent of the law.

To obtain a commercial- or business-use license for the HST

script(s), refer to the LICENSE.DOC file in the HST20 archive.

HST MODEM DEPENDENCY

The HST20 script will operate satisfactorily on USRobotics HST

modems with ROM levels 961, 963, and 964.  To determine the

HST's ROM level, run the HST20 script or type ATI0<Enter> while

in Qmodem terminal mode.

If your HST modem is NOT at level 964, I strongly recommend that

you get it upgraded.  The cost if extremely reasonable and the

upgrade is performed quickly by USR.  You will gain MNP levels 4

and 5.  Compared to ROM level 963/961, release 964 will gain you

an additional 100cps for Ymodem/G transfers of binary or

compressed files, and will outright double your effective

transfer speed for uncompressed text files when MNP 5

compression is used.   For details on the HST upgrade program,

call USR Technical Support at 800-982-5151 (in Illinois, call

312-982-5151).

QMODEM RELEASE DEPENDENCY

The HST20 script will operate with Qmodem 4.0 and later releases

of Qmodem.  This script relies on certain features of the Qmodem

release 4.0 Script Language and it WILL NOT OPERATE under pre-4.0

releases of Qmodem.

OPERATIONAL SUMMARY

HST20.SCR is intended as a configuration and setup help for the

USR HST modem.  Although it facilitates much of the modem's

setup tasks, some knowledge of the HST's operation and

technology is required, as well as a basic understanding of

computer communications.

If you are using HST20 to set up your USR Courier HST for use

with  Qmodem 4.0, the QM menu options will perform all firmware

configuration, leaving physical switch changes up to the you

(correct switch settings are displayed as part of the

script's operation).

HST20 automatically detects the ROM release level of your modem.

If your USR HST modem is at ROM level 964 (the latest ROM

level at the time of this release), HST20 will allow you

access to a second level of menu via the "AD" main menu option.

This second menu accesses HST features only available in

964-level modems, such as data compression (MNP level 5),

settings for the new S-register 15, 4800/7200 link rate

settings, and the new modem configuration screen.  Advanced

Functions are NOT supported for ROM releases prior to 964.

SETTING UP THE HST FOR QMODEM OPERATION

It is likely that you will have to change the modem's switches

from their default factory settings to the setup recommended in

the menu's QM option.  Unfortunately, HST20 can't quite "reach

out and touch switches"; you must do this yourself.  The script

makes you aware of this requirement.

Another very important part of the modem setup is the

configuration of QModem itself.  With release 4.0 of Qmodem,

this is done with the Alt-N command (with release 3.1A of

Qmodem, setup was performed with the QINSTALL Program).

The QMSETUP.DOC file, included in this ARChive, will help

help you select the proper QModem configuration parameters for

the HST modem.

I recommend that you keep your HST manual handy while you

experiment with the script.  Bug reports or suggestions for

changes or improvements should be reported to me on the Qmodem

Conference Echo carried by the InterLink and SmartNet BBS Mail

Networks.

FRIENDLY WARNING

HST20 can make permanent changes to the modem's internal

configuration.  Once a setting has been changed, writing the

change to Non-Volatile RAM (NRAM) implements the change until

that setting is changed and written to NRAM again.  Changes to

NRAM are maintained even when the modem power is OFF.  If you

have a working set of modem settings and wish to experiment, do

not copy the changes to NRAM.  The next hardware reset or

power-off / power-on sequence will re-establish the last NRAM

settings.  The HSTINTRO.DOC file describing the HST's basic

internal operation is included in this ARChive; you may wish to

review it to gain a better understanding of the modem's

operation.

MODEM COMMAND FILES

New in release 2.0 of the HST script is the support for special

"Modem Command Files", which can be read from disk and sent

directly to your modem.  Modem Command Files are not intended

for the HST novice.  Until you become familiar with the various

modem commands and parameters, as documented in the HST User's

Manual, you may wish to leave this feature alone.

INVOKING A COMMAND FILE

Modem command files may be invoked from the standard HST20 menu

with the CF option.  You will be prompted for the command file

name ONLY.  Once the command file is executed, the standard

HST20 menu will be re-displayed.

If you are using HST20 solely to execute a command file, you can

specify the command file name in response to the "Script

Parameters" prompt.  When this method is used, HST20 terminates

after the last command has been executed.  This feature allows

the use of HST20 as an autoexec script.

CREATING COMMAND FILES

Modem command files are created with any standard ASCII text

editor or word processing program with the ability to save

standard, unformatted ASCII files (each line in the file must

end with the standard carriage return/line feed characters).

Modem command files (.HSF) must be stored in your Qmodem script

directory, as defined in the Alt-N/D/S setup.  Any filename is

valid, but the .HSF file extension MUST be used.

Each line in a .HSF file may contain a VALID HST modem command,

or one of the following HST20-only commands, starting in the

first column:

@PROMPT  HST20 will pause and issue a "Press <Enter> to

continue..." prompt.

@CLRSCR  Clear the screen before issuing the next modem

command.

@CMD     Issue the script command in the next line.  See

the section on AUTOEXEC and HST20 programming

for additional details on this powerful

and potentially dangerous command.

The @PROMPT suspends HST20 operation and is used when lengthy

modem responses are displayed, to prevent the modem's output

from scrolling off the screen when the following command is

executed.

The @CLRSCR command is purely cosmetic; its only effect is to

cause the next command to be displayed at the top of the screen.

Commands sent to the HST modem may not exceed 40 characters in

length, not including the AT prefix and imbedded spaces.  Make

sure that lines in command files abide by this restriction.

This restriction is imposed by the modem, not the script

language or Qmodem.

COMMAND FILE PROCESSING

HST20 will send each line in the command file to the modem and

echo it to the screen.  The standard "OK" return code is expected

from the modem in response to each command.  If this return code

is not received within 5 seconds, the script will force a timeout

and will terminate with an appropriate error message.

Following is a sample 4-line command file: (Note: this file  is

included in the HST20 archive under the name TEST20.HSF)

***start of file***

AT&M4&K0

@PROMPT

ATI5

@PROMPT

@CLRSCR

ATI4

@PROMPT

***end of file***

- The first command enables MNP and auto MNP detect, and

disables MNP level 5 data compression.

- The second command causes the script to pause and display the

"Press <Enter> to continue " prompt following the OK response.

- The third command requests the HST to display NRAM settings.

- The fourth command causes the script to pause and display the

"Press <Enter> to continue " prompt following the NRAM display.

- The fifth command clears the screen

- The sixth command requests the HST to display RAM settings.

- The seventh and last command causes the script to pause and

display the "Press <Enter> to continue " prompt following the

RAM display.

THE AUTOEXEC COMMAND FILE AND DYNAMIC HST20 PROGRAMMING

If HST20 is called WITHOUT any parameters AND file called

AUTOEXEC.HSF is found in your Qmodem 4.0 script directory, the

file will be loaded and executed automatically, in a manner

similar to the DOS AUTOEXEC.BAT facility.

In addition to the AUTOEXEC.HSF facility, the HST20 is

externally  programmable via .HSF command files and the @CMD

keyword, mentioned earlier.

When the keyword @CMD is found in a modem command file, the line

which immediately follows will be executed as a SCRIPT command,

rather than as a modem command.  For example, the two lines:

@CMD

SETCOMM CJLM

when found in a .HSF file, will cause the serial port to be set

to 2400-N-8-1, as instructed by the SETCOMM Qmodem 4.0 script

command.  Any script command which requires WORD operands

(explicitly stated operands not bound by quotes) or no operand

at all can be executed following the @CMD keyword.  One of the

script commands which can be executed in this fashion is SYSTEM,

which actually terminates Qmodem and returns to DOS.  For

example, the following .HSF file will change the HST's NRAM

setup to use a CCITT answer sequence and will also decrease the

remote carrier length requirements in preparation for a call to

Singapore.  It will then invoke a second script called SINGAPORE

which will dial a bulleting board system there and upload a

file.  Once the SINGAPORE script terminates, the normal HST

parameters will be restored and Qmodem will be terminated:

ATB1

ATS9=4

@CMD

SCRIPT SINGAPORE

ATB0

ATS9=6

@CMD

SYSTEM Y

NOTE: The SINGAPORE script must terminate with a RETURN command

rather than an EXIT command.  IF EXIT is used, the process will

terminate at that point and control will never be returned to

the HST20 script.

The combination script command files, AUTOEXEC.HSF, and @CMD is

an extremely powerful one, which gives you almost total control

over Qmodem externally, without making a single change to the

HST20 script.  I recommend that you experiment very carefully

with these features and test them thoroughly before using them

for productive purposes.

SUPPORT

I will attempt to correct problems with this script, as well as

keep it current with releases of the HST's firmware and Qmodem

software as time allows.  This is a personal effort which I

am sharing, but for which I receive no remuneration (except from

commercial or business users of the script, and I am still

waiting for those).  Please make no unreasonable demands for

support, especially for things that are not broken; you got this

for free and if it does not perform as you would like it to, you

have a God-given right not to use it.  You may also chose to

alter the script to meet your unique requirements, although the

new Command File facility should address all specific needs not

covered in the script's generic menus.  If you modify HST20,

please do not distribute your version under its original name.

If you have questions or suggestions,  I can be contacted under

my name on the Qmodem Support Conference Echo on the InterLink

or SmartNet networks.

Dan Moore

1/19/1989