===========================================================================

BBS: Inland Empire Archive

Date: 03-22-92 (09:09)             Number: 166

From: ED BEROSET                   Refer#: 21

To: YOUSUF KHAN                   Recvd: NO

Subj: BSV detection                  Conf: (1) 80xxx

---------------------------------------------------------------------------

In a message to Tom Marshall <18 Mar 92 19:17> Yousuf Khan wrote:

> TM> Okay, okay, okay.  I've got your point.  However, this code would work

> TM> on a majority of PC's out there... and Ed Beroset has already

> TM> implemented the code into a boot sector [Thanks, Ed!].

YK> I'd be interested in seeing how you inserted it into the boot

YK> sector. What part of the interrupt list has the info on the boot

YK> sector, if any?

I'm the guy who "inserted it into the boot sector."

Actually, less ambiguously stated, it's in the partition

table loader.  Most of this is probably old hat to you,

Yousuf, but let me recap -- there seems to be some general

confusion about terminology and sequence.

When an XT (or AT, clone, whatever) powers on, it starts executing code at

address F000:FFF0, which is in the BIOS ROM.  The BIOS does

a number of tasks, including setting up the interrupt

vectors, and performing simple integrity checks.  The

bootstrap code in the BIOS then attempts to load one 512

byte sector from drive A:, side (head) 0, cylinder 0,

sector 1.  If that fails (e.g. because there's no disk in

drive A:), the BIOS attempts to load one 512 byte sector

from (physical) Hard Drive 0, side (head) 0, cylinder 0,

sector 1.  This sector is loaded into address 0:7C00h and a

jump is then made to that location.  This absolute first

sector on the PHYSICAL drive is usually referred to as the

"partition table."  In actuality, this sector contains not

only that table (which is what you're looking at when you

run FDISK), but also contains a small, unsophisticated bit

of code to perform the following steps:

1. copy itself to another location in memory (0:600h)

2. transfer control (JMP) to the copy's step 3

3. scan the partition table to assure that all entries are valid

4. scan the partition table to see which partition is marked "bootable"

5. using the head, cylinder, and sector information from

the partition table entry for the bootable partition, load

the indicated sector into memory at 0:7C00h (this is why

steps 1 and 2 are needed)

6. transfer control to 0:7C00h

The sector loaded in step 5 and then executed in step 6 is what is usually

referred to as the "boot sector."  This is somewhat more

complicated and version dependent code to load IO.SYS,

MSDOS.SYS, etc. and is identical to the code that WOULD

have been loaded from a bootable floppy.

Since it's the partition table loader that first gets control of the computer

after the BIOS, it's a favorite spot for virii to hide.

What some of them do when they infect a hard drive is make

a copy of the original partition table sector to some other location on the

drive, and install themselves in place of the original.

When the computer is next booted, it's the virus that gets

control first because it's sitting at the "magic location"

on the hard drive.  The virus is then free to do its dirty

work -- one frequent task is to load "infector" code high

up in memory, then chain INT13h (BIOS disk service

routines) to that location.  When it's finished with its

nefarious deeds, the virus retrieves the original partition

table sector (which, you'll remember, was stashed elsewhere

on the HD) and then transfers control to the original code

as though nothing had happened.

Of course something HAS happened, but no standard partition table loader

checks for it.  Mine does.  Right between steps 2 and 3 on

the chart above (step 2b or not 2b?) I inserted the code

snippet that Tom Marshall suggested to check whether the

INT13 vector's segment points into ROM.  If it does, then

all proceeds normally, otherwise, the code assumes the

worst, beeps and prints the message "Possible viral

infection," and locks up in a very tight infinite loop.  At

that point you would haul out the write protected bootable

floppy with all of your favorite sector reading utilities

and try to figure out what bug (if any) had invaded your

system.

I posted the complete code here a few days ago -- if you missed it and want a

fresh copy (incorporating some very good suggestions from

Luns Tee), send me email, and I'll make it available.

Before you install it on your system, however, you should

be aware of something that Keith Huntingdon pointed out to

me -- some ESDI and SCSI drivers (and possibly other

devices) may legitimately rerout INT13 to a location that

my code would incorrectly identify as being evidence of a

virus.  Look for his message to me just after I posted the

code for examples of some specific hardware that may do

this.

Now if I could only figure out the length of my PIQ, I'd be truly happy.  ;-)

-> Ed <-

--- QT via QMX/XRS

* Origin: > The Paradoxicon < 919-850-9289 > USR DS V42 < (1:151/143)