Date: Fri 18 May 90 19:14:41

From: tweten@prandtl.nas.nasa.gov

Subject: overwriting the psp

From: djb@wjh12.harvard.edu (David J. Birnbaum)

>... if it isn't too much trouble, I'd be interested in taking a look

>at the source for the tsrs you mention.

I've enclosed the source code for my simplest TSR for illustration of the

following points:

1.  By using the undocumented MS-DOS INT 21 function 52H, you can

find the beginning of the chain of MS-DOS memory blocks.  Run that

chain and you will find all environment and code blocks in the machine

(see the code after "blkscn" and the data structure at

"BLK_HDR_SEGMENT").

This is a better way to find a previously installed copy of a TSR

than is use of any MS-DOS interrupt such as the MicroSoft-recommended

"multiplex interrupt", 25H.  Even if you use an interrupt MicroSoft has

"reserved for user programs" all that means is that the inevitable

collision will be with some third party's program rather than with

MicroSoft's.

If you run the list of memory blocks, you can string compare some

portion of the block's contents with your own code.  If the string is

long enough and if it matches, you can bet it is another copy of your

TSR.  The most important considerations in selecting a "signature" to

compare are to make sure that it is long enough and is unchanging.  The

machine instructions in the resident portion of your program will do

nicely.  Just be sure not to embed any variable data in the comparison

range, or you'll never find it.

The technique can be used for simple duplicate detection, or can be

used to pass new parameters to the existing TSR without having to use

interrupts and risk collisions (see all references to "sibling").

2.	In a TSR, the non-resident code and data should be positioned

at the end of the program, so their space can be surrendered at initial

termination (see the code at "START" and "go").

3.  You can deal with the changed addresses of code relocated into

the tail end of the PSP by modifying CS (see all references to

"res_offs") before entry to the code, so it compensates for the move.

That requires that the code be originally assembled at a location an

even multiple of 16 bytes away from its final destination in the PSP.

The result may be the "waste" of a few bytes in the initial .COM image

(range between "START" and "beg_pr"), but you don't care because no

space is wasted after termination.

Some care must be devoted to the segment registers.  In the included

program, I simply told the assembler not to use any segment registers

but CS (see assume directive after "_TEXT").  This works well if you

have few instructions which use another segment register by default;

MASM will automatically prefix any such instructions with a CS segment

override.

If there would otherwise be a lot of prefixes, it may be worthwhile

to save and restore initial contents of some other segment registers,

and give them new values derived from CS.  If you do so, be sure to

reflect that fact in your "assume" directive.

I hope all this helps.  Good luck.

----------------------------8<--- CUT HERE ---8<--------------------------

page	58,132

TITLE	Go -- Control (and measure) speed for Heath/Zenith PC

;    (c) Copyright 1987 David E. Tweten

;    All rights reserved.  Permission for distribution is granted

;    provided that no direct commercial advantage is gained, and

;    provided that this copyright notice appears on all copies.

;    --------------------------------------------------------------------

;

;    Go serves three purposes relating to Dante Bencivengo's "TurboPlus

;    V2.0" modification to 4.77 MHz Heath/Zenith PCs.  It permits FORMAT

;    and VERIFY ON to work.  It also permits the user to speed or slow

;    the processor.  Finally, it measures the processor's speed setting

;    (primarily for batch file use).  "TurboPlus", the predecessor

;    version of "TurboPlus V2.0", was described in the June, 1986 issue

;    of REMark, the journal of the Heath/Zenith Users' Group.

;

;    Due to a quirk in the diskette BIOS, errors will occur when a track

;    format or verify operation is requested at any high speed, or when

;    any diskette operation is attempted at 8 MHz.  The terminate-and-

;    stay-resident portion of this code will prevent the errors from

;    occuring by slowing the processor for the duration of the

;    operation, unless it is already operating at the slow (4.77 MHz)

;    speed.

;

;    The optional switch for Go can specify the speed to use when not

;    formatting or verifying a floppy disk.  It can be "f", which will

;    set the clock speed to fast, or it can be "s", which will set the

;    clock speed to slow.  In any but its initial (TSR installing) call,

;    if no switch is given, Go does not change the speed setting.  On

;    the initial call, if no argument is given, Go assumes "f".  Go uses

;    whatever switch character is current.

;

;    The termination code for Go is set according to the processor speed

;    at call, and by whether or not it has previously been called.  The

;    meaning of its termination code is as follows:

;

;         0 - Go has been called before so the TSR portion is already

;             resident.  Before the call the clock was set to run at

;             4.77 MHz.

;

;         1 - Go has been called before so the TSR portion is already

;             resident.  Before the call the clock was set to run at

;             its fast speed.

;

;         2 - Go has not been called before.  This call installed the TSR

;             code and set the clock speed to the specified value (either

;             Fast or Slow).  If no speed was specified on the call, the

;             default of Fast was used.

;

;         3 - Go has been called in error.  It did nothing.

;

;    NOTE that the actual processor speed is the slower of the speed

;    specified and detected by Go, and the speed specified by the

;    hardware switch included in the "TurboPlus V2.0".  The actual speed

;    is indicated by the LED included with the "TurboPlus V2.0" package.

;

;    Written by:  David E. Tweten   <tweten@prandtl.nas.nasa.gov>

;

;                                   12141 Atrium Drive

;                                   Saratoga, CA  95070

;

;                                   (408) 446-4131

;

;    Inspiration: FV.COM, a program of unstated authorship, included on

;                 the "TurboPlus" software disk.

;

;    Modification History:

;

;       1.0      5-10-87  Initial version.

;

;    --------------------------------------------------------------------