SanyCOMM - A FOSSIL Driver For The Sanyo MBC-55x

Written by Paul Kelly

Copyright 1987 by Paul A. Kelly,  ALL RIGHTS RESERVED

This code is may be used and distributed in its unmodified form for ANY

NONCOMMERCIAL USE WHATEVER.  Modified versions of this software are NOT to

be distributed without the express written consent of the author.  You are

not allowed to sell or otherwise make money from the use of this software

without specific license from the author.

Paul A. Kelly

20 Forbes Road

Trafford, PA  15085

You may contact the author with bug reports or suggestions for improvement

via FidoNet mail at Net 129 Node 11 - First Sanyo Opus - for non-commercial

purposes only.

If you wish to distribute this driver with a commercial communications

package,  just write.  I will be glad to authorize free usage of this

type,  as long as no extra money is charged above the regular price of

package.  I wish to keep track of this kind of usage so that regular

updates can be distributed.

This program implements the Fido/Opus/SEAdog Standard Inferface Layer

(FOSSIL) functions for the the Sanyo MBC-55x.  It is a complete implementation

of the FOSSIL driver as defined by version 5 of the FOSSIL specification.

This driver implements all 27 functions defined in the current FOSSIL

spec.  It also implements the user appendage functions (the extended

dispatcher).

The current version of this driver ignores the port number passed in DL.

SanyCOMM is upwardly compatible with the Sanyo MBC-55x BIOS,  int 14h,

functions 0 - 4.  FOSSIL function 4 is a superset of int 14h,  function 4.

SanyCOMM is now an installable device driver.  Put SanyCOMM.SYS in your

root directory and add the following statement to your CONFIG.SYS file:

Device=SanyCOMM.SYS

SanyCOMM completely replaces the standard Sanyo driver for devices AUX

and COM1,  and adds a device named FOSSIL$.  All i/o to these devices

automatically is routed through the FOSSIL driver,  including redirected

console i/o.

SanyCOMM has been tested fully in production use with BinkleyTerm 1.30,

Opus 1.10.ii/gamma,  and SEAdog v4.1.  It has also been fully tested for

proper operation with GT-PowerComm v13.00F.  The driver uses a 4k receive

buffer and a 1k transmit buffer.

Paul Kelly

01/30/88

########################################