July 7, 1994

_ _ _

/ / (_)

_\ \ \

(_)_/_/  ASPI-WCE v1.0 -

Changes the WCE (Write Cache Enable) bit on the Mode Sense Caching

page 08h via the ASPI device driver.

Copyright 1994, Seagate Technology, Inc. All rights reserved.

Seagate Technology, Inc.

B-5 Technical Support

920 Disc Drive

Scotts Valley, CA 95066

===============================================================

Usage: ASPI-WCE [SCSI ID]

switches:

SCSI ID        - A valid SCSI ID in the range 0 to 6

(no switches)  - prompt for SCSI ID info

Do NOT run this utility until you completely understand its function.

This utility issues a SCSI Mode Select command to your SCSI drive and

makes a fundamental change to the way it operates.  The Write Cache

Enable bit (WCE) controls how the completion status of a disk write is

reported.  When WCE is ON, the drive reports status complete as soon as

the data is taken in to the drive's ram buffer.  This results in very

high turn around times and serves to dramatically speed up system IO.

When WCE is OFF, the drive reports status complete after the data is

physically written to the media.

Whether WCE should be ON or OFF usually depends on the operating

system.  For example, a single user desk top computer system would

enjoy the benefits of Write Cache Enable being ON.  Multi-user and

multi-tasking operating systems, such as Netware and unix based,

operate more efficiently if the bit is set to OFF.  This is because

the operating systems have elaborate schemes in place to protect data

integrity.  When the disk reports that a write has been successfully

completed, the operating system moves on to the next tasks.  If there

were a problem writing the data out to the media after the drive gave

a good status on the write (to the drive buffer because WCE was ON)

then the operating system may be unable to direct the recovery of the

data.

ASPI-WCE will report the current WCE setting and ask if you want to

make a change.  If you select YES, it will then ask if you want the

drive to remember this change after RESET.  Before using ASPI-WCE, be

sure to read the ASPI-ID documentation for further information about

ASPI device drivers, Mode Sense pages and SCSI commands.

ASPI-WCE is designed for the support of users with Seagate disc drives

and has been tested with Adaptec, Acculogic, Always, DPT and Ultrastor

SCSI host adapters with their associated ASPI device drivers. (These

represent the HA's we had laying around in Tech Support, other HA's

with ASPI drivers should also work.)

===============================================================

=====Rev=Hist===============

Written with Borland Pascal v6.0

Revision History:

7/94   v1.0    Initial Public release.

============================

LICENSE AGREEMENT AND WARRANTY DISCLAIMER

This is a legal agreement between you the purchaser and Seagate

Technology, Inc.  By accessing ASPI-WCE, (the "Software"), you agree

to be bound by the terms of this agreement.  If you do not agree, do

not access the software.

Seagate provides the accompanying object code of the Software and

nonexclusively licenses its use to you on the following terms and

conditions.  The Software is Seagate's proprietary, copyrighted

product. Seagate grants you a limited access to use one copy of the

Software.  You may not copy, distribute the Software for resale.  You

may not reverse engineer, modify, rent, or lease the Software.  In

addition, you may not disclose the information or data incorporated in

the Software to others, in any format.

You accept the Software "as is" without any warranty whatsoever.

Seagate does not warrant that the Software will meet your

requirements; that the Software is fit for any particular purpose or

that the use of the Software will be error free.

SEAGATE EXPRESSLY DISCLAIMS ALL WARRANTIES, WHETHER ORAL OR WRITTEN,

EXPRESSED OR IMPLIED, INCLUDING WITHOUT LIMITATION WARRANTIES OF

MERCHANTABILITY, OR FITNESS FOR A PARTICULAR PURPOSE.  IN NO EVENT

WILL SEAGATE BE LIABLE TO YOU, YOUR CUSTOMERS OR OTHER USERS IN ANY

WAY BASED ON USE OF THE SOFTWARE, INCLUDING WITHOUT LIMITATION FOR ANY

LOSS OF PROFITS, LOSS OF DATA OR USE OF THE SOFTWARE OR INDIRECT,

INCIDENTAL, CONSEQUENTIAL, SPECIAL, OR EXEMPLARY DAMAGES ARISING OUT

OF OR IN CONNECTION WITH THE USE OR INABILITY TO USE THE SOFTWARE.

(C) 1994 Seagate Technology, Inc.

End of License Agreement.

-=EOF: ASPI-WCE.DOC=-