ATIM

PC-AT High Resolution Execution Timer

ATIM is a utility to which will load and execute another program

and report elapsed time using the PC-AT high resolution timer.

This program is both hardware and operating system dependent. It

requires an IBM PC-AT or close clone with a high resolution

timer accessable at port 70h as well as bios support for INT 70h.

As supplied, this program uses the undocumented INT 2Eh back

door entry point into COMMAND.COM for its load & execute

function. For operation under Concurrent Dos 5.0 XM, the CCPM

equate is provided which will enable conditional assembly of

the traditional DOS load & execute function.

My primary reason for writing this program is to distinguish

between alternate assembly language subroutines in programs

which I develop. The high resolution timer generates a tick

1024 times a second as oposed to 18.2 ticks per second with

the low resolution timer. This provides accuracy to 3 decimal

places and minimizes the need to put sequences within repeat

loops in order to time them - after all, a repeat loop can

distort true performance because of its side effect of purging

the pre-fetch queue each time through. And use of the back door

INT 2Eh loader eliminates the masking overhead of loading a new

copy of COMMAND.COM.

But it should be kept in mind that with the high resolution

timer enabled, program execution is slowed down somewhat.

You might try commenting out calls to the DISABLE_70 and

running Norton's SI to get an idea of how much (then reboot).

This is inevitable since the interrupt handler is being called

so often. And to make matters worse, the IBM ROM bios shuts

off the timer after each interrupt requiring the program to

turn it back on again. The CLONE equate has been provided to

skip the reinitialization and should be set TRUE if your bios

is less ill mannered than IBM's (works fine on PC's LTD AT).

Most of the code for the high resolution timer was found

squirreled away in a listing published by Greg Weissman in

the March 1986 Dr. Dobbs starting on page 96. The INT 2Eh

technique was grafted in after Ray Duncan's December 1986

column, also from Dr. Dobbs.

Howard Vigorita, NYACC

December 27, 1986