ARCHIVE FILE FORMAT

-------------------

Component files are stored sequentially within an archive.  Each entry

is preceded by a 29-byte header, which contains the directory

information.  There is no wasted space between entries.  (This is in

contrast to the centralized directory used by Novosielski libraries.

Although random access to subfiles within an archive can be noticeably

slower than with libraries, archives do have the advantage of not

requiring pre-allocation of directory space.)

Archive entries are normally maintained in sorted name order.  The

format of the 29-byte archive header is as follows:

Byte 1:  1A Hex.

This marks the start of an archive header.  If this byte is not found

when expected, UNARC will scan forward in the file (up to 64K bytes)

in an attempt to find it (followed by a valid compression version).

If a valid header is found in this manner, a warning message is

issued and archive file processing continues.  Otherwise, the file is

assumed to be an invalid archive and processing is aborted.  (This is

compatible with MS-DOS ARC version 5.12).  Note that a special

exception is made at the beginning of an archive file, to accomodate

"self-unpacking" archives (see below).

Byte 2:  Compression version, as follows:

0 = end of file marker (remaining bytes not present)

1 = unpacked (obsolete)

2 = unpacked

3 = packed

4 = squeezed (after packing)

5 = crunched (obsolete)

6 = crunched (after packing) (obsolete)

7 = crunched (after packing, using faster hash algorithm) (obsolete)

8 = crunched (after packing, using dynamic LZW variations)

Bytes 3-15:  ASCII file name, nul-terminated.

(All of the following numeric values are stored low-byte first.)

Bytes 16-19:  Compressed file size in bytes.

Bytes 20-21:  File date, in 16-bit MS-DOS format:

Bits 15:9 = year - 1980

Bits  8:5 = month of year

Bits  4:0 = day of month

(All zero means no date.)

Bytes 22-23:  File time, in 16-bit MS-DOS format:

Bits 15:11 = hour (24-hour clock)

Bits 10:5  = minute

Bits  4:0  = second/2 (not displayed by UNARC)

Bytes 24-25:  Cyclic redundancy check (CRC) value (see below).

Bytes 26-29:  Original (uncompressed) file length in bytes.

(This field is not present for version 1 entries, byte 2 = 1.

I.e., in this case the header is only 25 bytes long.  Because

version 1 files are uncompressed, the value normally found in

this field may be obtained from bytes 16-19.)

SELF-UNPACKING ARCHIVES

-----------------------

A "self-unpacking" archive is one which can be renamed to a .COM file

and executed as a program.  An example of such a file is the MS-DOS

program ARC512.COM, which is a standard archive file preceded by a

three-byte jump instruction.  The first entry in this file is a simple

"bootstrap" program in uncompressed form, which loads the subfile

ARC.EXE (also uncompressed) into memory and passes control to it.  In

anticipation of a similar scheme for future distribution of UNARC, the

program permits up to three bytes to precede the first header in an

archive file (with no error message).

CRC COMPUTATION

---------------

Archive files use a 16-bit cyclic redundancy check (CRC) for error

control.  The particular CRC polynomial used is x^16 + x^15 + x^2 + 1,

which is commonly known as "CRC-16" and is used in many data

transmission protocols (e.g. DEC DDCMP and IBM BSC), as well as by

most floppy disk controllers.  Note that this differs from the CCITT

polynomial (x^16 + x^12 + x^5 + 1), which is used by the XMODEM-CRC

protocol and the public domain CHEK program (although these do not

adhere strictly to the CCITT standard).  The MS-DOS ARC program does

perform a mathematically sound and accurate CRC calculation.  (We

mention this because it contrasts with some unfortunately popular

public domain programs we have witnessed, which from time immemorial

have based their calculation on an obscure magazine article which

contained a typographical error!)

Additional note (while we are on the subject of CRC's): The validity

of using a 16-bit CRC for checking an entire file is somewhat

questionable.  Many people quote the statistics related to these

functions (e.g. "all two-bit errors, all single burst errors of 16 or

fewer bits, 99.997% of all single 17-bit burst errors, etc."), without

realizing that these claims are valid only if the total number of bits

checked is less than 32767 (which is why they are used in small-packet

data transmission protocols).  I.e., for file sizes in excess of about

4K bytes, a 16-bit CRC is not really as good as what is often claimed.

This is not to say that it is bad, but there are more reliable methods

available (e.g. the 32-bit AUTODIN-II polynomial).  (End of lecture!)

Bob Freed

62 Miller Road

Newton Centre, MA  02159

Telephone (617) 332-3533